## APPENDIX IX

Activities of Members in the Hydrology Component

CAMBODIA

- It has 124 hydrological stations.

- 30 automatic stations (not working).

- 42 flood plain.

- Method of flood forecasting: Regression Method and Station Relation Method.

- Type of Section: (1)Staff Gauge, (2)Slope Gauge, (3)Automatic Water Level Recorder, and (4) 4 telemetry stations.

CHINA

Establishment on flash flood warning system including debris flow and landslides: The State Council has approved the National Mountain Flood Prevention Planning in Oct. 2006, so that it will be put into operation. The plan is concerned with the mountain and hill areas for torrent disaster prevention in the 29 provinces of China, taking the smaller basins of less than 200 km2 as the main object, so that the plan is concerned with 32753 small river basins. The first target of mountain flood prevention is to reduce injury and death as much as possible, give priority to prevention and combine the measures of control and relief, give priority to non-engineering structures and combine non-engineering and engineering measures. Other measures include relocating people and strengthening management of hilly and mountainous areas (KRA1).

The OFFSIA international training program: To accelerate the RCPIP which we take lead, further disseminate China’s flood prediction technology, and promote China’s role in hydrological field, the OFFSIA international training program was held by the Bureau of Hydrology, the Ministry of Water Resources of China, between October 15 – 21, 2007. Nearly 30 representatives of many Asian countries such as North Korea, Laos, Malaysia, the Philippines, Thailand, Vietnam, and Singapore attended the training program.The training program comprised 1-2 days of technical lectures, and a 3-day hands-on training and result summary. Foreign trainees have learned the Xin’anjiang River flood forecast model and applications of flood forecast system, and successfully establish a flood forecast scheme for pilot rivers in their home countries by utilizing China flood forecast system platform (KRA1).

Preparation of inundation and water-related Hazard Maps: Since 2005, the SFCDRH ask various river basin authorities to select 2-3 provinces as pilots to draw flood risk maps for important rivers, reservoirs and flood storage areas. As of 2007, 36 pilot studies in seven major river basins (detention areas, cities, dike protected areas, dam failure events) have all been finished and passed the acceptation check. From Sep. 2007 to Sep. 2008, China has almost accomplished developing flood risk analysis method and flood damage assessment method, developing standard flood analysis software, developing flood hazard maps management platform, drawing historical typical flood inundation maps (KRA4).

Improve management of urban flood: The project of Urban Typhoon Disaster Risk Management led by China has been determined by the Typhoon Committee as the cooperative project of the three working groups. The purposes to launched this project are to exchange the experiences on management and mitigation of floods and typhoon-related disaster in urban area between TC Members; to share the technology of urban flood monitoring and methodology of urban flood forecasting and prediction, early warning and disaster assessment between TC Members and to promote management of urban flood and other typhoon–related disasters in TC area. The expected achievement of the project is to prepare “Guidelines on flood mitigation and management for urban planning and development in the Typhoon Area" (KRA2).

Establishment on the community-based flood forecasting and warning system: In last two years, China has stressed monitoring and early warning system building for the mountain flood disaster prevention effort.  Systems have been established based on local transportation, communication, and socio-economic conditions under the principles of suitability, combination of advanced technology and local conditions, economic feasibility, and easy operability. Consequently, a monitoring system may comprise automatic remote rainfall measuring station and water level station as well as some simple, man-controlled rainfall monitoring points. Some counties have established their flood control information processing platform, which is capable of collecting and processing information, helpful for decision-making, prediction and early warning, greatly enhancing capacity of those root-level agencies (KRA5).

Provision of on-the-job training on flood forecasting systems: With the aim to establish a mountain flood early warning system in most parts of China, improve mountain flood forecast technology, and learn foreign peers experience in this field, the Bureau of Hydrology, the Ministry of Water Resources, has organized and implemented a training trip to the United States which was approved by the State Administration of Foreign Experts Affairs of China. The Mountain Flood Early Warning and Forecast Technology and Application Software training program was attended by 15 professionals from the BOH and hydrological department of Heilongjiang, Liaoning, Beijing, Anhui, Sichuan, Hubei, Jiangxi, Zhejiang, Guangdong and Guangxi provinces. The training was conducted by the Hydrologic Research Center in San Diego, USA, in three weeks from January 7 to February 2, 2008 (KRA5).

Integrated Workshop on Coping with Climate Change: ESCAP/WMO Typhoon Committee Integrated Workshop on Coping with Climate Change in the Typhoon Committee Area was held in Beijing from 22 to 26 September 2008. This workshop was jointly organized by Ministry of Water Resources, China Meteorological Administration, U.N. Economic and Social Commission for Asia and the Pacific (ESCAP), and Typhoon Committee of World Meteorological Organization (WMO). The Workshop lasted 5 days and its main topic was “evaluate and minimize potential economic and social impacts of climate change”（KRA7）.

DPR OF KOREA

N/A

HONG KONG, CHINA

During the passages of Typhoons Neoguri and Fengshen, over 200 millimetres of rainfall were recorded in many parts of Hong Kong and the Black and Red Rainstorm Warning Signals were issued respectively. Over 100 millimetres of rainfall were recorded in parts of Hong Kong and the Amber Rainstorm Warning Signal was issued during the passage of Typhoon Hagupit. Storm surges associated with Hagupit combined with high tides led to flooding and damage in various coastal areas of Hong Kong.  More than 100 millimetres of rainfall were recorded in many parts of Hong Kong and the Amber Rainstorm Warning Signal was issued during the passage of the low pressure area associated with the remnant of Higos.

To alleviate flooding, the Hong Kong Special Administrative Region  Government (HKSARG) continued with major river-training works and flood-control projects, as well as constructed village flood pumping stations, stormwater drains and drainage tunnels.

The data from raingauges operated by the Drainage Services Department (DSD) and Geotechnical Engineering Office of the HKSARG were relayed to HKO to support the operation of warning systems. Drains, engineered channels, culverts and watercourses were inspected and maintained.  Local flood warning systems were installed and an automated flooding information dissemination system had been implemented.  A list of flooding blackspots was also compiled.

HKO provided DSD with forecast guidance on the rainstorm probability in Hong Kong to facilitate their flood control operations.

Dynamic hydrological and hydraulic computer models for the drainage systems in Hong Kong managed by DSD were developed to provide quantitative information on the risk of flooding, impacts of development and the performance of various flood loss mitigation options.

JAPAN

Adaptation Strategy against climate change in Japan: A policy report ”Climate Change Adaptation Strategies to Cope with Water-related Disasters due to Global Warming” was drawn up by Panel on Infrastructure Development in June 2008. The policy report aims at “Water-Disaster Adaptive Society” for sustainable socioeconomic activity and daily life by adequate mix of adaptation and mitigation measures, the basic direction of this report is to minimize damages. 5 responses to hazards and 5 adaptation measures are required, respectively (KRA 2).

Approaches of International Flood Network (IFNet) and Global Flood Alert System (GFAS):The Global Flood Alert System (GFAS) operated by the International Flood Network (IFNet) and Integrated Flood Analysis System (IFAS) developed by International Centre for Water Hazard and Risk Management (ICHARM) were presented at the First Asia-Pacific Water Summit held in December 2007 in Beppu Japan. And IFNet and ICHARM hosted a Workshop for GFAS Validation in October 2008 in Tsukuba Japan, in collaboration with the World Meteorological Organization (WMO). Respective officials from 6 countries and an international organization were invited to study applicability of the GFAS. (KRA 6)

Flood Hazard Mapping Project: The year 2008 is the second year of a three-year extension of this effort. In the WS held in Beijing China last September, Japan gave a detailed presentation, followed by reports from various countries on their efforts related to FHM. The WS has confirmed the activities undertaken in the past year and also discussed action plan for 2009. In 2009, a project undertaken by China and Malaysia will be promoted, the Philippines will proceed with the community-based FHM, and other members plan to prepare FHM for specific areas. Japan also intends to draft and compile the final report on FHM Activities with other members(KRA 5).

Proposals for Medium- and Long-Term Measures against Sediment Disasters: Japan is blessed with rich natural environment. Such environment, however, presents severe natural conditions. With a large number of rapids and its geological weakness, the Japanese islands are vulnerable to sediment-related disasters caused by weather conditions like typhoons and localized torrential rain. Efforts based on broad, long-term perspectives are needed. MLIT received a proposal from Advisory Group sessions to plan a safe and beautiful country while coping with the issues caused by changes in natural phenomena and social situations, And the proposal shall be reflected in the social infrastructure development priority plan and will be referred for drafting new medium- to long-term policies and systems (KRA2).

LAO PDR

N/A

MACAO, CHINA

The storm surge induced by Hagupit resulted in the most serious local flooding in Macao since 1993.  Water depth of 1.5 m above ground level was reported in low-lying coastal areas that affected greatly the living of citizens and tourists, and great financial losses were reported.

Cooperation among different government organizations for the establishment of an automatic flooding monitoring system has started right after the serious local flooding. The system consists of nine land- and 2 seashore-stations and one tide and wave recorder to monitor the actual flooding conditions in the low-lying areas, with the data centralized at SMG for further dissemination. SMG will also be responsible of planning and issuing the flooding warnings to the public, being at least 12 hours ahead if induced by storm surge, starting from March 2009.

MALAYSIA

The Department of Irrigation and Drainage (DID) has installed 362 telemetric stations in 38 river basins. To improve the flood information system, 405 manual river gauges and 1019 stick gauges have been set up in flood prone areas. 300 automatic flood warning sirens and 154 flood warning boards are being operated as part of the flood warning system. A further 18 telemetry systems were installed in the nation and 88 telemetry systems are being developed for the Klang Valley Integrated Flood Forecasting and Warning System (SERBAN). This flood modelling system, which comprises of hydrometeorological and hydrodynamic modelling also includes Geographical Information Systems (GIS) components to generate flood risks maps.

The Kuala Lumpur Stormwater and Management Road Tunnel Project (SMART), which has been in operation since July 2007, were proven to be a successful flood mitigation project. It has successfully diverting flood water from upstream of the Klang River, which is a major cause of flooding in Kuala Lumpur. There were at least 4 major storm events during the past one-year where diversion of excess flood water by the SMART system had averted massive flooding in Kuala Lumpur.

Presently flood-forecasting models are being operationally used in 13 river basins. Regression and Linear Transfer Function based hydrological modelling are being used in these flood-forecasting models. Some necessary improvements were made to the flood-forecasting models in three river basins in the state of Johor during the middle of the year. The state of Johor had been the most affected state by flooding for the past two years. Besides the above, DID has also organized various courses in regard to flood mitigation, forecasting, hydrological and catchments modelling for engineers. A flood mitigation course for officers from agencies involved in flood disaster relief efforts was also conducted by DID.

PHILIPPINES

Improvement of the Flood Forecasting and Warning System (FFWS) in the Pampanga and Agno River Basins under the JICA Grant and Establishment of Early Warning System for Disaster Mitigation in the Philippines under the KOICA Grant (KRA1).

The Joint Operation and Management Committee (JOMC) of the FFWSDO that oversees the operation and maintenance activities of monitored major river basins and reservoirs in the Philippines conducted four (4) regular meetings and its Sub-Committee on Hydrology convened its members four (4) times to discuss the integration of the operation of Angat and Ipo dams; Establishment of flood forecasting and warning system in the Caliraya-Botokan-Kalayaan (CBK) dams (KRA3).

Flood hazard mapping activities: One of the components of the UNDP Ready project is multi-hazard mapping for hydrometeorological and geological hazards.  Under the project, the PAGASA undertakes flood hazard mapping for selected areas using a 1:10K scale base maps and storm surge hazard mapping using a 1:50K scale base maps.  Project sites are selected in terms of their vulnerability to floods/flashfloods and storm surges (KRA4).

Hazards Mapping and Assessment for Effective Community Based Disaster Risk Management (UNDP Ready Project): This undertaking is being funded by the Australian Agency for International Development (AusAID) through the United Nations Development Programme (UNDP) and is implemented by the member agencies of the National Disaster Coordinating Council (NDCC) namely, the PAGASA, the Philippine Institute of  Volcanology (PHIVOLCS), the Mines and Geoscience Bureau (MGB), the National Mapping Resource and Information Authority (NAMRIA) and the Office of Civil Defense (OCD) who acts as the Project Management Office.  The project components include multi hazard identification and disaster risk assessment of hydrometeorological (floods/flash floods, rainfall induced landslides and storm surge) and geological (volcanic and earthquake related) hazards, development of community based early warning system (floods and tsunami), development of IEC strategies and materials for specific target groups and mainstreaming of risk reduction into the local development planning process (KRA5).

The calibration and verification for Sapang Buho forecasting point of the Pampanga river basin using the Multiple Linear Regression (MLR) model has been completed while those of San Isidro and Arayat forecasting point are on-going which are all under the Pampanga river basin.  Likewise, the same model is being fine tuned in several forecasting points of the Agno, Bicol and Cagayan river basins (KRA6).

REPUBLIC OF KOREA

- Set up the basic plan for the flood risk map in Korea

- Constructing the assessment system of flood control measures

- Building the hydrological flood forecasting model for the Chung-Ju Dam

- Building the quality control system for national hydrological data

- Utilization of the meteorological radar data

: Building link between the Imjin River rainfall radar and the meteorological radar

: Enabling real-time radar data analysis

: Providing radar analysis tool for individual PC

- Taking part in the 2008 TC Integrated workshop held at Beijing, China, on Sept. 22-26, 2008

SINGAPORE

Heavy rains from the indirect impact of tropical cyclones could cause flooding in low lying areas of Singapore. Singapore’s stringent drainage code requires low-lying grounds to be raised in conjunction with redevelopment proposals which thus helps to reduce the severity of floods caused by heavy rains. In addition, the ground levels of our reclaimed sites are at least 1.25 m above the highest tide.

In enhancement of services, improvement in content and mode of dissemination of heavy rain warnings have provided a faster and more efficient method to alert users in their preparedness against a heavy rain episode.

The Marina Barrage which is part of a comprehensive flood control scheme was officially opened in August 2008. The Barrage consists of a series of nine crest gates at a dam which can be activated to release excess storm water into the sea when the tide is low and in the case of high tide, giant pumps help to drain the excess water into the sea.

THAILAND

Royal Irrigation Department (RID) has the strategies in flood prevention and relief with the aims to decrease the loss of lives and their properties and reduce the damaged urban and cultivated area. Plan and management system were set in terms of monitoring, prediction and warning by Water Watch and Monitoring System For Warning Center (WMSC) in order to follow the situation throughout 24 hours. In addition, there was the collaboration with other related organizations to plan the flood prevention. Local flood protection system in important economic area where severe flood may occur was set. Early warning system using various technologies was established such as telemetering for water management. The example project was as follows:

Telemetering Project for Flood forecasting and warning in Upper Ping Watershed: Many irrigation projects in Upper Ping Watershed were developed, i.e. Mae Taeng regulator, Mae Ping regulator, medium reservoir project such as Mae Ngat Somboonchon Dam and Mae Kuang Udomtara project. These can partially relief flood, but damaged urban area in Chiang Mai caused by heavy rain and over-bank flow still occurs every year. Therefore, Royal Irrigation Department initiated the telemetering and flood warning system in Upper Ping Watershed with 9,733 sq.km area from Vieng Haeng District, Chiang Mai Province to Nong Saleak regulator in Muang District, Lumpoon Province after where Ping and Kuang River merge. The monitoring area was in Muang, Mae Taeng, Sunkumphaeng District in Chiang Mai Province and Muang District in Lumpoon Province. The Activities of project consist of (1) Flood forecasting system by MIKE11 and MIKE GIS; (2) Data-based system and web-site development; (3) Flood watch system by flood management system model developed by DHI water & Environment; (4) Telemetering system run by SCADA system consisting of 12 field stations and one main station in Chiang Mai. Nowadays, the project was completed and is in the validation and further planning process.

In 2005, Department of Water Resources developed and operated a flash flood and landslide warning system in mountain and upland area, dealing with system alerts activated by heavy rainfall and rising of river levels. We uses the system to monitor river level and  rainfall at appropriate recording sites, early warning signals (the traffic light concept Green®Yellow ® Red)  would be sent to subscribers and communities in real time in advance of disastrous impact which provide time for villagers to take response actions.

Department of Water Resources has developed and installed this early warning system in 458 villages of the total 2370 villages. In 2008, about 95 villages reported to set off warning and this could reduce the risk of natural disasters and lost of properties and lives such as: Ban Wang Hin, Prachinburi Province;  Ban Khlong Loi , Ban Nam Tok and Ban Dan Sing Khon, Prachup Kirikhun Province; Ban Don and Ban Nong Bua, Chanthaburi Province; Ban Nam Meed, Ban Nam Sod, Ban Tey Kue Hen, Ban Sakad Khang and Ban Nam Phod, Nan Province; Ban Pang Poi and Ban Sook Ru Thai, Chiangmai Province; Ban Pu Nam Ron and Ban Bung Nua, Ratchaburi Province; Ban Na Tong, Phrae Province and etc..

To support the hydrological activity of Thailand, Thai Meteorological Department launched: (1) the Quantitative Precipitation Estimate (QPE) using TMD’s automatic rain gauges and radar observations. and (2) the flood forecast using the MIKE11 to produce flood map in the selected river basin.

UNITED STATES OF AMERICA

NWS Pacific Region received funding to begin the replacement of the Hawaii Rain Gage Collection Network.  This is the first year of a 3-year program to replace the entire rain gage collection network system in Hawaii.  The new system will replace the aging rain gages with new technology and will use HF radio line of sight communication system rather than land or cell phone lines.

VIET NAM

Existing Contract for Network Improvement of Data Transfer from the National Center for Hydrometeorological Forecasting of Viet Nam NCHMF) to the Regional Flood Management and Mitigation Center between The Mekong River Commission Secretariat (MRCS) and NCHMF. The objective of  this Contract is to upgrade and improve the network coverage of the rainfall stations of the NCHMF  and to secure proper delivery to the MRCS timly, accurate and reliable real-time water level and rainfall data, which shall be used for  the operational flood forecasting.

Existing agreements between Mekong committee and NCHMF for exchanging the hydrological data from China, Thailand, Laos and Cambodia and for flood forecasting and warning agency and disaster management agency of  Viet Nam. Existing cooperation between HMS Vietnam and DHI for using Mike-11 model to flood forecasting.

On going the Project “Strengthening the flood warning and forecasting capability in Viet Nam (ODA Italy). The periods of the project runs from 2003 to 2009 and invested in 5 provinces including: Quang Binh, Quang Tri, Thua Thien Hue, Quang Nam, Quang Ngai. Invested Scope is focused on main issues: Install automatic observation equipments for 74 observation stations (new 15 rain gauges, 16 automatic  meteorological  stations, 43 automatic water level stations, install  calculating equipment for 6 forecasting Centers, install telecommunication for observed data, Apply and establish flood forecasting model Mike-11.

On going the Mahasri Project to improve the forecasting, warning capability for Central Viet Nam to speed up and facilitate the cooperation with other countries in the region and in the world. The budget is about 3,685 millions USD funded by Japanese Government. The project period is from 2008 to 2013. The scope is of: install 28  automatic rain gauges from Thua Thien Hue to Quang Ngai province, install 6 automatic water level stations on Huong and Thu Bon-Vu Gia river, install 01 automatic  meteorological  station at Ba Na, install 20 modems for transferring real time observed data for 20 stations. In 2008 was established the flood warning system with 10 automatic rainfall gauges in Thua Thien Hue province on frame of MAHASRI's near-realtime rainfall data in Central Vietnam.

There are several on-going research project: (1)On-going flash flood mapping Project with purposes: drawing up of flash flood map and establishing flash flood warning system in the North Viet Nam (the first phase 2006-2008 in the provinces of Northen Viet Nam with more than 70 automatically rainfall stations). (2)On-going the project of establish the alarm system of water level in Vietnam. (3)On-going National Project: Development and application of the American NWSRFS Model for Flood and inundation forecasting and warning in Hong – Thai Binh river system. (4)On-going Ministry Project: Development of flood prediction and inundation warning technology in Ve – Tra Khuc river system, the technological experiment and transfer. (5)On-going Ministry Project: Development of 5-day flow prediction technology to large reservoirs in Da and Lo river system. And (6)On-going Ministry Project: Development and application of the WETSPA and HECRAS for flood simulation and prediction in Thu Bon – Vu Gia river system.

Two forecasters from National Hydro-Meteorological Service of Viet Nam attended the water and energy Budget-based Distributed Hydrological Model Training Course at the Tokyo University from 13 to 27 November 2008 under the pilot project of AWCI (Asia Water Cycle Initiative).  The forecasters had to applying this DHM for Huong river to be used for societal benefits; Vietnam sent persons to take part in the International Hydrological Analysis and Forecasting Course in Colorado, USA from June 9 to June 27, 2008 and sent one person to the training course on Development of flood forecasting system from TANK model in Malaysia from 01 to 31 Dec. 2008.

UNESCAP

During the period since the 40th Session of the Typhoon Committee, major key activities of UNESCAP related to the Hydrological Component include the following:

1.	As part of researches on water resources policy issues, the Water Security Section has carried out various studies related to the annual theme study of the Commission on food security and sustainable agriculture on the one hand and development of water resources for inclusive sustainable socio-economic development in the region.  In this connection, UNESCAP has initiated the introduction of a new concept of water security, which would contribute to overcoming barriers to human well-being or human security and also creating opportunities for development of water resources for economic growth.

2.	In this context, efforts are being to develop “water insecurity index” so as to attract attention of policy makers for action on priority water resources issues, such as water-related and typhoon-related disasters.  It will be a good opportunity to further strengthen collaboration and cooperation among the Typhoon Committee, WMO and UNESCAP.  The proposed project by Working Group on Hydrology for the development of flood-preparedness index is therefore expected to contribute to the identification of major communities or locations of flood-insecurity in the Typhoon Committee Area.  It is also expected that similar efforts of the Typhoon Committee in identifying major communities or locations of typhoon-related insecurity in the Typhoon Committee Area would enable UNESCAP at various occasions, such as the Sixth Ministerial Conference on Environment and Development in 2010 or the Second Asia-Pacific Water Summit in 2010 to solicit support for action against typhoon-related disasters in the Typhoon Committee Area.

3.	In order to strengthen capacity of the region to cope with impacts of climate change on water resources, UNESCAP has initiated new activities on climate change.  Among the priority activities, UNESCAP participated in several regional initiatives, such as those initiated by ASEAN to strengthen capacity of its country members or networking on climate change adaptation and capacity-building on the development of eco-efficient water infrastructure.

4.	In connection with the above initiatives, UNESCAP organized the First Regional Workshop on Eco-efficient Water Infrastructure in Seoul in November 2008 with the participation of 22 countries, including 10 Members of the Committee.  As a follow up to this workshop, UNESCAP is identifying pilot projects for implementation or sharing experiences among the countries in the region.  In addition, UNESCAP will develop a set of guidelines for dissemination of experiences and technology on this subject.

5.	UNESCAP also supported the Typhoon Committee in its expansion of cooperation with the Asia-Oceania Geoscience Society, which provides a good regional forum to share findings of researches on hydrology and meteorology.  In June 2008, UNESCAP with the strong support including financial contribution of the Korea Water Forum, Korea Institute of Construction Technology and the National Disaster Preparedness Institute of Korea organized two sessions on “Monitoring of investment and results of integrated water resources management” and “Assessment of Socio-economic Impacts of Water-related Disasters” at the Fifth AOGS Symposium, held in Busan in June 2008.  In these two sessions, the TCS Hydrologist, Dr. Liu Jin-Ping together with a team of KICT led by Dr Hong Il-pyo, Vice Chair of WGH and a team of NIDP led by Dr. Waon-Ho Yih participated in the Symposium.  In continuation of this tradition, UNESCAP has initiated two sessions at the Sixth AOGS Symposium, scheduled to take place in August 2009 in Singapore, with focus on the following themes: “Development of guidelines for assessment of impacts of climate change on water resources” and “Advances in the Development of Eco-efficient Water Infrastructure”.  Interested experts of the Committee and international organizations are invited to participate in these sessions.

WMO

(1) The main outcomes of the XIII Session of the WMO Technical Commission for Hydrology (CHy) in November 2008 and the decisions and recommendations of the XIV session of the WMO Regional Association (RA-II) – Asia in December 2008:

- the development of the WMO “Strategy and Action Plan for the Enhancement of Cooperation between National Meteorological and Hydrological Services for Improved Flood Forecasting” that will be operationalized by a detailed activity plan to which the WGH could contribute from its own ongoing and planned activities

- the development of a Flash Flood Guidance System such as the one currently being implemented in the Mekong river basin;

- the setting up of a HelpDesk for Integrated Flood Management with support provided currently by the governments of Japan, and Switzerland. This HelpDesk was seen as a valuable tool that could be utilized by flood managers of hydrological services being members of the Typhoon Committee.

(2) Joint interest between the TC-WGH and WMO:

- Improvement of real-time hydro-meteorological data collection networks

- and timely hydrological information and disaster warning dissemination

- Extension of operational flood forecasting systems to more river basins

- Improvement of flood forecasting model accuracy/performance

- Development of flash flood monitoring and warning system

- Development of community-based flood forecasting and warning system

- Improvement of hydrological products in response to users’ needs

- Development of urban flood monitoring, forecasting and early warning

- System for urban flood management

- Research of Climate Change Impact Study on Water and Hydrology

- Development of Satellite-Based Rainfall – Runoff Modeling System

(3) Based on the information provided and recalling the activities and projects undertaken by the WGH of the TC, the WGH recommended to the TC to

- Establish a communication platform between the TC-WGH and the WMO RA-II WGH;

- Create a joint network of experts as a pool of knowledge and expertise in the region;

- Establish a liaison mechanism between the chairs of both WGH and Theme Leaders to develop and implement joint activities within the mandates of each of the Working Groups Hydrology;

- To engage as a first step of joint activities in the area of urban flood forecasting and management.