APPENDIX X

UPDATE OF THE TYPHOON COMMITTEE OPERATIONAL MANUAL (TOM)

1.	The Typhoon Committee Operational Manual - Meteorological Component (TOM) has been reviewed and updated every year since its first issue in 1987. The 2025 edition was completed and posted on the WMO website in 2025 in accordance with the approval of amendments to the 2024 edition by the Typhoon Committee 57th session (17– 20 February 2025, Manila Bay, the Philippines).

2.	At the 57th session, the Committee decided that the rapporteur of the RSMC Tokyo - Typhoon Center in Japan Meteorological Agency (JMA) continues arrangements for updating the TOM. In this connection, on 30 October 2025 in advance of the annual revision of TOM, the rapporteur, Dr. ISHIHARA Koji of the RSMC Tokyo - Typhoon Center requested the focal points of the meteorological component of the Members to input on tropical cyclone analysis and forecast procedure by the NMSs.

- Input on analysis and forecast procedure were submitted by seven focal points of China; Hong Kong, China; Japan; Malaysia; the Philippines; Republic of Korea and Thailand.

4.	On 26 January 2026, the rapporteur proposed some revisions, including reflection of the inputs from the members, to the focal points of the meteorological component of the Members and invited them to further provide comments for the revision and proposals for updates.

5.	Proposed revisions, updates and amendments are embedded as a format of MS-WORD track-changes as attached in Annex 1. The major points of the revisions are given below:

- Proposal by the RSMC-Tokyo – Typhoon Center

- Removal of the Tropical Cyclone Passage Report activity (Section 2.6; Appendix 2-J)

- Update of the information on the ICAO technical specifications (Section 4.5)

- Removal of the RSMC Tokyo - Typhoon Center’s GTS bulletins for tropical cyclone best-track data (Section 6.3; Appendix 5-E; Appendix 6-B)

- Update of the information on the list of names for tropical cyclones adopted by the Typhoon Committee for the western North Pacific Ocean and the South China Sea (Appendix 1-B)

- Addition of the information on the list of acronyms used in the operational manual – meteorological component - (Appendix 1-C)

- Update of the information on the products provided by RSMC Tokyo – Typhoon Center (Appendix 3-A)

- Update of the information on the present operational status of the meteorological telecommunication network for the Typhoon Committee region (Appendix 5-B)

- Proposal by the seven focal points of China; Hong Kong, China; Japan; Malaysia; the Philippines, Republic of Korea and Thailand

- Update of the information on the list of stations from which enhanced upper-air observations are available (Appendix 2-B)

- Update of the information on the list of buoy observations by Typhoon Committee Members (Appendix 2-C)

- Update of the distribution map of the radar stations of Typhoon Committee Members (Appendix 2-D)

- Update of the information on the technical specifications of radars of Typhoon Committee Members (Appendix 2-E)

- Update of the information on the technical specifications of satellite operated by Typhoon Committee Members (Appendix 2-F)

- Update of the table on the satellite imagery receiving facilities at Typhoon Committee Members (Appendix 2-G)

- Update of the information on the analysis methods, forecasting methods and NWP (Appendix 3-B)

- Update of the contact details (Appendix 5-C)

WORLD METEOROLOGICAL ORGANIZATION

TECHNICAL DOCUMENT

WMO/TD-No. 196

TROPICAL CYCLONE PROGRAMME

Report No. TCP-23

TYPHOON COMMITTEE

OPERATIONAL MANUAL

METEOROLOGICAL COMPONENT

2026 Edition

SECRETARIAT OF THE WORLD METEOROLOGICAL ORGANIZATION

GENEVA SWITZERLAND

© World Meteorological Organization, 2026

The right of publication in print, electronic and any other form and in any language is reserved by WMO. Short extracts from WMO publications may be reproduced without authorization, provided that the complete source is clearly indicated. Editorial correspondence and requests to publish, reproduce or translate this publication in part or in whole should be addressed to:

Chairperson, Publications Board

World Meteorological Organization (WMO)

7 bis, avenue de la Paix

P.O. Box 2300	Tel.: +41 (0) 22 730 84 03

CH-1211 Geneva 2, Switzerland	E-mail: Publications@wmo.int

NOTE

The designations employed in WMO publications and the presentation of material in this publication do not imply the expression of any opinion whatsoever on the part of WMO concerning the legal status of any country, territory, city or area, or of its authorities, or concerning the delimitation of its frontiers or boundaries.

The mention of specific companies or products does not imply that they are endorsed or recommended by WMO in preference to others of a similar nature which are not mentioned or advertised.

The findings, interpretations and conclusions expressed in WMO publications with named authors are those of the authors alone and do not necessarily reflect those of WMO or its Members.

This publication has been issued without formal editing.

#### CONTENTS

Page

CHAPTER 1  GENERAL	1

1.1	Introduction	1

1.2	Terminology used in the region	2

1.2.1	General	2

1.2.2	Classification of tropical cyclones	2

1.2.3	Tropical cyclone characteristics	2

1.2.4	Terms related to the warning and warning system	3

1.3	Meaning of terms used for regional exchange	3

1.4	Units used for regional exchange	6

1.5	Identification of tropical cyclones	7

1.6	Acronyms	7

CHAPTER 2  OBSERVING SYSTEM AND OBSERVING PROGRAMME	8

2.1	Networks of synoptic land stations	8

2.1.1	Surface observations	8

2.1.2	Upper-air synoptic observations	8

2.2	Ship and buoy observations	9

2.3	Radar observations	9

2.4	Meteorological satellite observations	9

2.4.1	Satellite imagery data and related products	9

2.4.2	SAREP reports	10

2.5	Aircraft observations	10

10

CHAPTER 3  TROPICAL CYCLONE ANALYSIS AND FORECAST	12

3.1	Analysis at RSMC Tokyo - Typhoon Center	12

3.2	Forecast at RSMC Tokyo - Typhoon Center	12

3.3	Operational analysis and forecast at centres of Typhoon Committee Members	13

CHAPTER 4  TROPICAL CYCLONE WARNINGS AND ADVISORIES	14

4.1	General	14

4.2	Classification of tropical cyclones, 	14

4.3	Tropical cyclone advisories	14

4.4	Tropical cyclone warnings for the high seas (WWMIWS)	14

4.5	Tropical cyclone SIGMET and advisory information for international aviation	16

CHAPTER 5  TELECOMMUNICATIONS	18

5.1	General	18

5.2	Dissemination of data and products	18

5.3	Schedule for exchange of cyclone advisories	18

5.4	Meteorological telecommunication network for the Typhoon Committee region	18

5.5	Addresses, telex/cable and telephone numbers of the tropical cyclone warning centres	18

5.6	Abbreviated headings of tropical cyclone advisories and warnings	18

5.7	Exchange of information related to tropical cyclones	19

CHAPTER 6  MONITORING AND QUALITY CONTROL OF DATA	20

6.1	Quality control of observational data	20

6.2	Monitoring of exchange of information	20

6.3	Verification	20

CHAPTER 7  ARCHIVAL OF DATA	21

7.1	Data to be archived by Typhoon Committee Members	21

7.2	Data to be archived by RSMC Tokyo - Typhoon Center	21

7.3	Exchange of archived data	21

CHAPTER 8  CAPACITY DEVELOPMENT	22

8.1	Tropical Cyclone Forecast Competency in the Typhoon Committee Region	22

8.2	Capacity development activities conducted by RSMC Tokyo - Typhoon Center	22

8.3	Capacity development activities conducted by Members	22

________________

#### APPENDICES

1-A	Guidelines for converting between various wind averaging periods in tropical cyclone conditions

1-B	List of names for tropical cyclones adopted by the Typhoon Committee for the western North Pacific Ocean and the South China Sea

1-C	List of acronyms used in the Operational Manual - Meteorological Component -

2-A	List of stations from which enhanced surface observations are available

2-B	List of stations from which enhanced upper-air observations are available

2-C	List of buoy observation by Typhoon Committee Members

2-D	Distribution of the radar stations of Typhoon Committee Members

2-E	Technical specifications of radars of Typhoon Committee Members

2-F	Technical specifications of satellites operated by Typhoon Committee Members

2-G	Satellite imagery receiving facilities at Typhoon Committee Members

2-H	List of SAREP reports issued by Typhoon Committee Members

2-I	Reconnaissance flights conducted by Typhoon Committee Members

3-A	Products provided by RSMC Tokyo - Typhoon Center

3-B	Analysis methods, forecasting methods and NWP system for forecasting currently used by the NMSs of Typhoon Committee Members

4-A	Classifications of Tropical Cyclones in the western North Pacific internally used by Members

4-B	Examples of advisories issued from RSMC Tokyo - Typhoon Center

4-C	Stations broadcasting cyclone warnings for ships on the high seas

5-A	Meteorological telecommunication network for the Typhoon Committee

5-B	Present operational status of the meteorological telecommunication network for the Typhoon Committee region

5-C	List of addresses, telex/cable and telephone numbers of the Tropical Cyclone Warning Centers in the region

5-D	Abbreviated headings for the tropical cyclone warnings

5-E	Collection and distribution of information related to tropical cyclones

5-F	Table of abbreviated headings (TTAAii CCCC)

6-A	Examples of the message format for inquiry on doubtful and garbled reports

6-B	Standard procedures for the verification of typhoon analysis and forecast at National Meteorological Centres

6-C	Verification sheet for positioning of the centre, prediction of movement, and analysis and forecast of intensity of tropical cyclones

7-A	List of data archived by RSMC Tokyo - Typhoon Center

7-B	Global tropical cyclone track and intensity data set - report format

8-A	Tropical cyclone forecast competency in the Typhoon Committee region

## CHAPTER 1  GENERAL

7

##### 1.1	Introduction

Typhoons have always been a major threat to the Typhoon Committee region. As a result, they are a common target for meteorological services in the region to monitor, analyse, forecast and warn against.

Under the spirit of international co-operation, a regional programme to mitigate the damage due to tropical cyclones was launched by the Typhoon Committee which was established in 1968. Since its establishment under the auspices of ESCAP in co-operation with the World Meteorological Organization (WMO), the Typhoon Committee has developed its area of activities to consist of three components, i.e., meteorological, hydrological and disaster prevention and preparedness.

Of these components, the meteorological component aims at improving and upgrading the analysis and forecast used for the routine operation. For this purpose, the Typhoon Committee has arranged a variety of co-operation efforts. One of the epoch-making events in the history of the Committee was the Typhoon Operational Experiment (TOPEX), which was organized for all three components. The third component was specifically organized as Warning Dissemination and Information Exchange Component.

The Meteorological Component of TOPEX had a co-operation programme where concerted efforts were exerted to analyze and forecast specified typhoons using common technical procedures. The procedures were described in the TOPEX Operational Manual which had been utilized in meteorological services in the Typhoon Committee region during the operational phase of TOPEX.

Activities of the Meteorological Component of the Typhoon Committee - including execution of the meteorological component of TOPEX for three years (1981-1983) - had been planned and organized under the Tropical Cyclone Programme (TCP) of the WMO. The main long-term objective of the TCP is to assist Members in upgrading the capabilities of National Meteorological and Hydrological Services (NMHSs) to provide better tropical cyclone, related flood and storm surge forecasts and more effective warnings through regionally coordinated systems, and to encourage Members to establish national disaster prevention and preparedness measures.

As a result of international cooperation and coordination, and with the aid of meteorology and modern technology, such as satellites, weather radars and computers, all tropical cyclones around the globe are now being monitored from their early stages of formation and throughout their lifetime. Six centres designated by WMO as Regional Specialized Meteorological Centres (RSMCs) located in Honolulu, La Reunion, Miami, Nadi (Fiji), New Delhi and Tokyo, as well as other centres of National Meteorological Services (NMSs) carry out these activities. These centres also provide forecasts on the behaviour of tropical cyclones, their movements and changes in intensity and on associated phenomena - principally storm surges and flash floods.

The responsibility of the RSMC Tokyo - Typhoon Center is the provision of information on tropical cyclones for Members of the Typhoon Committee. Information should include formation, movement and development of tropical cyclones and associated meteorological phenomena. In addition, synoptic scale atmospheric situation which affects the behaviour of tropical cyclones should also be prepared by the RSMC Tokyo - Typhoon Center and disseminated to National Meteorological Centers (NMCs) in the appropriate format for operational processing. The RSMC Tokyo - Typhoon Center should be operational throughout the year and be manned round the clock when a tropical cyclone exists over the region concerned. The RSMC Tokyo - Typhoon Center should also carry out non-operational functions such as training.

In order to implement the RSMC Tokyo - Typhoon Center in the Typhoon Committee region, the Regional Co-operation Programme was discussed and adopted by the Typhoon Committee at its Extraordinary Session (Manila, March 1986). At the same time, the Committee approved a draft of the Typhoon Committee Operational Manual which specifies in more detail the extent and type of activity of the RSMC Tokyo - Typhoon Center and shows the direction of realizing the regional co-operation between Members.

The Operational Manual consists of the text and the appendices. Items included in the text relate to the Typhoon Committee agreement, in particular, basic information for executing meteorological operation, whilst the appendices contain national practices and procedures (it is felt that the Member concerned should have the right to be able to change without having to get prior formal agreement of the Typhoon Committee) together with detailed and technical information for meteorological operation. Information described in WMO official publications such as Manuals is only referred to and not included in this Manual.

Since March 1986, the draft of the Operational Manual has been revised and is still subject to further refinement and revision through experience gained in the use of the Operational Manual. It is also intended that the text of the Manual be updated or revised from time to time by the Typhoon Committee and that each item of information given in the appendices relating to the Manual be kept up to date by the Members concerned.

##### 1.2	Terminology used in the region

###### 1.2.1	General

Typhoon Committee Members

###### 1.2.2	Classification of tropical cyclones

(i)	Low pressure area				(L)

(ii)	Tropical depression		(TD)

(iii)	Tropical storm					(TS)

(iv)	Severe tropical storm				(STS)

(v)	Typhoon					(TY)

###### 1.2.3	Tropical cyclone characteristics

(i)	position of centre

(ii)	confidence in the centre position

(iii)	size and shape of eye, if any

(iv)	central pressure

(v)	direction of movement

(vi)	speed of movement

(vii)	maximum sustained wind

(viii)	gusts

(ix)	storm radius

(x)	gale radius

(xi)	storm surge potential for a particular coastal location

(xii)	storm tide potential for a particular coastal location

###### 1.2.4	Terms related to the warning and warning system

(i)	typhoon season

(ii)	tropical cyclone advisory

(iii)	tropical cyclone information bulletin

(iv)	gale warning

(v)	storm warning

(vi)	typhoon warning

(vii)	visual storm signals

(viii)	high sea bulletin

(ix)	coastal weather bulletin

(x)	bulletin or cyclone warning bulletin

##### 1.3	Meaning of terms used for regional exchange

Astronomical tide: An Astronomical tide refers to the rise and fall of water due solely to gravitational interactions between the Earth, Moon, and Sun.

Average wind speed: Speed of the wind averaged over the previous 10 minutes (mean surface wind) as read from the anemogram or the 3 minutes mean determined with the non-recording anemometer or wind averaged over the previous 1 minute (mean surface wind) at 10 meter height or estimated wind at sea by mariners using the Beaufort scale.

Bulletin: Cyclone warning bulletin

Central pressure of a tropical cyclone: Surface pressure at the centre of the tropical cyclone as measured or estimated.

Centre fix of the tropical cyclone: The estimated location of the centre of a tropical cyclone.

Centre of the tropical cyclone: The centre of the cloud eye, or if not discernible, of the wind/pressure centre.

Confidence in the centre position: Degree of confidence in the centre position of a tropical cyclone expressed as the radius of the smallest circle within which the centre may be located by the analysis. "Position good" implies a radius of 30 nautical miles (55 kilometres) or less. "Position fair", a radius of 30 to 60 nautical miles (55 to 110 km) and "Position poor", a radius of greater than 60 nautical miles (110 km).

Cyclone: Tropical cyclone

Cyclone warning bulletin: A priority message for exchange of tropical cyclone information and advisories.

Direction of movement of the tropical cyclone: The direction towards which the centre of the tropical cyclone is moving.

Extra-tropical cyclone: A former tropical cyclone that has gone through extra-tropical transition and lost its initial tropical characteristics.

Extra-tropical transition: is an evolutionary process by which a symmetric warm core tropical cyclone transforms to an asymmetric cold core extratropical cyclone. This process includes a change in the distribution of clouds, winds, and precipitation. Also, the primary energy source changes from latent heat release in deep convective clouds of the tropical cyclone to baroclinic conversion of available potential energy in the extratropical cyclone.

Eye of the tropical cyclone: The relatively clear and calm area inside the circular wall of convective clouds, the geometric centre of which is the centre of the tropical cyclone.

Gale force: Average wind speed in the range of 34 knots (17.2 m/s, 62 km/h) to 47 knots (24.4 m/s, 88 km/h), or wind force 8 or 9 in the Beaufort scale.

Gale-force wind warning: Meteorological message intended to warn those concerned of the occurrence or expected occurrence of gale force wind.

Gust: Instantaneous peak value of surface wind speed.

Hurricane force: Average wind speed of 64 knots (32.7 m/s, 118 km/h) and above, or wind force 12 in the Beaufort scale.

Hurricane-force wind warning: Meteorological message intended to warn those concerned of the occurrence or expected occurrence of hurricane-force wind.

Low pressure area: Region of the atmosphere in which the pressures are lower than those of the surrounding region at the same level. (On the weather map, the low pressure area is denoted with the capital L within the innermost isobar without showing the centre position.)

Maximum sustained wind: Maximum value of the average wind speed at the surface.

Mean wind speed: Average wind speed.

Meteorological tide: A meteorological tide is the rise and fall of water due to wind and fluctuations in atmospheric pressure.

Reconnaissance aircraft centre fix of the tropical cyclone, vortex fix: The location of the centre of a tropical cyclone obtained by reconnaissance aircraft penetration.

Severe tropical storm: A tropical cyclone with the maximum sustained winds at storm force near the centre.

Speed of movement of the tropical cyclone: Speed of movement of the centre of the tropical cyclone.

Storm force: Average wind speed of 48 knots (24.5 m/s, 89 km/h) to 63 knots (32.6 m/s, 117 km/h), or wind force 10 or 11 in the Beaufort scale.

Storm-force wind warning: Meteorological message intended to warn those concerned of the occurrence or expected occurrence of storm force wind.

Storm surge: The difference between the actual water level under the influence of a tropical cyclone or developing disturbance (storm tide) and the level which would have been attained in the absence of the meteorological disturbance (i.e. astronomical tide). (Storm surge results mainly from the shoreward movement of water under the action of wind stress. A minor contribution is also made by the hydrostatic rise of water resulting from the lowered barometric pressure.)

Storm tide: The actual sea level as influenced by a weather disturbance. The storm tide consists of the normal astronomical tide and the storm surge.

Sub-tropical cyclone: A low pressure system, developing over sub-tropical waters which initially contains few tropical characteristics. With time the sub-tropical cyclone can become tropical.

Sustained wind speed: Average wind speed. Average period of one, three or ten minutes is depending upon the regional practices.

Tropical cyclone: Generic term for a non-frontal synoptic scale cyclone originating over tropical or sub-tropical waters with organized convection and definite cyclonic surface wind circulation. (The term is also used for a storm in the South-West Indian Ocean in which the maximum of the sustained wind speed is estimated to be in the range of 64 to 90 knots and in the South Pacific and South-East Indian Ocean with the maximum of the sustained over 33 knots.)

Tropical cyclone advisory: A priority message for exchanging information, internationally, on tropical cyclones.

Tropical cyclone coastal crossing: Cyclone centre passage across the coast.

Tropical depression: A tropical cyclone with the maximum sustained winds of 33 knots (17.1 m/s, 61 km/h) or less near the centre.

Tropical disturbance: A non-frontal synoptic scale cyclone originating in the tropics or sub-tropics with enhanced convection and light surface winds.

Tropical cyclone impact: Evidence of damage or disruption caused by tropical cyclone-generated hazard(s) either direct or indirect. (includes damaging large swells from distant tropical cyclones).

Tropical cyclone island crossing: Cyclone centre passage across the island.

Tropical cyclone landfall: refer to tropical cyclone coastal crossing.

Tropical storm: A tropical cyclone with the maximum sustained winds at gale force near the centre.

Tropical wave: A trough or cyclonic curvature maximum in the trade wind easterlies or equatorial westerlies. The wave may reach maximum amplitude in the lower middle troposphere, or may be the reflection of an upper-troposphere cold low or equatorial extension of a mid-latitude trough.

Typhoon: A tropical cyclone with the maximum sustained winds at typhoon force near the centre.

Typhoon force: Average wind speed of 64 knots (32.7 m/s, 118 km/h) or more, or wind force 12 in the Beaufort scale.

Typhoon warning: Meteorological message intended to warn those concerned of the occurrence or expected occurrence of typhoon force wind.

Visual storm signals: Visual signals displayed at coastal points to warn ships of squally winds, gales and tropical cyclones.

Weather warning: Meteorological message issued to provide appropriate warnings or hazardous weather conditions.

Zone of disturbed weather: A zone in which the pressure is low relative to the surrounding region and there are convective cloud masses which are not organized.

##### 1.4	Units used for regional exchange

(a)	The following units/indicators are used for marine (WWMIWS) purposes, in accordance with the WMO Manual on Marine Meteorological Services (WMO No.558):

- Distance in nautical miles, the unit (nm) being stated;

- Location (position) by degrees and where possible tenths of degrees of latitude and longitude preferably expressed in numbers e.g.	"12.2S, 168.4E";

- Direction of motion to the nearest sixteen points of the compass or in degree to the nearest ten, given in figures, e.g. "SOUTHSOUTHEAST" or "160 DEGREES";

- Speed (wind speed and speed of movement of tropical cyclones) in knots, the unit (kt) being stated;

- Confidence in the centre position in nautical miles (nm) or in position good, fair or poor;

- Pressure in hectopascals (hPa), the unit being stated;

- Time in Coordinated Universal Time (UTC), the unit being stated.

(b)	The following units/indicators are used in non-coded segments of exchanges, other than marine bulletins:

- Distance in kilometres (km) or nautical miles (nm);

- Location (position) by degrees and tenths of degrees in figures of latitude and longitude and/or bearing on the sixteen point compass and distance from well-known fixed place(s);

- Direction in sixteen points of compass given in figures;

- Speed (wind speed and speed of movement of system) in knots (kt), metres per second (m/s) or kilometres per hour (km/h);

- Confidence in the centre position in kilometres (km), nautical miles (nm) or in position good, fair or poor.

##### 1.5	Identification of tropical cyclones

As soon as the wind speed in a tropical cyclone in the responsible area of the RSMC Tokyo - Typhoon Center (between 0°N and 60°N and between 100°E and 180°E) attains 34 knots, it will be given an identification name with a 4-digit number by the RSMC Tokyo - Typhoon Center. Each tropical cyclone should be identified by one of the names in Appendix 1-B, followed by the 4-digit number in brackets, whose number will consist of a year identification and a serial number identification (in two digits each). For example, the first tropical cyclone attaining the 34 knots threshold value in 2000 in the responsible area of the RSMC Tokyo - Typhoon Center was identified as Damrey (0001). If the life of a tropical cyclone spans across two calendar years, it will be accounted for in the year in which it has intensified to the stage where the wind speed has attained the 34 knots threshold value.

##### 1.6	Acronyms

A list of acronyms used in this Operational Manual is shown in Appendix 1-C.

_______________

## CHAPTER 2  OBSERVING SYSTEM AND OBSERVING PROGRAMME

11

##### 2.1	Networks of synoptic land stations

The surface and upper-air stations in the regional basic synoptic network are those of the Typhoon Committee Members and are registered in OSCAR/Surface.

The RSMC Tokyo - Typhoon Center and all Typhoon Committee Members should initiate enhanced observation programmes for their stations in the area within 300 km of the centre of a tropical cyclone of TS intensity or higher. All the observations should be made available to the RSMC Tokyo - Typhoon Center and all Members. Enhanced observations should include:

- surface observations - hourly;

- buoy observations - hourly;

- radar observations - hourly;

- upper-air observations - 6-hourly.

###### 2.1.1	Surface observations

All surface stations included in the regional basic synoptic network should make surface observations at the four main standard times of observation, i.e., 0000, 0600, 1200 and 1800 UTC, and at the four intermediate standard times of observation, i.e., 0300, 0900, 1500 and 2100 UTC. Any surface station that cannot carry out the full observational programme should give priority to carrying out the observations at the main standard times. Additional surface observations at hourly intervals may be requested by any Member, whenever a tropical cyclone becomes an imminent threat to the Member, from the stations shown in Appendix 2-A.

###### 2.1.2	Upper-air synoptic observations

All the upper-air stations included in the regional basic synoptic network should carry out radiosonde and radiowind observations at 0000 and 1200 UTC, and radiowind/wind profiler observations at 0600 and 1800 UTC. The radiosonde/radiowind observations carried out at 0000 and 1200 UTC should reach the 30 hPa level for more than 50 per cent of the ascents. The carrying out of the radiosonde/radiowind observations at 0000 and 1200 UTC should receive priority over the radiosonde/radiowind/wind profiler observations at 0600 and 1800 UTC.

Upper-air stations in the areas affected by tropical cyclones of TS intensity or higher should also make radiowind observations at 0600 and 1800 UTC which should aim at reaching the 70 hPa level.

Enhanced upper-air observations given in Appendix 2-B will be made as appropriate whenever a tropical cyclone of TS intensity or higher is centred within 300 km of the station. The minimum required is two observations per day, but for a better understanding of the ambient wind field three or even four ascents per day on some days should be made when possible. All data of these enhanced upper-air observations will be distributed among the Members.

In addition to the upper-air synoptic observations, other upper air wind observations such as wind profiler observations should be carried out when possible and the data should be made available to the Members.

##### 2.2	Ship and buoy observations

Hourly marine meteorological observations are made by the JMA research vessels (call signs of them are: JPBN and 7KPB) in the seas adjacent to Japan and in the western North Pacific.

Upper-air observations are usually made twice a day (00, 12UTC) on board the JMA research vessels JPBN and 7KPB. Enhanced upper-air observations are carried out sixhourly when the vessel is in the vicinity of a tropical cyclone of TS intensity or higher.

Marine meteorological observations are made by the Voluntary Observing Ships which are recruited by the Members in accordance with the Voluntary Observing Ship Scheme of the Global Ocean Observing System (GOOS). These are generally carried out every six hours and transmitted over the GTS. In addition, marine meteorological observations are reported hourly by on-board automatic weather stations on some of the Voluntary Observing Ships.

Marine meteorological observations, such as air pressure, sea surface temperature, significant wave height and period, are also made by the drifting ocean data buoys by the Members. All reports are coded in the BUFR code (FM-94) with drifting buoys Template (TM315009), and immediately put onto the GTS. A list of the drifting buoy observations by the Members is shown in Appendix 2-C.

##### 2.3	Radar observations

It is essential that radar observations continue as long as a tropical cyclone of TS intensity or higher remains within the detection range of the radar. All meteorological centres should co-operate to ensure that the radar observations are transmitted through the GTS to the RSMC Tokyo - Typhoon Center and all Members. Reports will be coded in the BUFR code (FM-94) with RADOB Template (TM316050) and/or the RADOB code (FM 20-VIII).

In case the report is in plain language, the full range of information available at the radar station should be given. The message will therefore include, where available, the confirmation of the determination of the centre, the shape, definition, size and character tendency of the eye, the distance between the end of the outermost band and the centre of the cyclone and the direction and speed of movement with a statement of the interval of time over which the movement was calculated.

Distribution of the radar stations and detailed information on the radar equipment of the Typhoon Committee Members are given in Appendices 2-D and 2-E.

##### 2.4	Meteorological satellite observations

###### 2.4.1	Satellite imagery data and related products

Satellite imagery data and related products are essential for monitoring and analyzing tropical cyclones. Members which operate satellites are expected to provide those data and/or products to the Members. CMA, JMA and KMA currently operate geostationary meteorological satellites, and have been providing their imagery data and related products to the users of the western North Pacific and the South China Sea region to support their operations on tropical cyclones.

Detailed information on the satellites operated by Typhoon Committee Members is given in Appendix 2-F.

A list of satellite imagery receiving facilities at meteorological centres of the Typhoon Committee Members is given in Appendix 2-G.

###### 2.4.2	SAREP reports

SAREP reports (Part A) are disseminated eight times a day in the following cases from the RSMC Tokyo - Typhoon Center to Typhoon Committee Members through the GTS under the heading of IUCC10 RJTD in the BUFR code (FM 94):

- when a tropical cyclone of TS intensity or higher is located in the responsible area of the RSMC Tokyo - Typhoon Center;

- when a tropical depression existing in the responsible area is forecasted to have an intensity of TS or higher within 24 hours; or

- when an area of wind speed of 34 knots or higher caused by a tropical cyclone is forecasted to be in the responsible area within 24 hours.

SAREP reports are also issued by other Typhoon Committee Members. A list of SAREP reports issued by the RSMC Tokyo - Typhoon Center and other Typhoon Committee Members is shown in Appendix 2-H.

##### 2.5	Aircraft observations

States within the ICAO Asia and Pacific Regions exchange reports from aircraft in flight prepared in conformity with ICAO requirements for meteorological reporting (known as air-reports or AIREPs) in accordance with the Regional OPMET Bulletin Exchange (ROBEX) scheme.

AIREPs in the north-east Pacific area are also collected by the centres at Honolulu, Washington, etc., and relayed to Tokyo.

AMDAR (Aircraft Meteorological Data Relay) reports are collected by the NMHSs involved in respective AMDAR Programmes and relayed via the GTS to the centre at Tokyo.

All reports will be disseminated in real-time to the RSMC Tokyo - Typhoon Center and to other Members through GTS and AFTN/AMHS circuits.

The Members conduct reconnaissance flights for selected tropical cyclones. Detailed information of reconnaissance flights conducted by the Members is given in Appendix 2-I.

_______________

## CHAPTER 3  TROPICAL CYCLONE ANALYSIS AND FORECAST

13

##### 3.1	Analysis at RSMC Tokyo - Typhoon Center

The RSMC Tokyo - Typhoon Center should produce analyses of various meteorological parameters in chart form and/or in grid point value depending on the facilities of NMCs to process these products. These analyses should include pressure distribution at the sea level and temperature, geo-potential height, humidity and wind at selected pressure levels.

The streamline analysis is indispensable over the tropical region for forecasting tropical cyclones. The RSMC Tokyo - Typhoon Center should produce streamline analyses of the upper and lower atmospheric levels utilizing cloud motion wind, aircraft reports, as well as upper-air observations. Furthermore, the RSMC Tokyo - Typhoon Center should issue analyses of ocean wave and sea surface temperature for the western North Pacific. A list of products provided by the RSMC Tokyo - Typhoon Center is given in Appendix 3-A.

The RSMC Tokyo - Typhoon Center should produce additional analyses of the tropical cyclone when it is in the responsible area, based on the enhanced observations. Such analyses should be disseminated in the form of additional bulletins consisting of information on:

- position of the tropical cyclone;

- direction and speed of movement;

- central pressure;

- maximum wind and wind distribution.

Various analyses based on Himawari data other than cloud imagery itself should be produced by the RSMC Tokyo - Typhoon Center. Analysis of sea surface temperature combining satellite data and in-situ measurements should be prepared every day. These analyses are useful for the better understanding of the tropical atmosphere and medium-range assessment of forecasting tropical cyclones.

##### 3.2	Forecast at RSMC Tokyo - Typhoon Center

The RSMC Tokyo - Typhoon Center should prepare the products for numerical weather prediction shown in the WMO Manual on the WMO Integrated Processing and Prediction System (WIPPS) (WMO-No.485). These products should be made available to Members in real-time, and should include the following:

(i) deterministic forecast products of a high resolution global model to predict the change in large-scale atmospheric circulation patterns as well as the tropical cyclone movement and intensity

(ii) ensemble forecast products using a lower resolution version of the global model to enable estimation of uncertainties in tropical cyclone movement and intensity as well as to reduce forecast errors by using statistical methods such as ensemble mean.

The RSMC Tokyo - Typhoon Center should also prepare several statistical models for predicting the track of the tropical cyclone and apply the Dvorak method for the prediction of the intensity change of the tropical cyclone. Other relevant synoptic methods should also be applied for predicting the tropical cyclone.

The RSMC Tokyo - Typhoon Center should summarize in a consolidated form all available information and prepare the final forecasts of the tropical cyclone when it exists in the responsible area. These forecasts should include:

- 24, 48, 72, 96 and 120-hour forecast position;

- 24, 48, 72, 96 and 120-hour forecast intensity and wind distribution;

- prognostic reasoning;

- tendency assessment if possible.

Furthermore, the RSMC Tokyo - Typhoon Center should prepare a 24-hour ocean wave forecast twice a day for the western North Pacific. Storm surge products suitable for the Typhoon Committee region should be provided by the RSMC Tokyo - Typhoon Center. A list of forecast products of the RSMC Tokyo - Typhoon Center, other than alphanumeric form, is shown in Appendix 3-A.

##### 3.3	Operational analysis and forecast at centres of Typhoon Committee Members

The NMSs of Typhoon Committee Members are performing analysis and forecasting development and movement of tropical cyclones in the region. The analysis methods, the forecasting methods and NWP systems for forecasting currently used by the NMSs of Typhoon Committee Members are given in Appendix 3-B.

The final responsibility for the operational analysis and forecasting will be with the NMSs of each of the Members.

_______________

## CHAPTER 4  TROPICAL CYCLONE WARNINGS AND ADVISORIES

17

##### 4.1	General

The responsibility for warning the human settlements on land which are threatened by a tropical cyclone rests in all cases with the NMSs. These national responsibilities are not subject to regional agreement. Therefore, only the cyclone warning systems intended for international users and exchanges among the Typhoon Committee Members are described in this chapter.

##### 4.2	Classification of tropical cyclones,

Classifications of tropical cyclones for the exchange of messages among the Typhoon Committee Members are given below:

| (i) Low pressure area | (L) | Central position cannot be accurately assessed. |
| --- | --- | --- |
| (ii) Tropical depression | (TD) | Central position can be identified, but the maximum sustained wind is 33 kt or less. |
| (iii) Tropical storm | (TS) | Maximum sustained wind is between 34 and 47 kt. |
| (iv) Severe tropical storm | (STS) | Maximum sustained wind is between 48 and 63 kt. |
| (v) Typhoon | (TY) | Maximum sustained wind is 64 kt or more. |

##### 4.3	Tropical cyclone advisories

The RSMC Tokyo - Typhoon Center should disseminate six to three-hourly analyses and forecasts of tropical cyclones in the form of bulletins (tropical cyclone advisories - see examples in Appendix 4-B):

- analysis of the central position, intensity and wind distribution;

- 24, 48, 72, 96 and 120-hour forecasts of the central position;

- 24, 48, 72, 96 and 120-hour forecasts of intensity and wind distribution;

- prognostic reasoning;

- tendency assessment if possible.

##### 4.4	Tropical cyclone warnings for the high seas (WWMIWS)

The IMO/WMO Worldwide Met-Ocean Information and Warning Service (WWMIWS) is the internationally coordinated service for the promulgation of meteorological warnings and forecasts.

The WWMIWS produces marine meteorological maritime safety information messages for issuance on Enhanced Group Call (EGC) satellite systems, NAVTEX and High-frequency Narrow-band Direct Printing (HF NBDP) communication systems covering the following areas:

- warnings and forecasts for the High Seas;

- warnings and forecasts for coastal, offshore and local waters (including ports, lakes and harbour areas).

Operational guidance for handling and formatting meteorological information is given in detail in the Annex VI of the WMO Technical Regulations (Manual on Marine Meteorological Services - WMO-No. 558).

The provision of warnings for weather systems that produce average wind speeds of 34 knots and greater is a mandatory requirement of the WWMIWS.

In relation to international marine requirements, the WWMIWS coordinates the broadcast of forecasts and warnings to vessels at sea through the Global Maritime Distress and Safety System (GMDSS), which includes EGC satellite communications.

As part of the WWMIWS coordination, there are the following types of Centres:

Issuing service means a National Meteorological Service which has accepted responsibility for ensuring that meteorological warnings and forecasts for shipping are disseminated through approved EGC satellite systems to the designated area (METAREA) for which the Service has accepted responsibility under the WWMIWS.

Preparation service means a National Meteorological Service which has accepted responsibility for the preparation of warnings and forecasts for parts of or an entire designated area (METAREA) in the WMO system for the dissemination of meteorological forecasts and warning to shipping under the WWMIWS and for their transfer to the relevant Issuing Service for broadcast.

The METAREA Coordinator is responsible for ensuring that Tropical Cyclone warnings for the WWMIWS in their METAREA are issued onto the appropriate GMDSS communication system.

Areas of responsibility

Members having official responsibility as an Issuing Service within the WWMIWS for issuing warnings on approved EGC satellite systems are Japan and China (METAREA XI).

The pre-assigned forecast areas of Typhoon Committee Members were agreed upon by Regional Associations II and V (Res. 17 (IV-RA II; WMO-181, 1966) and Res.10 (IV-RA V; WMO-187, 1966)). Weather forecast areas fixed nationally by individual Typhoon Committee Members are shown in WMO Publication No. 9, Weather Reporting Volume D - Information for Shipping.

Format and content of bulletins

The format and content of warnings issued for the WWMIWS, as outlined below, has been derived from guidance provided in the Manual on Marine Meteorological Services (WMO No.558).

Tropical Cyclone warnings for the WWMIWS shall use the following wind warning category labels:

  - Gale force wind warning (Beaufort force 8 or 9);

  - Storm-force wind warning (Beaufort force 10 or 11);

  - Typhoon-force/Hurricane-force wind warning (Beaufort force 12 or over).

Any Tropical Cyclone related wind warning issued for the WWMIWS should include the following content (excluding any relevant system metadata requirements):

- Header label for marine radio broadcast purposes (“SECURITE”)

Note: This label needs to be visible on any product provided to mariners with the potential to be read out on marine radio systems.

- Type of wind warning (GALE-FORCE, STORM-FORCE, TYPHOON-FORCE/HURRICANE-FORCE WIND WARNING)

- Name of the issuing centre

- Name of the system and name of the basin

- Date and time of reference in UTC

- Type of disturbance (Tropical cyclone)

- Location of disturbance (latitude and longitude)

- Central pressure (hPa)

- Intensity (maximum 10-minute average winds in knots)

- Direction and speed of movement of the disturbance

- Extent of affected area in nautical miles

- Wind speed (knots) and direction in the affected areas

- Sea and swell condition in affected areas (in qualitative terms)

- Expected location and intensity at 12 or 24 hour time periods.

- Indication of when next warning will be issued.

When no more warnings are to be issued, that fact shall be stated in the bulletins.

The radio stations broadcasting tropical cyclone forecasts and warnings for the benefit of the ships on the high seas in the Typhoon Committee Members are listed in Appendix 4-C, where are shown the names of coastal radio stations with their call signs and the area covered by their bulletins. The details are shown in WMO Publication No. 9, Weather Reporting Volume D - Information for Shipping.

##### 4.5	Tropical cyclone SIGMET and advisory information for international aviation

In accordance with the International Civil Aviation Organization (ICAO) Annex 3 - Meteorological Service for International Air Navigation, SIGMET is information issued by a (designated) meteorological watch office (MWO) concerning the occurrence or expected occurrence of specified en-route weather and other phenomena in the atmosphere (including tropical cyclone) that may affect the safety of aircraft operations and of the development of those phenomena in time and space.

Each designated MWO is required to maintain continuous watch over meteorological conditions affecting flight operations within one or more designated flight information regions (FIRs) and prepare, supply and disseminate SIGMET information (including for tropical cyclone as necessary) relating to its designated area of responsibility. Each designated MWO should coordinate SIGMET with neighbouring MWO(s), especially when the en-route weather phenomenon extends or is expected to extend beyond the MWO’s specified area of responsibility, in order to ensure harmonized SIGMET provision. The ICAO Asia and Pacific Regions Air Navigation Plan (APAC ANP) describes the FIRs in the Asia and Pacific Regions and lists the designated MWOs and the requirements for the issuance of SIGMET information (including for tropical cyclone).

SIGMET information (for tropical cyclone) shall be prepared, formatted and disseminated in accordance with ICAO Annex 3 and should be based on advisory information provided by Tropical Cyclone Advisory Centres (TCACs) designated by regional air navigation agreement. The data type designator to be used in the WMO abbreviated heading of such messages shall be T1T2 = WC (WMO-No. 386, Manual on GTS refers).

In accordance with ICAO Annex 3 and the ICAO Asia and Pacific Regions Air Navigation Plan, the designated TCAC Tokyo shall:

- monitor the development of tropical cyclones in its area of responsibility;

- issue advisory information concerning the position of the cyclone centre, its direction and speed of movement, changes in intensity at time of observation, central pressure and maximum surface wind near the centre, in abbreviated plain language to:

  - MWOs in its area of responsibility;

  - other TCACs whose areas of responsibility may be affected; and

  - World Area Forecast Centres (WAFCs) [London and Washington], and international OPMET databanks; and

- issue updated advisory information to MWOs for each tropical cyclone, as necessary, but at least every six hours.

The tropical cyclone advisory information shall be prepared, formatted and disseminated in accordance with the technical specifications and detailed criteria in ICAO Annex 3 and PANS-MET (Doc10157). The data type designator to be used in the WMO abbreviated heading of such messages shall be T1T2 = FK (WMO-No. 386, Manual on GTS, refers).

____________

## CHAPTER 5  TELECOMMUNICATIONS

19

##### 5.1	General

The basic meteorological telecommunication network for the exchange of forecasts, warnings and observational data will be the Global Telecommunication System (GTS).

Note: With respect to meteorological service for international air navigation (as described in sections 2.5 and 4.5), the telecommunications facilities used for the exchange of operational meteorological information should be the aeronautical fixed service (AFS).

##### 5.2	Dissemination of data and products

The RSMC Tokyo - Typhoon Center should have adequate telecommunication facilities for the real-time collection and dissemination of data and products. A large amount of grid point data produced at the RSMC Tokyo - Typhoon Center should be exchanged between the RSMC Tokyo - Typhoon Center and NMCs where adequate circuits for this purpose exist, such as GTS and Internet.

Conventional radio facsimile broadcasts are widely used in the region, though they have some disadvantages, i.e., it takes a long time to transmit a number of charts and received charts are sometimes distorted due to noises. Nevertheless, facsimile broadcasts and reception facilities shall be retained in full operation until telecommunications via satellite is introduced to transmit products both in chart and in grid point value form.

##### 5.3	Schedule for exchange of cyclone advisories

Tropical cyclone advisories issued by the RSMC Tokyo - Typhoon Center shall be transmitted at intervals of six to three hours. These messages shall be given high priority.

##### 5.4	Meteorological telecommunication network for the Typhoon Committee region

The network is shown in Appendix 5-A and its present status is summarized in Appendix 5-B.

##### 5.5	Addresses, telex/cable and telephone numbers of the tropical cyclone warning centres

A list of addresses of the tropical cyclone warning centres of the Typhoon Committee Members, together with their telex/cable and telephone numbers and e-mail addresses, is given in Appendix 5-C.

##### 5.6	Abbreviated headings of tropical cyclone advisories and warnings

The abbreviated headings of meteorological messages containing tropical cyclone advisories issued by the RSMC Tokyo - Typhoon Center shall be:

- prognostic reasoning - WTPQ30 RJTD through WTPQ35 RJTD;

- analysis and five-day forecast - WTPQ50 RJTD through WTPQ55 RJTD;

- numerical prediction by global deterministic model - FXPQ20 RJTD through FXPQ25 RJTD;

- numerical prediction by global ensemble model - FXPQ30 RJTD through FXPQ35 RJTD.

The abbreviated headings of meteorological bulletins used for the exchange of tropical cyclone warnings by the Typhoon Committee Members are given in Appendix 5-D.

##### 5.7	Exchange of information related to tropical cyclones

Collection and dissemination of observational and processed data plus warnings related to tropical cyclones at Regional Telecommunication Hubs (RTHs) and NMCs are summarized in Appendix 5-E.

The meanings of the symbols used in abbreviated headings in the meteorological messages transmitted to the GTS are listed in Appendix 5-F. The details are described in the Manual on the Global Telecommunication System (WMO Publication No. 386) and Weather Reporting Volume C - Transmissions, Chapter I Catalogue of Meteorological Bulletins (WMO Publication No. 9).

______________

## CHAPTER 6  MONITORING AND QUALITY CONTROL OF DATA

20

##### 6.1	Quality control of observational data

NMCs will make additional efforts to ensure that all observational data disseminated during periods of cyclone threat to the area are specifically free from errors. Wherever appropriate, verification of reports or of elements of reports will be requested of the observing station and communication channels will be kept open to facilitate this, particularly in cases where an enhanced observing programme is being carried out.

In the exchange of data during periods of cyclone threat, queries concerning reports on which there is doubt should be addressed to the relevant NMC.

Examples of message format for inquiry on doubtful and garbled reports are shown in Appendix 6-A.

##### 6.2	Monitoring of exchange of information

Monitoring will be carried out by the RSMC Tokyo - Typhoon Center and all Typhoon Committee Members in accordance with their standard procedures. Special attention will be given to identification of deficiencies during the typhoon season in the flow of observational data and processed information relating to cyclone analysis and forecast with a view to appropriate remedial action.

The Members will inform the RSMC Tokyo - Typhoon Center of any shortcomings in the flow of data (raw and processed) and also indicate any requirements over and above those already agreed upon for tropical cyclone warning purposes.

##### 6.3	Verification

Immediately after the dissipation of a tropical cyclone of TS grade or stronger, the RSMC Tokyo - Typhoon Center should report on the tropical cyclone to provide Members with data needed for verification, such as position and intensity of the tropical cyclone.

After the end of each typhoon season, each Member will conduct the verification for its analyses and forecasts and send the report to the RSMC Tokyo - Typhoon Center in accordance with the standard procedure as shown in Appendix 6-B. Verification sheets for positioning of the centre, prediction of movement, and analysis and forecast of intensity of a tropical cyclone are shown in Appendix 6-C.

The RSMC Tokyo - Typhoon Center should summarize the reports issued in a year and the results of verification conducted by Members. It should publish an annual report with respect to tropical cyclones and activities of the RSMC Tokyo - Typhoon Center and Members. The report should also identify specific areas where further co-operative research needs to be carried out by Members.

_______________

## CHAPTER 7  ARCHIVAL OF DATA

21

##### 7.1	Data to be archived by Typhoon Committee Members

Members should establish tropical cyclone data files and information services nationally, archiving all appropriate available data.

##### 7.2	Data to be archived by RSMC Tokyo - Typhoon Center

The RSMC Tokyo - Typhoon Center should archive as far as possible tropical cyclone related data received at the centre. The data set should be produced during the period when tropical cyclone(s) is (are) in the range of 1,000 km around Typhoon Committee Members. Except for satellite imagery data, all data should be recorded by the RSMC Tokyo - Typhoon Center preferably on electronic media. A proposed list of data to be archived by the RSMC Tokyo - Typhoon Center is shown in Appendix 7-A.

##### 7.3	Exchange of archived data

Whenever possible Members should supply the RSMC Tokyo - Typhoon Center with all additional data requested by the RSMC Tokyo - Typhoon Center. The RSMC Tokyo - Typhoon Center should make available the archived data to Members on request for use in research, studies, investigations and training. As to distribution, similar arrangements should be made as for the TOPEX data sets which were provided by the Japan Meteorological Agency to Typhoon Committee Members (one set each) with financial assistance from UNDP. The detailed arrangements for exchange of data should be agreed upon bilaterally. Request for data sets by non-Typhoon Committee Members should be made through the WMO Secretariat upon payment of net cost (for electronic media, copying, handling, postal fees, etc.) by the requesting WMO Members.

In accordance with the directive of the WMO Executive Council (EC-XLV), (Geneva, June 1993) an international format for the archiving of tropical cyclone data is to be used by all RSMCs with activity specialization in tropical cyclones.

Complete historical data using the international format given in Appendix 7-B will be made available for research applications. RSMC Tokyo - Typhoon Center will provide such data to the Director of the National Climatic Data Center (NCDC), USA.

The WMO Secretariat has the responsibility for the maintenance of the format, including assignment of the source codes to appropriate organizations, and authorizing additions and changes.

_______________

## CHAPTER 8  CAPACITY DEVELOPMENT

22

##### 8.1	Tropical Cyclone Forecast Competency in the Typhoon Committee Region

Tropical Cyclone Forecast Competency in the Typhoon Committee Region is shown in Appendix 8-A.

##### 8.2	Capacity development activities conducted by RSMC Tokyo - Typhoon Center

The RSMC Tokyo - Typhoon Center should carry out capacity development activities in accordance with the Tropical Cyclone Forecast Competency in the Typhoon Committee Region.

##### 8.3	Capacity development activities conducted by Members

Members should establish and maintain capacity development strategy and conduct necessary training activities or give opportunities to participate in activities conducted by other centers, to develop, maintain and enhance capacity of staff members for tropical cyclone analysis, forecast and related activities, in accordance with the Tropical Cyclone Forecast Competency in the Typhoon Committee Region.

______________________________

## APPENDICES

### APPENDIX 1-A

25

#### GUIDELINES FOR CONVERTING BETWEEN VARIOUS WIND AVERAGING PERIODS IN TROPICAL CYCLONE CONDITIONS

This note is based on recommendations from Harper et al. (2010) and extracts from Knaff and Harper (2010), providing advice on why, when and how “wind averaging conversions” can be made.

##### a) Why Convert Wind Speeds?

From the observational perspective, the aim is to process measurements of the wind so as to extract an estimate of the mean wind at any time and its turbulence properties. From the forecasting viewpoint, the aim is, given a specific wind speed metric derived from a process or product, to usefully predict other metrics of the wind. Typically, these needs revolve around the concept of the mean wind speed and an associated peak gust wind speed; such that the statistical properties of the expected level of wind turbulence under different exposures can be used to permit useful conversions between peak gust wind speed estimates.

##### b) When to Convert Wind Speeds?

Wind speed conversions to account for varying averaging periods only apply in the context of a maximum (peak gust) wind speed of a given duration observed within some longer interval. Simply measuring the wind for a shorter period of time at random will not ensure that it is always higher than the mean wind (given that there are both lulls and gusts). It is important that all wind speed values be correctly identified as an estimate of the mean wind or an estimate of a peak gust.

Once the mean wind is reliably estimated, the random effects of turbulence in producing higher but shorter-acting wind gusts, typically of greater significance for causing damage, can be estimated using a “gust factor”. In order for a gust factor to be representative, certain conditions must be met, many of which may not be exactly satisfied during a specific weather event or at a specific location:

- Wind flow is turbulent with a steady mean wind speed (statistically stationary);

- Constant surface features exist within the period of measurement, such that the boundary layer is in equilibrium with the underlying surface roughness (exposure);

- The conversion assumes the mean wind speed and the peak gust wind speed are at the same height (e.g. the WMO standard observation height +10 m) above the surface.

##### c) How to Convert Individual Point-Specific Wind Speeds

Firstly, the mean wind speed estimate V should be explicitly identified by its averaging period To in seconds, described here as VTo , e.g.

V600	 is a 10-min averaged mean wind estimate;

V60	 is a 1-min averaged mean wind estimate;

V3	 is a 3-sec averaged mean wind estimate.

Next, a peak gust wind speed should be additionally prefixed by the gust averaging period  , and the time period over which it is observed (also termed the reference period), described here as V,To , e.g.

V60,600 	is the highest 1-min mean (peak 1-min gust) within a 10-min observation period;

V3,60	 is the highest 3-sec mean (peak 3-sec gust) within a 1-min observation period.

The “gust factor” G,To then relates as follows to the mean and the peak gust:

,

where the (true) mean wind V is estimated on the basis of a suitable sample, e.g. V600 or V3600.

On this basis, Table 1 provides the recommended near-surface (+10 m) conversion factors G,To between typical peak gust wind averaging periods, which are a strong function of the exposure class because the turbulence level varies depending on the surface roughness. Table 1 only provides a range of indicative exposures for typical forecasting environments and Harper et al. (2010) or WMO (2008) should be consulted for more specific advice regarding particular types of exposures - especially if it is intended to calibrate specific measurement sites to “standard exposure”.

Table 1  Wind speed conversion factors for tropical cyclone conditions (after Harper et al. 2010).

| Exposure at +10 m |  | Reference | Gust Factor G,To |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Description | Period | Gust Duration  (s) |  |  |  |  |
|  |  | To (s) | 3 | 60 | 120 | 180 | 600 |
| In-Land | Roughly open terrain | 3600 | 1.75 | 1.28 | 1.19 | 1.15 | 1.08 |
|  |  | 600 | 1.66 | 1.21 | 1.12 | 1.09 | 1.00 |
|  |  | 180 | 1.58 | 1.15 | 1.07 | 1.00 |  |
|  |  | 120 | 1.55 | 1.13 | 1.00 |  |  |
|  |  | 60 | 1.49 | 1.00 |  |  |  |
| Off-Land | Offshore winds at a coastline | 3600 | 1.60 | 1.22 | 1.15 | 1.12 | 1.06 |
|  |  | 600 | 1.52 | 1.16 | 1.09 | 1.06 | 1.00 |
|  |  | 180 | 1.44 | 1.10 | 1.04 | 1.00 |  |
|  |  | 120 | 1.42 | 1.08 | 1.00 |  |  |
|  |  | 60 | 1.36 | 1.00 |  |  |  |
| Off-Sea | Onshore winds at a coastline | 3600 | 1.45 | 1.17 | 1.11 | 1.09 | 1.05 |
|  |  | 600 | 1.38 | 1.11 | 1.05 | 1.03 | 1.00 |
|  |  | 180 | 1.31 | 1.05 | 1.00 | 1.00 |  |
|  |  | 120 | 1.28 | 1.03 | 1.00 |  |  |
|  |  | 60 | 1.23 | 1.00 |  |  |  |
| At-Sea | > 20 km offshore | 3600 | 1.30 | 1.11 | 1.07 | 1.06 | 1.03 |
|  |  | 600 | 1.23 | 1.05 | 1.02 | 1.00 | 1.00 |
|  |  | 180 | 1.17 | 1.00 | 1.00 | 1.00 |  |
|  |  | 120 | 1.15 | 1.00 | 1.00 |  |  |
|  |  | 60 | 1.11 | 1.00 |  |  |  |

Some example applications of the above recommendations are:

- To estimate the expected “off-land” 3-sec peak gust in a 1-min period, multiply the estimated “off-land” mean wind speed by 1.36

- To estimate the expected “off-sea” 3-sec peak gust in a 10-min period, multiply the estimated “off-sea” mean wind speed by 1.38

- To estimate an “at-sea” 1-min peak gust in a 10-min period, multiply the estimated “at-sea” mean wind speed by 1.05

Note that it is not possible to convert from a peak gust wind speed back to a specific time-averaged mean wind - only to the estimated true mean speed. Hence to estimate the “off-sea” mean wind speed given only a peak observed gust of 1-min duration ( = 60 s) measured in a 10-min period (To = 600 s), multiply the observed 1-min peak gust by (1/1.11) = 0.90. This does not guarantee that the estimated mean wind will be the same as the 10-min averaged wind at that time but, because the 10-min average is normally a reliable estimate of the true mean wind, it will likely be similar. In all cases, measurement systems should aim to reliably measure the mean wind speed and the standard deviation using a sample duration of not less than 10-min (WMO 2008), i.e. V600. Additional shorter averaging periods and the retaining of peak information should then be targeted at operational needs.

##### d) Converting Between Agency Estimates of Storm Maximum Wind Speed Vmax

This is a slightly different situation from converting a point specific wind estimate because the concept of a storm-wide maximum wind speed Vmax is a metric with an associated spatial context (i.e. anywhere within or associated with the storm) as well as a temporal fix context (at this moment in time or during a specific period of time). While it may be expressed in terms of any wind averaging period it remains important that it be unambiguous in terms of representing a mean wind or a peak gust. Agencies that apply the WMO standard 10-min averaged Vmax wind have always applied a wind-averaging conversion to reduce the maximum “sustained” 1-min wind value (a 1-min peak gust) that has been traditionally associated with the Dvorak method (Dvorak 1984, Atkinson and Holliday 1977). As noted in the previous section, it is technically not possible to convert from a peak gust back to a specific time-averaged mean wind - only to the estimated true mean wind speed. However, in Harper et al. (2010) a practical argument is made for nominal conversion between Vmax,60 and Vmax,600 values via an hourly mean wind speed reference, and the recommendations are summarised in Table 2.

It can be noted that the recommended conversion for at-sea exposure is about 5% higher than the “traditional” value of 0.88 (WMO 1993), which is more appropriate to an off-land exposure. This has special implications for the Dvorak method because “at sea” is the typical exposure of interest where such conversions have been traditionally applied.

Table 2  Conversion factors between agency estimates of maximum 1-min and maximum 10-min averaged tropical cyclone wind speed Vmax. (after Harper et al. 2010).

| Vmax,600=K Vmax,60 | At-Sea | Off-Sea | Off-land | In-Land |
| --- | --- | --- | --- | --- |
| K | 0.93 | 0.90 | 0.87 | 0.84 |

##### e) References

Atkinson, G.D., and C. R. Holliday, 1977: Tropical cyclone minimum sea level pressure/maximum sustained wind relationship for the Western North Pacific. Mon. Wea. Rev., 105, 421-427.

Dvorak, V.F., 1984: Tropical cyclone intensity analysis using satellite data. NOAA Tech. Rep. NESDIS 11, National Oceanic and Atmospheric Administration, Washington, DC, 47 pp.

Knaff, J.A. and B.A. Harper, 2010: Tropical cyclone surface wind structure and wind-pressure relationships. In: Proc. WMO IWTC-VII, World Meteorological Organization, Keynote 1, La Reunion, Nov.

Harper, B.A., J. D. Kepert, and J. D. Ginger, 2010: Guidelines for converting between various wind averaging periods in tropical cyclone conditions. World Meteorological Organization, TCP Sub-Project Report, WMO/TD-No. 1555.

WMO 1993: Global guide to tropical cyclone forecasting. Tropical Cyclone Programme Report No. TCP-31, World Meteorological Organization, WMO/TD - No. 560, Geneva.

WMO 2008: Guide to meteorological instruments and methods of observation. World Meteorological Organization, WMO-No. 8, 7th Ed, 681pp.

### APPENDIX 1-B

28

#### LIST OF NAMES FOR TROPICAL CYCLONES ADOPTED BY THE TYPHOON COMMITTEE FOR THE WESTERN NORTH PACIFIC OCEAN AND THE SOUTH CHINA SEA

(Valid as of 2025)

| Contributed by | I | II | III | IV | V |
| --- | --- | --- | --- | --- | --- |
|  | Name | Name | Name | Name | Name |
| Cambodia | Damrey | Kong-rey | Nakri | Krovanh | Trases |
| China | Tianma | Yinxing | Fengshen | Dujuan | Mulan |
| DPR Korea | Kirogi | Toraji | Kalmaegi | Surigae | Meari |
| Hong Kong, China | Yun-yeung | Man-yi | Fung-wong | Choi-wan | Tsing-ma |
| Japan | Koinu | Usagi | Koto | Koguma | Tokage |
| Lao PDR | Bolaven | Pabuk | Nokaen | Champi | Ong-mang |
| Macao, China | Sanba | Wutip | Penha | In-fa | Muifa |
| Malaysia | Jelawat | Sepat | Nuri | Cempaka | Merbok |
| Micronesia | Ewiniar | Mun | Sinlaku | Nepartak | Nanmadol |
| Philippines | Maliksi | Danas | Hagupit | Lupit | Talas |
| RO Korea | Gaemi | Nari | Jangmi | Mirinae | Hodu |
| Thailand | Prapiroon | Wipha | Mekkhala | Nida | Kulap |
| U.S.A. | Maria | Francisco | Higos | Omais | Roke |
| Viet Nam | Son-Tinh | Co-May | Bavi | Luc-Binh | Sonca |
| Cambodia | Ampil | Krosa | Maysak | Chanthu | Nesat |
| China | Wukong | Bailu | Haishen | Dianmu | Haitang |
| DPR Korea | Jongdari | Podul | Noul | Mindulle | Jamjari |
| Hong Kong, China | Shanshan | Lingling | Dolphin | Lionrock | Banyan |
| Japan | Yagi | Kajiki | Kujira | Tokei | Yamaneko |
| Lao PDR | Leepi | Nongfa | Chan-hom | Namtheun | Pakhar |
| Macao, China | Bebinca | Peipah | Peilou | Malou | Sanvu |
| Malaysia | Pulasan | Tapah | Nangka | Nyatoh | Mawar |
| Micronesia | Soulik | Mitag | Saudel | Sarbul | Guchol |
| Philippines | Cimaron | Ragasa | Narra | Amuyao | Talim |
| RO Korea | Jebi | Neoguri | Gaenari | Gosari | Bori |
| Thailand | Krathon | Bualoi | Atsani | Chaba | Khanun |
| U.S.A. | Barijat | Matmo | Etau | Aere | Lan |
| Viet Nam | Trami | Halong | Bang-Lang | Songda | Saobien |

##### Replaced names

| Aere | for | Kodo | (2002) | Rai | for | Fanapi | (2012) | Co-May | for | Lekima | (2021) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Morakot | for | Hanuman | (2002) | Hato | for | Washi | (2013) | Nongfa | for | Faxai | (2021) |
| Matmo | for | Chataan | (2004) | Ampil | for | Bopha | (2014) | Ragasa | for | Hagibis | (2021) |
| Nuri | for | Rusa | (2004) | Jongdari | for | Sonamu | (2015) | Koto | for | Kammuri | (2021) |
| Peipah | for | Vamei | (2004) | Barijat | for | Utor | (2015) | Nokaen | for | Phanfone | (2021) |
| Molave | for | Imbudo | (2004) | Mun | for | Fitow | (2015) | Penha | for | Vongfong | (2022) |
| Noul | for | Pongsona | (2006) | Bailu | for | Haiyan | (2015) | Peilou | for | Linfa | (2022) |
| Dolphin | for | Yanyan | (2006) | Lan | for | Vicente | (2015) | Narra | for | Molave | (2022) |
| Mujigae | for | Maemi | (2006) | Bualoi | for | Rammasun | (2016) | Gaenari | for | Goni | (2022) |
| Mirinae | for | Sudal | (2006) | Saudel | for | Soudelor | (2017) | Bang-Lang | for | Vamco | (2022) |
| Lionrock | for | Tingting | (2006) | Surigae | for | Mujigae | (2017) | Jamjari | for | Nalgae | (2024) |
| Fanapi | for | Rananim | (2006) | Koguma | for | Koppu | (2017) | Tsing-ma | for | Ma-on | (2024) |
| Pakhar | for | Matsa | (2007) | Cempaka | for | Melor | (2017) | Tokei | for | Kompasu | (2024) |
| Doksuri | for | Nabi | (2007) | Nyatoh | for | Meranti | (2018) | Ong-mang | for | Hinnamnor | (2024) |
| Haikui | for | Longwang | (2007) | Trases | for | Sarika | (2018) | Amuyao | for | Malakas | (2024) |
| Sanba | for | Chanchu | (2008) | Mulan | for | Haima | (2018) | Gosari | for | Megi | (2024) |
| Maliksi | for | Bilis | (2008) | Hinnamnor | for | Nock-ten | (2018) | Hodu | for | Noru | (2024) |
| SonTinh | for | Saomai | (2008) | Yamaneko | for | Hato | (2019) | Sarbul | for | Rai | (2024) |
| Leepi | for | Xangsane | (2008) | Yun-yeung | for | Kai-tak | (2019) | Luc-Binh | for | Conson | (2024) |
| Mangkhut | for | Durian | (2008) | Koinu | for | Tembin | (2019) | Tianma | for | Haikui | (2025) |
| Atsani | for | Morakot | (2011) | Pulasan | for | Rumbia | (2020) | Bori | for | Doksuri | (2025) |
| Champi | for | Ketsana | (2011) | Krathon | for | Mangkhut | (2020) | Saobien | for | Saola | (2025) |
| In-fa | for | Parma | (2011) | Yinxing | for | Yutu | (2021) |  |  |  |  |

##### Corrected spelling

| Megkhla | to | Mekkhala | (2002) | Kaemi | to | Gaemi | (2008) | Koni | to | Goni | (2008) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kularb | to | Kulap | (2002) | Chebi | to | Jebi | (2008) | SonTinh | to | Son-Tinh | (2008) |
| Ramasoon | to | Rammasun | (2002) | Noguri | to | Neoguri | (2008) |  |  |  |  |
| Vipa | to | Wipha | (2002) | Changmi | to | Jangmi | (2008) |  |  |  |  |

#### OPERATIONAL PROCEDURES FOR THE ASSIGNMENT OF NAMES OF TROPICAL CYCLONES

(a)		RSMC Tokyo - Typhoon Center will assign a name each time a 4-digit identification number is to be assigned. That is, names on the Typhoon Committee list will only be given to tropical cyclones of tropical storm strength or above. Each tropical cyclone should be identified by its name followed by the 4-digit number in brackets. The same names and numbers should also be used in bulletins issued by the Tokyo Tropical Cyclone Advisory Centre under the umbrella of the International Civil Aviation Organization (ICAO) as well as in bulletins for Meteorological Area (METAREA)-XI of the IMO/WMO Worldwide Met-Ocean Information and Warning Service (WWMIWS), issued by both China and Japan. This would contribute to the standardization of the usage of names of tropical cyclones as was desired by the Typhoon Committee.

(b)		The exchange of observational data should be promoted as much as possible in addition to what is already exchanged among the warning centres and the meteorological services in the region, to ensure that RSMC Tokyo - Typhoon Center would benefit from the best possible data and information needed for it to carry out its work.

(c)		On the operation of the name list, the names will be assigned following the pre-determined order. The name would remain unchanged throughout the life history of the tropical cyclone. To avoid confusion, tropical cyclones given a name before crossing the Date Line or 100°E and entering the western North Pacific should be assigned a number by RSMC Tokyo - Typhoon Center but should not be assigned a new name in the Typhoon Committee list. RSMC Honolulu Hurricane Center and RSMC New Delhi will continue the use of the tropical cyclone names assigned by RSMC Tokyo - Typhoon Center when tropical cyclones cross the Date Line from west to east or 100°E from east to west, respectively.

(d)		The names and numbers assigned by RSMC Tokyo - Typhoon Center will be used by all Typhoon Committee Members when issuing warning bulletins intended for the international community including the press, aviation and shipping.

(e)		The Typhoon Committee, as the authority to maintain the list, shall review the list of names and its operation regularly at its annual sessions as the need arises.

(f)	Members may request the retirement of a name from the list particularly in case of tropical cyclones causing extensive destruction or for other reasons. Such notification shall be made preferably within a year of the event. The decision to retire names should be made at the regular review at annual sessions of the Typhoon Committee.

### APPENDIX 1-C

31

#### LIST OF ACRONYMS USED IN THE OPERATIONAL MANUAL - METEOROLOGICAL COMPONENT -

AFTN		Aeronautical Fixed Telecommunication Network

AIREP	Air-report

AMV	Atmospheric Motion Vector

APT	Automatic Picture Transmission

ASCAT	Advanced SCATterometer

BoM	Bureau of Meteorology

BUFR	Binary Universal Form for the Representation of meteorological data

BUOY	Report of a buoy operation

CAPPI	Constant Altitude Plan Position Indicator

CMA	China Meteorological Administration

CMC	Canadian Meteorological Centre

CSR	Clear Sky Radiance

DDN	DataDirect Networks

DWD	Deutscher Wetterdienst

ECMWF	European Centre for Medium-Range Weather Forecasts

EGC	Enhanced Group Call

EUMETSAT	European Organisation for the Exploitation of Meteorological Satellites

EPS	Ensemble Prediction System

ESCAP	Economic and Social Commission for Asia and the Pacific

FAX	Facsimile

FTP	File Transfer Protocol

FY	Feng-Yun

FY-ESM	Feng-Yun Emergency Support Mechanism

GEO-KOMPSAT	Geostationary Korea Multi-Purpose Satellite

GEPS	Global EPS

GMDSS	Global Maritime Distress and Safety System

GNSS	Global Navigation Satellite System

GOOS	Global Ocean Observing System

GRIB	General regularly distributed information in binary form

GSM	Global Spectral Model

GTS	Global Telecommunication System

HF NBDP	High Frequency Narrow-Band Direct Printing

HKO	Hong Kong Observatory

HRPT	High Resolution Picture Transmission

HWRF	Hurricane Weather Research and Forecast System

ICAO	International Civil Aviation Organization

IMO	International Maritime Organization

IR	Infrared

JCSAT	Japan Communications Satellite

JMA	Japan Meteorological Agency

JTWC	Joint Typhoon Warning Center

KMA		Korea Meteorological Administration

METAR	Aerodrome/aviation routine meteorological report

METAREA	A geographical sea region that coordinates the transmission of meteorological information to mariners

MPLS	Multi-Protocol Label Switching

MSI	Maritime Safety Information

MSTP	Multiple Spanning Tree Protocol

MTI	Moving Target Indicator

MWO	Meteorological Watch Office

NAVTEX	Navigational Telex

NCEP	National Centers for Environmental Prediction

NESDIS	National Environmental Satellite, Data and Information Service

NHM	Non-Hydrostatic Model

NMC	National Meteorological Centre

NMHS	National Meteorological and Hydrological Service

NMS	National Meteorological Service

NOAA	National Oceanic and Atmospheric Administration

NRL	Naval Research Laboratory

NWP	Numerical Weather Prediction

OLR	Outgoing Longwave Radiation

OPMET	Operational Meteorological information

OSCAT	OceanSat Scatterometer

PAGASA	Philippine Atmospheric, Geophysical, and Astronomical Services Administration

PBL	Planetary Boundary Layer

PILOT	Upper-wind report from a fixed land station

PNG	Portable Network Graphics

PWV	Precipitable Water Vapour

R/A	Radar/raingauge-Analyzed precipitation

RADOB	Report of ground radar weather observations

RO	Radio Occultation

ROBEX	Regional OPMET Bulletin Exchange

RSMC	Regional Specialized Meteorological Centre

RTH	Regional Telecommunication Hub

S-VISSR	Stretched VISSR

SAR	Synthethic aperture radar

SAREP	Report of synoptic interpretation of cloud data obtained by a meteorological satellite

SATAID	SATellite Animation and Interactive Diagnosis

SCAT	Scatterometer overpass

SHIP	Report of surface observation from a sea station

SHIPS	Statistical Hurricane Intensity Prediction Scheme

SST	Sea Surface Temperature

SYNOP	Report of surface observation from a fixed land station

TAC	Traditional Alphanumeric Code Form

TC	Typhoon Committee

TCAC	Tropical Cyclone Advisory Centre

TCP	Tropical Cyclone Programme

TCP/IP	Transmission Control Protocol / Internet Protocol

TCS	Typhoon Committee Secretariat

TDCF	Table-Driven Code Form

TEMP	Upper-level pressure, temperature, humidity and wind report from a fixed land station

TIFS	Typhoon Intensity Forecast scheme based on SHIPS

TOPEX	Typhoon Operational Experiment

TRAMS	Tropical Regional Atmosphere Model for the South China Sea

TS	Tropical Storm

TWRF	Typhoon Weather Research and Forecast System

UKMO	United Kingdom Met Office

UNDP	United Nations Development Programme

UTC	Universal Time Coordinated

VIS	Visible

VISSR											Visible and Infrared Spin Scan Radiometer

VPN	Virtual Private Network

WMO	World Meteorological Organization

WV	Water Vapour

WWMIWS	IMO/WMO Worldwide Met-Ocean Information and Warning Service

### APPENDIX 2-A

33

#### LIST OF STATIONS FROM WHICH ENHANCED SURFACE OBSERVATIONS ARE AVAILABLE

The following stations will make hourly surface observations when they are within 300 km of the centre of a tropical cyclone of TS intensity or higher:

| Cambodia |  |  |
| --- | --- | --- |
| China |  |  |
|  | (54): | 324, 337, 342, 346, 405, 423, 436, 471, 493, 497, 511, 534, 539, 602, 618, 662, 715, 751, 753, 776, 823, 826, 836, 843, 857, 863, 929, 945 |
|  | (58): | 040, 141, 150, 238, 251, 265, 345, 362, 457, 472, 477, 543, 556, 569, 646, 652, 666, 752, 754, 834, 847, 911, 921, 926, 931, 944 |
|  | (59): | 007, 023, 046, 058, 072, 082, 087, 096, 117, 134, 209, 211, 254, 278, 287, 293, 316, 417, 431, 456, 493, 501, 632, 644, 658, 663, 673, 758, 838, 845, 855, 948, 981 |
| Democratic People's Republic of Korea |  |  |
|  | (47): | 003, 005, 008, 014, 016, 020, 022, 025, 028, 031, 035, 037, 039, 041, 045, 050, 052, 055, 058, 060, 061, 065, 067, 068, 069 |
| Hong Kong, China |  |  |
|  | (45): | 007 |
| Japan |  |  |
|  | (47): | 401, 407, 409, 412, 418, 420, 421, 426, 430, 570, 575, 582, 584, 590, 600, 604, 605, 610, 624, 629, 636, 648, 651, 655, 662, 675, 678, 740, 741, 746, 750, 765, 772, 778, 800, 807, 815, 817, 827, 830, 843, 887, 891, 893, 895, 909, 918, 927, 936, 945, 971, 991 |
| Lao People's Democratic Republic |  |  |
| Macao, China |  |  |
|  | (45): | 011 |
| Malaysia |  |  |
|  | (48): | 601, 615, 620, 647, 650, 657, 665, 603, 604, 618, 679 |
|  | (96): | 413, 421, 441, 449, 465, 471, 481, 491, 420, 450, 467, 477 |
| Philippines |  |  |
|  | (98): | 132, 134, 222, 223, 232, 233, 324, 325, 327, 328, 334, 336, 426, 427, 428, 429, 430, 431, 432, 433, 434, 435, 440, 444, 446, 526, 531, 536, 538, 543, 546, 548, 553, 558, 618, 630, 642, 644, 646, 648, 653, 741, 746, 751, 752, 753, 836, 851 |
| Republic of Korea |  |  |
|  | (47): | 090, 093, 095, 098, 099, 100, 101, 102, 105, 106, 108, 112, 114, 115, 119, 121, 127, 129, 130, 131, 133, 135, 136, 137, 138, 140, 143, 146, 152, 155, 156, 159, 162, 165, 168, 169, 170, 172, 174, 175, 177, 184, 185, 188, 189, 192 , 201, 202, 203, 211, 212, 214, 216, 217, 221, 226, 232, 235, 236, 243, 244, 245, 247, 248, 251, 252, 253, 254, 255, 257, 258, 259, 260, 261, 262, 263, 264, 266, 268, 271, 272, 273, 276, 277, 278, 279, 281, 283, 284, 285, 288, 289, 294, 295 |
| Thailand |  |  |
|  | (48): | 303, 351, 352, 353, 357, 378, 383, 407, 432, 437, 462, 465, 480, 500, 501, 517, 551, 552, 560, 568, 580, 583 |
| USA |  |  |
|  | (91): | 203, 212, 258, 317, 324, 334, 339, 348, 353, 356, 366, 367, 369, 371, 376, 378, 408, 413, 425, 434 |
| Viet Nam |  |  |
|  | (48): | 820, 826, 839, 845, 848, 855, 870, 877, 900, 914, 917, 918, 920 |

Note:	Name, latitude, longitude and elevation of these stations are included in OSCAR/Surface.

### APPENDIX 2-B

35

#### LIST OF STATIONS FROM WHICH ENHANCED UPPER-AIR OBSERVATIONS ARE AVAILABLE

The following stations will make 6-hourly upper-air observations when they are within 300 km of the centre of a tropical cyclone of TS intensity or higher:

| Cambodia |  |  |
| --- | --- | --- |
| China |  |  |
|  | (54): | 511, 727, 857 |
|  | (57): | 083, 494, 972 |
|  | (58): | 150, 362, 457, 665, 847, 968 |
|  | (59): | 134, 316, 758, 981 |
| Democratic People's Republic of Korea |  |  |
|  | (47): | 041, 058 |
| Hong Kong, China |  |  |
|  | (45): | 004 # upper-air observations are made by wind profiler at 06 and 18 UTC normally, but radiosondes will be launched when warranted by local wind conditions |
| Japan |  |  |
|  | (47): | 418, 600, 646, 678, 741, 778, 807, 827, 909, 918, 945 |
| Lao People's Democratic Republic |  |  |
| Macao, China |  |  |
| Malaysia |  |  |
|  | (48): | 615, 650 |
|  | (96): | 413, 441, 471 |
| Philippines |  |  |
|  | (98): | 223, 233, 433, 444, 558, 618, 646, 753 |
| Republic of Korea |  |  |
|  | (47): | 102, 104, 122, 138, 158, 169, 186 |
| Thailand |  |  |
|  | (48): | 327, 378, 381, 407, 431, 453, 480, 500, 551, 565, 568 |
| USA |  |  |
|  | (91): | 212, 334, 348, 366, 376, 408, 413 |
| Viet Nam |  |  |
|  | (48): | 820, 855, 900 |

Note:	Name, latitude, longitude and elevation of these stations are included in OSCAR/Surface.

### APPENDIX 2-C

36

#### LIST OF BUOY OBSERVATIONS BY TYPHOON COMMITTEE MEMBERS

| Member | Area | Observation Elements | Frequency | Heading in the BUFR code (FM 94) |
| --- | --- | --- | --- | --- |
| Hong Kong, China | South China Sea and western North Pacific | Air pressure and sea surface temperature | Every hour during tropical cyclone seasons | IOBC01 VHHH for buoys operated solely by Hong Kong, China IOBX02 KWBC for buoys operated under the Barometer Upgrade Scheme of the Global Drifter Programme of Data Buoy Cooperation Panel of GOOS |
| Japan | Western North Pacific | Air pressure, sea surface temperature, significant wave height and period | Every 3 hours (Every hour when waves are higher than thresholds set beforehand) | IOBC11 RJTD |

### APPENDIX 2-D

37

#### DISTRIBUTION OF THE RADAR STATIONS OF TYPHOON COMMITTEE MEMBERS

### APPENDIX 2-E

67

#### TECHNICAL SPECIFICATIONS OF RADARS OF TYPHOON COMMITTEE MEMBERS

##### Name of the Member  China

| NAME OF STATION |  |  | Shanghai | Wenzhou | Fuzhou | Shantou | Xishadao |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 58367 | 58659 | 58941 | 59316 | 59981 |
|  | Location of station |  | 31° 02´ N | 27° 51´ N | 25° 59´ N | 23° 17´ N | 16° 50´ N |
|  |  |  | 121° 57´ E | 120° 49´ E | 119° 32´ E | 116° 44´ E | 112° 20´ E |
|  | Antenna elevation | m | 68 | 294 | 652.5 | 196.7 | 8.5 |
|  | Wave length | cm | 10.6 | 10.6 | 10.4 | 10.4 | 10.6 |
|  | Peak power of transmitter | kW | 500 | 500 | 500 | 500 | 500 |
|  | Pulse length | µs | 1 | 3.0 | 1.0 | 1 | 3 |
|  | Sensitivity minimum of receiver | dBm | -110 | -110 | -119 | -109 | -110 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 2.0 | 2.0 | 2.0 | 1.2 | 2.0 |
|  | Detection range | km | 600 | 600 |  |  |  |
|  | Scan mode in observation |  | 1 2 3 | 1 2 3 | 1 2 3 | 1 2 3 | 2 |
|  | 1.Fixed elevation |  |  |  |  |  |  |
|  | 2.CAPPI |  |  |  |  |  |  |
|  | 3.Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 2 | 2 | 1 | 1 | 2 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 2 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1 | 1 | 1 | 1 | 1 |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Democratic People’s Republic of Korea

| NAME OF STATION |  |  | Pyongyang |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 47058 |  |  |  |  |
|  | Location of station |  | 39° 02´ N |  |  |  |  |
|  |  |  | 125° 47´ E |  |  |  |  |
|  | Antenna elevation | m | 90 |  |  |  |  |
|  | Wave length | cm | 3.2 |  |  |  |  |
|  | Peak power of transmitter | kW | 150 |  |  |  |  |
|  | Pulse length | µs | 1, 2 |  |  |  |  |
|  | Sensitivity minimum of receiver | dBm | -132 |  |  |  |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 44 |  |  |  |  |
|  | Detection range | km | 300 |  |  |  |  |
|  | Scan mode in observation |  | 1 2 3 |  |  |  |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 |  |  |  |  |
|  | 1. Yes, 2.No |  |  |  |  |  |  |
|  | Doppler processing |  | 2 |  |  |  |  |
|  | 1.Yes, 2.No |  |  |  |  |  |  |
|  | Display |  | 1 |  |  |  |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1 |  |  |  |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 |  |  |  |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Hong Kong, China

| NAME OF STATION |  |  | Tai Mo Shan | Tate’s Cairn |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 45009 | 45010 |  |  |  |
|  | Location of station |  | 22° 25´ N | 22° 21´ N |  |  |  |
|  |  |  | 114° 07´ E | 114° 13´ E |  |  |  |
|  | Antenna elevation | m | 968 | 586 |  |  |  |
|  | Wave length | cm | 10.6 | 10.3 |  |  |  |
|  | Peak power of transmitter | kW | 650 | 750 |  |  |  |
|  | Pulse length | µs | 1.0/2.0 | 1.0/2.0 |  |  |  |
|  | Sensitivity minimum of receiver | dBm | -109/-112 | -111/-114 |  |  |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 0.9(H) 0.9(V) | 0.9(H) 0.9(V) |  |  |  |
|  | Detection range | km | 500 | 500 |  |  |  |
|  | Scan mode in observation |  | 2 | 2 |  |  |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 |  |  |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (Continuous) | 3 (Continuous) |  |  |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 |  |  |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Japan - 1

| NAME OF STATION |  |  | Sapporo /Kenashiyama | Kushiro /Kombumori | Hakodate /Yokotsudake | Sendai | Akita |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 47415 | 47419 | 47432 | 47590 | 47582 |
|  | Location of station |  | 43° 08´ N | 42° 58´ N | 41° 56´ N | 38° 16´ N | 39° 43´ N |
|  |  |  | 141° 01´ E | 144° 31´ E | 140° 47´ E | 140° 54´ E | 140° 06´E |
|  | Antenna elevation | m | 749.0 | 121. 6 | 1141.7 | 98.0 | 55.3 |
|  | Wave length | cm | 5.61 | 5. 59 | 5.60 | 5.59 | 5.59 |
|  | Peak power of transmitter | kW | 250 | 4( H/V each) | 4(H/V each) | 3(H/V each) | 250 |
|  | Pulse length | µs | 1.1/2.6 | 1.0 32/64/128 | 1.0 32/64/128 | 1.0 32/64/128 | 1.1/2.6 |
|  | Sensitivity minimum of receiver | dBm | -109/-112 | (H)-111/-113 (V)-111/-114 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | (H)-112/-114 (V)-112/-114 (short pulse / long pulse) | -108/-112 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1.1(H) 1.1(V) | (H)0.9/0.9 (V)0.9/0.9 (H plane / V plane) | (H)0.9/0.9 (V)0.9/0.9 (H plane / V plane) | (H)1.0/1.0 (V)1.0/1.0 (H plane / V plane) | 1.0(H) 0.9(V) |
|  | Detection range | km | 400 | 400 | 400 | 400 | 400 |
|  | Scan mode in observation |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1 | 1 | 1 | 1 | 1 |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Japan - 2

| NAME OF STATION |  |  | Tokyo /Kashiwa | Niigata /Yahikoyama | Fukui /Tojimbo | Nagano /Kurumayama | Shizuoka /Makinohara |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 47695 | 47572 | 47705 | 47611 | 47659 |
|  | Location of station |  | 35° 52´ N | 37° 43´ N | 36° 14´ N | 36° 06´ N | 34° 45´ N |
|  |  |  | 139° 58´ E | 138° 49´ E | 136° 09´ E | 138° 12´ E | 138° 08´E |
|  | Antenna elevation | m | 74.0 | 648.1 | 106.9 | 1937.1 | 186.0 |
|  | Wave length | cm | 5.60 | 5.61 | 5.60 | 5.64 | 5.66 |
|  | Peak power of transmitter | kW | 3(H/V each) | 4(H/V each) | 4(H/V each) | 250 | 250 |
|  | Pulse length | µs | 1.0 32/64/128 | 1.0 32/64/128 | 1.0 32/64/128 | 1.0/2.6 | 1.1/2.6 |
|  | Sensitivity minimum of receiver | dBm | (H)-112/-114 (V)-112/-114 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | -110/-114 | -110/-113 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | (H)1.0/1.0 (V)1.0/1.0 (H plane / V plane) | (H)1.0/0.9 (V)0.9/1.0 (H plane / V plane) | (H)0.9/0.9 (V)0.9/0.9 (H plane / V plane) | 1.1(H) 1.0(V) | 1.1(H) 1.1(V) |
|  | Detection range | km | 400 | 400 | 400 | 400 | 400 |
|  | Scan mode in observation |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1 | 1 | 1 | 1 | 1 |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Japan - 3

| NAME OF STATION |  |  | Nagoya | Osaka /Takayasuyama | Matsue /Misakayama | Hiroshima /Haigamine | Murotomisaki |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 47636 | 47773 | 47791 | 47792 | 47899 |
|  | Location of station |  | 35° 10´ N | 34° 37´ N | 35° 33´ N | 34° 16´ N | 33° 15´ N |
|  |  |  | 136° 58´ E | 135° 39´ E | 133° 06´ E | 132° 36´ E | 134° 11´E |
|  | Antenna elevation | m | 73.1 | 497.5 | 552.9 | 751.5 | 207.0 |
|  | Wave length | cm | 5.59 | 5.60 | 5.61 | 5.59 | 5.60 |
|  | Peak power of transmitter | kW | 4(H/V each) | 4(H/V each) | 4(H/V each) | 3(H/V each) | 4(H/V each) |
|  | Pulse length | µs | 1.0 32/64/128 | 1.0 32/64/128 | 1.0 32/64/128 | 1.0 32/64/128 | 1.0 32/64/128 |
|  | Sensitivity minimum of receiver | dBm | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | (H)-112/-114 (V)-112/-114 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | (H)1.0/0.9 (V)0.9/0.9 (H plane / V plane) | (H)0.9/0.9 (V)0.9/0.9 (H plane / V plane) | (H)1.0/0.9 (V)0.9/0.9 (H plane / V plane) | (H)1.0/0.9 (V)0.9/1.0 (H plane / V plane) | (H)0.9/1.0 (V)1.0/0.9 (H plane / V plane) |
|  | Detection range | km | 400 | 400 | 400 | 400 | 400 |
|  | Scan mode in observation |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1 | 1 | 1 | 1 | 1 |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Japan - 4

| NAME OF STATION |  |  | Fukuoka /Sefuriyama | Tanegashima /Nakatane | Naze /Funchatoge | Okinawa /Itokazu | Ishigakijima /Omotodake |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 47806 | 47869 | 47909 | 47937 | 47920 |
|  | Location of station |  | 33° 26´ N | 30° 38´ N | 28° 24´ N | 26° 09´ N | 24° 26´ N |
|  |  |  | 130° 21´ E | 130° 59´ E | 129° 33´ E | 127° 46´ E | 124° 11´E |
|  | Antenna elevation | m | 983.2 | 302.5 | 318.6 | 208.4 | 533.5 |
|  | Wave length | cm | 5.59 | 5. 59 | 5.61 | 5.60 | 5.61 |
|  | Peak power of transmitter | kW | 3(H/V each) | 4(H/V each) | 4(H/V each) | 4(H/V each) | 250 |
|  | Pulse length | µs | 1.0 32/64/128 | 1.0 32/64/128 | 1.0 32/64/128 | 1.0 32/64/128 | 1.1/2.7 |
|  | Sensitivity minimum of receiver | dBm | (H)-112/-114 (V)-112/-114 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | (H)-111/-113 (V)-111/-113 (short pulse / long pulse) | -107/-111 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | (H)1.0/1.0 (V)0.9/1.0 (H plane / V plane) | (H)1.0/0.9 (V)0.9/0.9 (H plane / V plane) | (H)0.9/0.9 (V)0.9/0.9 (H plane / V plane) | (H)1.0/1.0 (V)0.9/1.0 (H plane / V plane) | 1.1(H) 1.1(V) |
|  | Detection range | km | 400 | 400 | 400 | 400 | 400 |
|  | Scan mode in observation |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1 | 1 | 1 | 1 | 1 |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Macao, China

| NAME OF STATION |  |  | Taipa Grande | Zhuhai-Macao Radar |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 45011 |  |  |  |  |
|  | Location of station |  | 22.1599°N | 22.0240°N |  |  |  |
|  |  |  | 113.5624°E | 113.3756°E |  |  |  |
|  | Antenna elevation | m | 183 | 250 |  |  |  |
|  | Wave length | cm | 3.4 | ~10 |  |  |  |
|  | Peak power of transmitter | kW | 200 | > 800 |  |  |  |
|  | Pulse length | µs | 0.4, 0.8, 1.0, 2.0 | 0.5, 1.57, 4.5 |  |  |  |
|  | Sensitivity minimum of receiver | dBm | -113 | -114 for 4.5 µs -111 for 1.57 µs |  |  |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1° | < +/- 0.01° |  |  |  |
|  | Detection range | km | 128 | 230/460 |  |  |  |
|  | Scan mode in observation |  | 3 | 3 |  |  |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 |  |  |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 | 3 |  |  |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 2 | 1 |  |  |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Malaysia - 1

| NAME OF STATION |  |  | Alor Star | Kota Bharu | Kuala Lumpur (Sepang) | Kuala Lumpur (Subang) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 48603 | 48615 | 48650 | 48647 |  |
|  | Location of station |  | 6° 11´ N | 6° 9´ N | 2° 51´ N | 3° 09´ N |  |
|  |  |  | 100° 24´ E | 102° 18´ E | 101° 40´ E | 101° 34´ E |  |
|  | Antenna elevation | m | 33 | 57 | 12 | 117 |  |
|  | Wave length | cm | 10.71 | 10.71 | 10.44 | 10.71 |  |
|  | Peak power of transmitter | kW | 650 | 900 | 750 | 650 |  |
|  | Pulse length | µs | 0.8 and 2 | 1.8 | 0.5,1 and 2 | 0.8 and 2 |  |
|  | Sensitivity minimum of receiver | dBm | -110 | -110 | -116 | -110 |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 2 | 1 | 1 | 1 |  |
|  | Detection range | km | 300 | 300 | 480 | 300 |  |
|  | Scan mode in observation |  | 2 | 1,2 | 1,2 | 2 |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 | 2 | 2 |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (every 10 mins) | 3 (every 10 mins) | 3 (every 10 mins) | 3 (every 5 mins) |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 (from May 2005) | 1 (upgrade Nov 2022) | 1 (upgrade in 2016) | 1 (upgrade in 2015) |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Malaysia - 2

| NAME OF STATION |  |  | Kuantan | Kuching | Bintulu | Miri |
| --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |
|  | Index number |  | 48657 | 96413 | 96441 | 96449 |
|  | Location of station |  | 3° 45´ N | 1° 29´ N | 3° 13´ N | 4° 23´ N |
|  |  |  | 103° 11´ E | 110° 20´ E | 113° 04´ E | 113° 59´ E |
|  | Antenna elevation | m | 52 | 77 | 171 | 182 |
|  | Wave length | cm | 10.71 | 5.3 | 5.3 | 10.73 |
|  | Peak power of transmitter | kW | 900 | 220 | 250 | 900 |
|  | Pulse length | µs | 1.83 | 0.8 and 2 | 0.8 and 1.8 | 1.76 |
|  | Sensitivity minimum of receiver | dBm | -110 | -110 | -110 | -110 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1 | 1.7 | 1.7 | 1 |
|  | Detection range | km | 300 | 300 | 300 | 300 |
|  | Scan mode in observation |  | 1,2 | 1 | 1 | 1,2 |
|  | 1. Fixed elevation |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 | 2 | 2 |
|  | 1. Yes, 2. No |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (every 10 mins) | 3 (every 10 mins) | 3 (every 10 mins) | 3 (every 10 mins) |
|  | cyclone is within range of detection) |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 (upgrade Aug 2022) | 1 (from 2000) | 1 (from 2001) | 1 (upgrade Sep 2022) |
|  | 1. Operational |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |

##### Name of the Member  Malaysia - 3

| NAME OF STATION |  |  | Kota Kinabalu | Sandakan | Temerloh | Kuala Krai | Cameron Highlands |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 96471 | 96491 | 48653 | 48616 | 48632 |
|  | Location of station |  | 5° 56´ N | 5° 54´ N | 3° 28´ N | 5° 34´ N | 4° 29´ N |
|  |  |  | 116° 03´E | 118° 04´ E | 102° 22´ E | 102° 12´ E | 102° 22´ E |
|  | Antenna elevation | m | 27 | 28 | 82 | 80 | 1602 |
|  | Wave length | cm | 5.3 | 5.3 | 3.2 | 3.2 | 3.2 |
|  | Peak power of transmitter | kW | 210 | 250 | 79 | 84 | 79 |
|  | Pulse length | µs | 0.8 and 1.8 | 0.8 and 1.8 | 0.5/1.0/2.0 | 0.5/1.0/2.0 | 0.5/1.0/2.0 |
|  | Sensitivity minimum of receiver | dBm | -110 | -110 | -118 | -118 | -118 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1.7 | 1.7 | 1.0 | 1.0 | 1.0 |
|  | Detection range | km | 300 | 300 | 100 | 100 | 100 |
|  | Scan mode in observation |  | 2 | 2 | 1,2 | 1,2 | 1,2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (every 10 mins) | 3 (every 10 mins) | 3 (every 5 mins) | 3 (every 5 mins) | 3 (every 5 mins) |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 (from 2000) | 1 (from 2001) | 1 (from 2021) | 1 (from 2021) | 1 (from 2022) |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Malaysia - 4

| NAME OF STATION |  |  | Marang | Rompin | Sibu | Kuala Gula | Tawau |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  |  |  |  |  |  |
|  | Location of station |  | 5° 5´ N | 2° 44´ N | 2° 17´ N | 4° 58´ N | 4° 15´ N |
|  |  |  | 103° 31´ E | 103° 31´ E | 111° 51´ E | 100° 28´ E | 117° 53´ E |
|  | Antenna elevation | m | 38 | 56.7 | 38 | 56 | 51 |
|  | Wave length | cm | 10.60 | 11.1 | 10.95 | 10.741 | 11.091 |
|  | Peak power of transmitter | kW | 910 | 910 | 910 | 910 | 910 |
|  | Pulse length | µs | 0.5/1.0/2.0 | 0.5/1.0/2.0 | 0.5/1.0/2.0 | 0.5/1.0/2.0 | 0.5/1.0/2.0 |
|  | Sensitivity minimum of receiver | dBm | -118 | -118 | -118 | -118 | -118 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | Detection range | km | 300 | 300 | 300 | 300 | 300 |
|  | Scan mode in observation |  | 1,2 | 1,2 | 1,2 | 1,2 | 1,2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (every 10 mins) | 3 (every 10 mins) | 3 (every 10 mins) | 3 (every 10 mins) | 3 (every 10 mins) |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 (from 2022) | 1 (from 2022) | 1 (from 2023) | 2023 | 2024 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Philippines - 1

| NAME OF STATION |  |  | Aparri | Virac | Guiuan | Subic (EEC) | Subic (SELEX) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 98231 | 98447 | 98558 |  |  |
|  | Location of station |  | 18° 21’ 35’’ N | 13° 37’ 47.16’’ N | 11° 02’ 42.72’’ N | 14° 49’ 19.44’’ N | 14° 49’ 19.44’’ N |
|  |  |  | 121° 37’ 48.50’’ E | 124° 20’ 02.59’’ E | 125° 45’ 20.56’’ E | 120° 21’ 49.68’’E | 120° 21’ 49.68’’E |
|  | Antenna elevation | m | 34 | 33.5 | 25 | 40 | 40 |
|  | Wave length | cm | 10.52 | 10.52 | 10.52 | 10.4 |  |
|  | Peak power of transmitter | kW | 10 | 10 | 10 | 850 | 1000 |
|  | Pulse length | µs | 2 & 100 - intensity mode 1 @ 50 - Doppler mode | 2 & 100 - intensity mode 1 @ 50 - Doppler mode | 2 & 100 - intensity mode 1 @ 50 - Doppler mode | 2.0, 1.0, 0.8, 0.4 | 2.0, 1.0, 0.5 |
|  | Sensitivity minimum of receiver | dBm | -114 | -114 | -114 | -114 | -117 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1.8 | 1.8 | 1.8 | 1.83 | 1 |
|  | Detection range | km | 440 | 440 | 440 | 480 | 480 |
|  | Scan mode in observation |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 2 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 2 | 2 | 2 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 (Operational) | 2 (damage by Typhoon GONI) for installation of new Radar | 1 (Operational) | 1 (Operational) | 1 (Operational) |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Philippines - 2

| NAME OF STATION |  |  | Tagaytay | Mactan | Tampakan | Ilo-Ilo Iloilo | Bohol |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  |  | 98646 |  | 98637 |  |
|  | Location of station |  | 14° 08’ 31.70’’ N | 10° 19’ 21.80’’ N | 06° 25’ 03.30’’ N | 10° 46’ 22.30’’ N | 09° 38’ 48.72’’ N |
|  |  |  | 121° 01’ 20.20’’ E | 123° 58’ 49.01’’ E | 125° 01’ 51.41’’ E | 122° 34’ 46.00’’ E | 123° 57’ 02.70’’ E |
|  | Antenna elevation | m | 30 | 21 | 23 | 21 | 30 |
|  | Wave length | cm | 5.34 | 5.33 | 10.4 | 10.44 | 10.7 |
|  | Peak power of transmitter | kW | 250 | 250 | 850 | 850 | 1000 |
|  | Pulse length | µs | 2.0, 1.0, 0.8, 0.4 | 2.0, 1.0, 0.8, 0.4 | 2.0, 1.0, 0.8, 0.4 | 2.0, 1.0, 0.8, 0.4 | 2.0, 1.0, 0.8, 0.4 |
|  | Sensitivity minimum of receiver | dBm | -114 | -114 | -114 | -114 | -114 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1.0 | 1.0 | 1.3 | 1.3 | 0.9 |
|  | Detection range | km | 250 | 480 | 480 | 480 | 480 |
|  | Scan mode in observation |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 2 (Not operational) | 1 (Operational) | 2 (Decommissioned) | 1 (Operational) | 2 (Not operational) |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

Name of the Member  Philippines - 3

| NAME OF STATION |  |  | Hinatuan (EEC) | Hinatuan (SELEX) | Baguio | Daet | Baler |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 98755 | 98755 |  |  | 98334 |
|  | Location of station |  | 08° 22’ 02.37’’ N | 08° 22’ 02.37’’ N | 16° 21’ 22.60’’ N | 14° 07’ 43.10’’ N | 15° 44’ 56.30’’ N |
|  |  |  | 126° 20’ 18.73’’ E | 126° 20’ 18.73’’ E | 120° 33’ 32.60’’ E | 122° 58’ 58.46’’ E | 121° 37’ 55.62’’ E |
|  | Antenna elevation | m | 34 | 34 | 15 | 21 | 6 |
|  | Wave length | cm | 10.78 | 10.78 |  |  |  |
|  | Peak power of transmitter | kW | 850 | 850 | 500 | 1000 | 1000 |
|  | Pulse length | µs | 2.0, 1.0, 0.8, 0.4 | 2.0, 1.0, 0.8, 0.4 | 2.0, 1.0, 0.5 | 2.0, 1.0, 0.5 | 2.0, 1.0, 0.5 |
|  | Sensitivity minimum of receiver | dBm | -114 | -114 | -117 | -117 | -117 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1.3 | 1.3 | 1 | 1 | 1 |
|  | Detection range | km | 480 | 480 | 480 | 480 | 480 |
|  | Scan mode in observation |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 | 2 | 2 | 2 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 2 (Not operational/Damaged by Earthquake) | 2 (Not operational/Damaged by Earthquake) | 1 (Operational) | 1 (Operational) | 1 (Operational) |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

Name of the Member  Philippines - 4

| NAME OF STATION |  |  | Basco | Quezon Palawan | Busuanga Palawan | Zamboanga |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 98134 |  |  |  |  |
|  | Location of station |  | 20° 25’ 40.21’’ N | 09° 13’ 50.10’’ N | 12° 05’ 20.11’’ N | 06° 54’ 55.10’’ N |  |
|  |  |  | 121° 58’ 13.60’’ E | 118° 00’ 20.90’’ E | 119° 56’ 15.43’’ E | 122° 02’ 29.15’’ E |  |
|  | Antenna elevation | m | 6 | 21 | 18.6 | 24.6 |  |
|  | Wave length | cm | 5.35 | 5.35 | 5.35 | 5.35 |  |
|  | Peak power of transmitter | kW | 250 | 250 | 250 | 250 |  |
|  | Pulse length | µs | 2.0, 1.0, 0.8, 0.5 | 2.0, 1.0, 0.8, 0.5 | 2.0, 1.0, 0.8, 0.5 | 2.0, 1.0, 0.8, 0.5 |  |
|  | Sensitivity minimum of receiver | dBm | -115.8 | -115.8 | -115.8 | -115.8 |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1 | 1 | 1 | 1 |  |
|  | Detection range | km | 500 | 500 | 500 | 500 |  |
|  | Scan mode in observation |  | 2 | 2 | 2 | 2 |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 2 | 2 | 2 | 2 |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) | 3 (constantly tracking) |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 2 (Not operational) | 1 (Operational) | 2 (Not operational) | 2 (Not operational) |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Republic of Korea - 1

| NAME OF STATION |  |  | Gosan | Seongsan | Gangneung | Oseongsan | Baengnyeong-do |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 47185 | 47188 | 47105 | 47144 | 47102 |
|  | Location of station |  | 33.294329°N | 33.387103°N | 37.817669°N | 36.012700°N | 37.967549°N |
|  |  |  | 126.163073°E | 126.879986°E | 128.865647°E | 126.784168°E | 124.630307°E |
|  | Antenna elevation | m | 103 | 68 | 99 | 234 | 185 |
|  | Wave length | cm | 10.61 | 10.88 | 10.50 | 10.96 | 10.45 |
|  | Peak power of transmitter | kW | 850 | 850 | 850 | 850 | 850 |
|  | Pulse length | µs | 0.5, 1.0 2.0, 4.5 | 0.5, 1.0 2.0, 4.5 | 0.5, 1.0 2.0, 4.5 | 0.5, 1.0 2.0, 4.5 | 0.5, 1.0 2.0, 4.5 |
|  | Sensitivity minimum of receiver | dBm | -114 | -114 | -114 | -114 | -114 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | Detection range | km | 240, 480 | 240, 480 | 240, 480 | 240, 480 | 240, 480 |
|  | Scan mode in observation |  | 1, 2 | 1, 2 | 1, 2 | 1, 2 | 1, 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (5-minutely) | 3 (5-minutely) | 3 (5-minutely) | 3 (5-minutely) | 3 (5-minutely) |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Republic of Korea - 2

| NAME OF STATION |  |  | Jindo | Gwangdeok -san | Myeonbong -san | Gwanaksan |  | Gudeoksan |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |  |
|  | Index number |  | 47175 | 47094 | 47148 | 47116 |  | 47160 |
|  | Location of station |  | 34.472553°N | 38.117316°N | 36.179323°N |  | 37.444119°N | 35.118694°N |
|  |  |  | 126.323994°E | 127.433708°E | 128.997319°E |  | 126.963994°E | 128.999744°E |
|  | Antenna elevation | m | 497 | 1066 | 1136 |  | 641 | 549 |
|  | Wave length | cm | 10.37 | 10.38 | 10.99 |  | 11.03 | 11.05 |
|  | Peak power of transmitter | kW | 850 | 850 | 850 |  | 850 | 850 |
|  | Pulse length | µs | 0.5, 1.0 2.0, 4.5 | 0.5, 1.0 2.0, 4.5 | 0.5, 1.0 2.0, 4.5 | 0.5, 1.0 2.0, 4.5 |  | 0.5, 1.0 2.0, 4.5 |
|  | Sensitivity minimum of receiver | dBm | -114 | -114 | -114 | -114 |  | -114 |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 1.0 | 1.0 | 1.0 | 1.0 |  | 1.0 |
|  | Detection range | km | 240, 480 | 240, 480 | 240, 480 | 240, 480 |  | 240, 480 |
|  | Scan mode in observation |  | 1, 2 | 1, 2 | 1, 2 | 1, 2 |  | 1, 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 |  | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 |  | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 |  | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (5-minutely) | 3 (5-minutely) | 3 (5-minutely) | 3 (5-minutely) |  | 3 (5-minutely) |
|  | cyclone is within range of detection) |  |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 |  | 1 |
|  | 1. Operational |  |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |  |

##### Name of the Member  Republic of Korea - 3

| NAME OF STATION |  |  | Korean Aviation Meteorological Agency |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 47113 |  |  |  |  |
|  | Location of station |  | 37° 28´ N |  |  |  |  |
|  |  |  | 126° 21´ E |  |  |  |  |
|  | Antenna elevation | m | 145 |  |  |  |  |
|  | Wave length | cm | 5.32 |  |  |  |  |
|  | Peak power of transmitter | kW | 250 |  |  |  |  |
|  | Pulse length | µs | 1.0; 2.0 |  |  |  |  |
|  | Sensitivity minimum of receiver | dBm | -110 |  |  |  |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) | deg | 0.53 |  |  |  |  |
|  | Detection range | km | 130, 428 |  |  |  |  |
|  | Scan mode in observation |  | 1, 2 |  |  |  |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 |  |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 |  |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 |  |  |  |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (continuous) |  |  |  |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 |  |  |  |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Singapore

| NAME OF STATION |  |  | Changi |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 48698 |  |  |  |  |
|  | Location of station |  | 1° 22´ N |  |  |  |  |
|  |  |  | 103° 59´ E |  |  |  |  |
|  | Antenna elevation | m | 35 |  |  |  |  |
|  | Wave length | cm | 10 |  |  |  |  |
|  | Peak power of transmitter | kW | 750 |  |  |  |  |
|  | Pulse length | µs | 1 or 3 |  |  |  |  |
|  | Sensitivity minimum of receiver | dBm | -110 |  |  |  |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) |  | < 1 |  |  |  |  |
|  |  | deg |  |  |  |  |  |
|  | Detection range | km | 480 |  |  |  |  |
|  | Scan mode in observation |  | 2 |  |  |  |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 |  |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 |  |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 |  |  |  |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 (continuous) |  |  |  |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 |  |  |  |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Thailand - 1

| NAME OF STATION |  |  | Chiang Rai | Sakol Nakon | Phitsanulok | Khon Khaen | Ubon Ratchathani |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 48303 | 48356 | 48378 | 48381 | 48407 |
|  | Location of station |  | 19° 57´ N | 17° 09´ N | 16° 47´ N | 16° 27´ N | 15° 14´ N |
|  |  |  | 99° 52´ E | 104° 07´ E | 100° 16´ E | 102° 47´ E | 105° 01´ E |
|  | Antenna elevation | m | 440 | 198 | 56 | 215 | 155 |
|  | Wave length | cm | 5 | 5 | 5 | 5 | 5 |
|  | Peak power of transmitter | kW | 350 | 350 | 350 | 350 | 350 |
|  | Pulse length | µs | 0.8&2 | 0.8&2 | 0.8&2 | 0.8&2 | 0.8&2 |
|  | Sensitivity minimum of receiver | dBm | -110 | -110 | -110 | -100 | -100 |
|  | Beam width (Width of over -3dB antenna gain of maximum) |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | deg |  |  |  |  |  |
|  | Detection range | km | 240 | 240 | 240 | 240 | 240 |
|  | Scan mode in observation |  | 1, 2 | 1, 2 | 1, 2 | 1, 2 | 1, 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1, 3 | 1, 3 | 1, 3 | 1, 3 | 1, 3 |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Thailand - 2

| NAME OF STATION |  |  | Samut Songkram | Hua Hin | Chumporn | Surat Thani | Krabi |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 48438 | 48475 | 48517 | 48551 | 48563 |
|  | Location of station |  | 13° 24´ N | 12° 35´ N | 10° 29´ N | 9° 08´ N | 8° 06´ N |
|  |  |  | 100° 24´ E | 99° 57´ E | 99° 11´ E | 99° 09´ E | 98° 58´ E |
|  | Antenna elevation | m | 29 | 30 | 28 | 33 | 51 |
|  | Wave length | cm | 5 | 10 | 5 | 5 | 5 |
|  | Peak power of transmitter | kW | 350 | 350 | 350 | 350 | 350 |
|  | Pulse length | µs | 0.8&2 | 0.8&2 | 0.8&2 | 0.8&2 | 0.8&2 |
|  | Sensitivity minimum of receiver | dBm | -110 | -115 | -110 | -110 | -110 |
|  | Beam width (Width of over -3dB antenna gain of maximum) |  | 1.0 | 2.1 | 1.0 | 1.0 | 1.0 |
|  |  | deg |  |  |  |  |  |
|  | Detection range | km | 240 | 240 | 240 | 240 | 240 |
|  | Scan mode in observation |  | 1, 2 | 1, 2 | 1, 2 | 1, 2 | 1, 2 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1, 3 | 1, 3 | 1, 3 | 1, 3 | 1, 3 |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Thailand - 3

| NAME OF STATION |  |  | Sathing Pra (Songkla) | Narathiwat |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 48568 | 48583 |  |  |  |
|  | Location of station |  | 7° 26´ N | 6° 25´ N |  |  |  |
|  |  |  | 100° 27´ E | 101° 49´ E |  |  |  |
|  | Antenna elevation | m | 30 | 29 |  |  |  |
|  | Wave length | cm | 5 | 5 |  |  |  |
|  | Peak power of transmitter | kW | 350 | 350 |  |  |  |
|  | Pulse length | µs | 0.8&2 | 0.8&2 |  |  |  |
|  | Sensitivity minimum of receiver | dBm | -115 | -110 |  |  |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) |  | 1.0 | 1.0 |  |  |  |
|  |  | deg |  |  |  |  |  |
|  | Detection range | km | 240 | 120 |  |  |  |
|  | Scan mode in observation |  | 1, 2 | 1, 2 |  |  |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 |  |  |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1, 3 | 1, 3 |  |  |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 |  |  |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  USA

| NAME OF STATION |  |  | Guam | Kwajalein |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 91217 | 91366 |  |  |  |
|  | Location of station |  | 13° 33´ N | 8° 44´ N |  |  |  |
|  |  |  | 144° 50´ E | 167° 44´ E |  |  |  |
|  | Antenna elevation | m | 110 | 30 |  |  |  |
|  | Wave length | cm | 10.6 | 10.0 |  |  |  |
|  | Peak power of transmitter | kW | 750 | 500 |  |  |  |
|  | Pulse length | µs | 1.57/ 4.5 | 0.8 |  |  |  |
|  | Sensitivity minimum of receiver | dBm | -113 | -107 |  |  |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) |  | 0.96 | 1.0 |  |  |  |
|  |  | deg |  |  |  |  |  |
|  | Detection range | km | 399 | 250 |  |  |  |
|  | Scan mode in observation |  | 2 | 2 |  |  |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 2 |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 | 1 |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 |  |  |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 3 6-minute continuous | 3 continuous |  |  |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 |  |  |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Viet Nam - 1

| NAME OF STATION |  |  | Phu Lien | Viet Tri | Vinh | Tam Ky | Nha Trang |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  | 48826 | 48813 | 48845 | 48833 | 48877 |
|  | Location of station |  | 20.48 °N | 21.18 °N | 18.40 °N | 15.34 °N | 12.13 °N |
|  |  |  | 106.38 °E | 105.25 °E | 105.41 °E | 108.28 °E | 109.12 °E |
|  | Antenna elevation | m | 140 | 56 | 27 | 40 | 52 |
|  | Wave length | cm | 5.3 | 5.3 | 5.3 | 5.6 | 5.6 |
|  | Peak power of transmitter | kW | 250 | 250 | 250 | 250 | 250 |
|  | Pulse length | µs | 2 | 2 | 2 | 0.8;2.0 | 0.8;2.0 |
|  | Sensitivity minimum of receiver | dBm | -110 | -110 | -110 | -113 | -113 |
|  | Beam width (Width of over -3dB antenna gain of maximum) |  | 1.1 | 1.1 | 1.1 | 1 | 1 |
|  |  | deg |  |  |  |  |  |
|  | Detection range | km | 384 | 384 | 384 | 480 | 480 |
|  | Scan mode in observation |  | 1, 3 | 1, 3 | 1, 3 | 1, 2, 3 | 1, 2, 3 |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 2 | 2 | 2 | 1 | 1 |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1, 3 | 1, 3 | 1, 3 | 1, 3 | 1, 3 |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 | 1 | 1 | 1 | 1 |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

##### Name of the Member  Viet Nam - 2

| NAME OF STATION |  |  | Nha Be |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATIONS | Unit |  |  |  |  |  |
|  | Index number |  |  |  |  |  |  |
|  | Location of station |  | 10° 49´ N |  |  |  |  |
|  |  |  | 106° 43´ E |  |  |  |  |
|  | Antenna elevation | m | 25 |  |  |  |  |
|  | Wave length | cm | 5.6 |  |  |  |  |
|  | Peak power of transmitter | kW | 250 |  |  |  |  |
|  | Pulse length | µs | 0.4; 0.8; 2.0 |  |  |  |  |
|  | Sensitivity minimum of receiver | dBm | -122 |  |  |  |  |
|  | Beam width (Width of over -3dB antenna gain of maximum) |  | 1 |  |  |  |  |
|  |  | deg |  |  |  |  |  |
|  | Detection range | km | 480 |  |  |  |  |
|  | Scan mode in observation |  | 1, 2, 3 |  |  |  |  |
|  | 1. Fixed elevation |  |  |  |  |  |  |
|  | 2. CAPPI |  |  |  |  |  |  |
|  | 3. Manually controlled |  |  |  |  |  |  |
|  | DATA PROCESSING |  |  |  |  |  |  |
|  | MTI processing |  | 1 |  |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Doppler processing |  | 1 |  |  |  |  |
|  | 1. Yes, 2. No |  |  |  |  |  |  |
|  | Display |  | 1 |  |  |  |  |
|  | 1. Digital, 2. Analog |  |  |  |  |  |  |
|  | OPERATION MODE (When tropical |  | 1, 3 |  |  |  |  |
|  | cyclone is within range of detection) |  |  |  |  |  |  |
|  | 1. Hourly |  |  |  |  |  |  |
|  | 2. 3-hourly |  |  |  |  |  |  |
|  | 3. Others |  |  |  |  |  |  |
|  | PRESENT STATUS |  | 1 |  |  |  |  |
|  | 1. Operational |  |  |  |  |  |  |
|  | 2. Not operational (for research etc.) |  |  |  |  |  |  |

### APPENDIX 2-F

71

#### TECHNICAL SPECIFICATIONS OF SATELLITE OPERATED BY TYPHOON COMMITTEE MEMBERS

##### 1. FY-2G (operational since 2015) / FY-2H (operational since 2019) [China]

  - Observations

    - Full-Disk Observations (FY-2G/H): 		Every hour

    - Regional Observations based on request (FY-ESM):				 	Every 6 minutes

  - Products

    - Full-Disk Observation Data (FY-2G/H): 	Every hour

    - Regional Observation Data based on request (FY-ESM8):	 Every 6 minutes

    - Full-Disk AMV Product:

  - Dissemination ways

    - Direct Broadcast Services

    - CMAcast (communication satellite dissemination service)

    - Internet Services

[National Satellite Meteorological Center Portal Site]

https://nsmc.org.cn/nsmc/en/home/index.html

[FengYun Satellite Data Center Site]

http://data.nsmc.org.cn/DataPortal/en/home/index.html

[Real-time imagery, FengYun Satellite Weather Application Platform(SWAP)]

http://rsapp.nsmc.org.cn/test_geofy/en

##### 2. FY-4B (operational since 2022) [China]

- Observations

- Full-Disk Observations (FY-4B):	 Every 15 minutes

- Regional Observations (FY-4B): 	1 minutes

- Regional Observations based on request (FY-ESM8):				 	Every 15 or 1 minutes

- Products

- Full-Disk Observation (FY-4B): 	Every 15 minutes

- Asia region Observation (FY-4B): 	1 minutes

- Regional Observations Data based on request (FY-4B): Every 15 or 1 minutes

- Dissemination ways

- Direct Broadcast Service

- CMACast (communication satellite dissemination service)

- Internet Services

[National Satellite Meteorological Center Portal Site]

https://nsmc.org.cn/nsmc/en/home/index.html

[FengYun Satellite Data Center Site]

http://data.nsmc.org.cn/DataPortal/en/home/index.html

[Real-time imagery, FengYun Satellite Weather Application Platform (SWAP)]

http://rsapp.nsmc.org.cn/test_geofy/en

3. Himawari-8(backup operation since 2022) / Himawari-9 (observation operation since 2022) [Japan]

- Observations

- Full-Disk Observations: 	Every 10 minutes

- Japan Area Observations:	 Every 2.5 minutes

- Target Area Observations Including Those Based on Request by NMHSs (HimawariRequest): Every 2.5 minutes

- Products

- Full-Disk Observation Data: 		Every 10 minutes

- Japan Area Observation Data: 		Every 2.5 minutes

- Target Area Observation Data: Every 2.5 minutes

- Full-Disk AMV:			 Every hour

- Full-Disk Clear Sky Radiance (CSR):	 Every hour

- AMV-based Sea-surface Wind data (ASWind) (Full-Disk) : Every 30 minutes

- AMV-based Sea-surface Wind data (ASWind) (Target Area) : Every 10 minutes

- Dissemination ways

- HimawariCloud (Internet Cloud Service)

Service which distributes full-spec imagery derived from the Himawari-series satellites

(https://www.data.jma.go.jp/mscweb/en/himawari89/cloud_service/cloud_service.html)

- HimawariCast (communication satellite dissemination service)

Service which disseminates primary sets of imagery from the Himawari-series satellites via a communication satellite

(https://www.data.jma.go.jp/mscweb/en/himawari89/himawari_cast/himawari_cast.php)

- Internet Services for National Meteorological and Hydrological Services (NMHSs)

[JMA real-time satellite imagery webpage]

https://www.jma.go.jp/bosai/map.html#contents=himawari&lang=en

[MSC (Meteorological Satellite Center of JMA) real-time satellite imagery webpage]

https://www.data.jma.go.jp/mscweb/data/himawari/

[SATAID (Satellite Animation and Interactive Diagnosis) Service]

https://www.wis-jma.go.jp/cms/sataid/

[JDDS (JMA Data Dissemination Service)]

https://www.jma.go.jp/jma/jma-eng/satellite/jdds.html

##### 4. GEO-KOMPSAT-2A (operational since 2019) [Republic of Korea]

- Observations

- Full-Disk Observations: 		Every 10 minutes

- Extended Local Area Observations:	 Every 2 minutes

- Local Area Observations: Every 2 minutes

- Products

- Full-Disk Observation Data:			 Every 10 minutes

- Extended Local Area Observation Data:	 Every 2 minutes

- Local Area Observation Data: Every 2 minutes

- Dissemination ways

- Direct Broadcast Service

Request application form for receiving station

(https://nmsc.kma.go.kr/enhome/html/base/cmm/selectPage.do?page=static.utilization.reqStation))

- Internet Services

[FTP-based Service]

All sixteen channels data of full-disk image will be put on KMA’s FTP server designated for GEO-KOMPSAT-2A data dissemination in every 10 minutes.

(Account policy: 1 account per 1 organization)

Need personal contact (kmasod@korea.kr)

Request application form for FTP Service (Realtime FTP Service)

(ftp://210.125.45.78/SAT/GK2A/AMI/L1B/FD/{YYYY}{MM}/{DD}/{HH})

To obtain permission to download data via real-time FTP, an institution-specific ID application is required.

[National Meteorological Satellite Center website]

https://nmsc.kma.go.kr/enhome/html/main/main.do

[KMA Weather Data Service]

https://data.kma.go.kr/resources/html/en/aowdp.html

[Data Collection or Production Centre website]

http://dcpc.nmsc.kma.go.kr/openwis-user-portal/srv/en/main.home

### APPENDIX 2-G

72

#### SATELLITE IMAGERY RECEIVING FACILITIES AT TYPHOON COMMITTEE MEMBERS

| Member | Station |  | FengYun-2 | FengYun-3 | FengYun-4 | FengYun-2/3/4 | Himawari-8/9 |  | GEO-KOMPSAT-2A | NOAA/JPSS | AQUA/TERRA | METOP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Direct Broadcast | Direct Broadcast | Direct Broadcast | CMAcast | HimawariCast | HimawariCloud | Direct Broadcast | Direct Broadcast | Direct Broadcast | Direct Broadcast |
| Cambodia |  |  |  |  |  |  |  |  |  |  |  |  |
| China | Beijing | 39.9°N, 116.4°E |  |  |  |  |  |  |  |  |  |  |
| DPR Korea | Pyongyang | 39.0°N, 125.8°E |  |  |  |  |  |  |  |  |  |  |
| Hong Kong, China | Kowloon | 22.3°N, 114.2°E |  |  |  |  |  |  |  |  |  |  |
| Japan | Kiyose | 35.8°N, 139.5°E |  |  |  |  |  |  |  |  |  |  |
| Lao PDR |  |  |  |  |  |  |  |  |  |  |  |  |
| Macao, China | Macao | 22.2°N, 113.5°E |  |  |  |  |  |  |  |  |  |  |
| Malaysia | Petaling Jaya | 3.1°N, 101.7°E |  |  |  |  |  |  |  |  |  |  |
| Philippines | Lapu-Lapu City | 10.3°N, 124.0°E |  |  |  |  |  |  |  |  |  |  |
|  | Davao City | 7.1°N, 125.6°E |  |  |  |  |  |  |  |  |  |  |
|  | El Salvador City | 8.5°N, 124.6°E |  |  |  |  |  |  |  |  |  |  |
|  | Legaspi City | 13.1°N, 123.7°E |  |  |  |  |  |  |  |  |  |  |
|  | Quezon City | 14.7°N, 121.0°E |  |  |  |  |  |  |  |  |  |  |
|  | Tacloban City | 11.2°N, 125.0°E |  |  |  |  |  |  |  |  |  |  |
|  | Tuguegarao City | 17.6°N, 121.8°E |  |  |  |  |  |  |  |  |  |  |
| Republic of Korea | Jinchoen | 36.7°N, 127.4°E |  |  |  |  |  |  |  |  |  |  |
| Singapore | Changi Airport | 1.4°N, 104.0°E |  |  |  |  |  |  |  |  |  |  |
| Thailand | Bangkok | 13.7°N, 100.6°E |  |  |  |  |  |  |  |  |  |  |
| USA | Guam | 13.4°N, 144.6°E |  |  |  |  |  |  |  |  |  |  |
|  | NCEP/College Park | 39.0°N, 76.9°W |  |  |  |  |  |  |  |  |  |  |
| Viet Nam | Hanoi | 21.0°N, 105.5°E |  |  |  |  |  |  |  |  |  |  |
|  | Ho Chi Ming City | 10.5°N, 106.4°E |  |  |  |  |  |  |  |  |  |  |

### APPENDIX 2-H

73

#### LIST OF SAREP REPORTS ISSUED BY TYPHOON COMMITTEE MEMBERS

| Member | Frequency | Heading in the BUFR code (FM 94) | Issuance Condition |
| --- | --- | --- | --- |
| RSMC Tokyo - Typhoon Center | 8 times/day | IUCC10 RJTD | - When a tropical cyclone of TS intensity or higher is located in the responsible area of the RSMC Tokyo - Typhoon Center; - When a tropical depression existing in the responsible area is forecasted to have an intensity of TS or higher within 24 hours. |
| Hong Kong, China | 8 times/day | IUCC01 VHHH IUCC02 VHHH IUCC03 VHHH IUCC04 VHHH | When a tropical cyclone is located within 10°N to 30°N and 105°E to 125°E. |
| China | 8 times/day | TCPQ40 BABJ | When a tropical cyclone is located within 0°N to 50°N and 105°E to 180°E. |

### APPENDIX 2-I

74

#### RECONNAISSANCE FLIGHTS CONDUCTED BY TYPHOON COMMITTEE MEMBERS

HKO conducts dropsonde reconnaissance flights for selected tropical cyclones over the northern part of the South China Sea. Data are disseminated in real time to near real time in BUFR format through GTS circuit. Automatic data quality control algorithms are implemented to remove suspicious and erratic data from the dropsonde.

### APPENDIX 3-A

10

#### PRODUCTS PROVIDED BY RSMC TOKYO - TYPHOON CENTER

##### Chart-form products provided by RSMC Tokyo - Typhoon Center for regional purposes

| Area | Contents and Level | Forecast hours | Initial time | Availability |
| --- | --- | --- | --- | --- |
| A’ (Far East) | 500 hPa (Z, ζ) | Analysis | 00, 12UTC | GTS |
|  |  | 24, 36 | 00, 12UTC | GTS, JMH |
|  | 500 hPa (T), 700 hPa (D) | 24, 36 | 00, 12UTC | GTS, JMH |
|  | 700 hPa (ω), 850 hPa (T, A) | Analysis | 00, 12UTC | GTS |
|  |  | 24, 36 | 00, 12UTC | GTS, JMH |
|  | Surface (P, R, A) | 24, 36 | 00, 12UTC | GTS, JMH |
| C (East Asia) | 300 hPa (Z, T, W, A) | Analysis | 00UTC | GTS |
|  | 500 hPa (Z, T, A) | Analysis | 00, 12UTC | GTS, JMH |
|  | 500 hPa (Z, ζ) | 48, 72 | 00, 12UTC | GTS |
|  | 700 hPa (Z, T, D, A) | Analysis | 00, 12UTC | GTS |
|  | 700 hPa (ω), 850 hPa (T, A) | 48, 72 | 12UTC | GTS |
|  | 850 hPa (Z, T, D, A) | Analysis | 00, 12UTC | GTS, JMH |
|  | Surface (P, R) | 24 | 00, 12UTC | GTS |
|  |  | 48, 72 | 00, 12UTC | GTS, JMH |
|  |  | 96, 120 | 12UTC | GTS, JMH |
| O (Asia) | 500 hPa (Z, ζ) | 96, 120, 144, 168, 192 | 12UTC | GTS |
|  | 850 hPa (T), Surface (P) |  |  |  |
| Q (Asia Pacific) | 200 hPa (Z, T, W), Tropopause (Z) | Analysis | 00, 12UTC | GTS |
|  | 250 hPa (Z, T, W) | Analysis, 24 | 00, 12UTC |  |
|  | 500 hPa (Z, T, W) | 24 | 00, 12UTC |  |
| D (N.H.) | 500 hPa (Z, T) | Analysis | 12UTC | GTS |
| W (NW Pacific) | 200 hPa (streamline) | Analysis, 24, 48 | 00, 12UTC | GTS |
|  | 850 hPa (streamline) |  | 00, 12UTC |  |
| X (Japan) | Ocean Wave (J, M, G and observation plots) | Analysis | 00, 12UTC | GTS, JMH |
|  | Ocean Wave (J, M, G, and rough sea area ) | 24 |  |  |
| C’’ (NW Pacific) | Ocean Wave (J, M, G) | Analysis, 12, 24, 48, 72 | 00, 12UTC | GTS, JMH |
|  | Ocean Wave (J, M, G and rough sea area) | 24 |  |  |
| C’’2 (NW Pacific) | Sea Surface Temperature | Daily analysis | - | GTS, JMH |
| C’2 (Asia Pacific) | Surface (P) | Analysis | 00,06,12, 18UTC | GTS, JMH |
|  |  | 24 | 00, 12UTC |  |
|  |  | 48 |  |  |
|  | Surface (Typhoon Forecast) | 12,24,48,72 | 00,06,12, 18UTC |  |
|  |  | 24,48,72,96, 120 |  | JMH |

Notes:

(a) Area

A’, C, O, Q, D, W, X, C’’, C’’2 and C’2 are illustrated in figure of the next page.

(b) Contents

Z: geopotential height		ζ: vorticity		T: temperature

D: dewpoint depression 	ω: vertical velocity 	W: wind speed by isotach

A: wind arrows		P: sea level pressure 	R: rainfall

J: wave height		M: wave period	G: arrow for prevailing wave direction

Output areas for facsimile charts transmitted through GTS

and radio facsimile JMH

##### NWP products (GSM and GEPS) provided by RSMC Tokyo - Typhoon Center

(Available at https://www.wis-jma.go.jp/cms/)

| Model | GSM | GSM | GSM |
| --- | --- | --- | --- |
| Area and resolution | Whole globe, 1.25°×1.25° | 20°S-60°N, 60°E-160°W 1.25°×1.25° | Whole globe, 2.5°×2.5° |
| Levels and elements | 10 hPa: Z, U, V, T 20 hPa: Z, U, V, T 30 hPa: Z, U, V, T 50 hPa: Z, U, V, T 70 hPa: Z, U, V, T 100 hPa: Z, U, V, T 150 hPa: Z, U, V, T 200 hPa: Z, U, V, T, ψ, χ 250 hPa: Z, U, V, T 300 hPa: Z, U, V, T, H, ω 400 hPa: Z, U, V, T, H, ω 500 hPa: Z, U, V, T, H, ω, ζ 600 hPa: Z, U, V, T, H, ω 700 hPa: Z, U, V, T, H, ω 850 hPa: Z, U, V, T, H, ω, ψ, χ 925 hPa: Z, U, V, T, H, ω 1000 hPa: Z, U, V, T, H, ω Surface: P, U, V, T, H, R† | 10 hPa: Z, U, V, T 20 hPa: Z, U, V, T 30 hPa: Z, U, V, T 50 hPa: Z, U, V, T 70 hPa: Z, U, V, T 100 hPa: Z, U, V, T 150 hPa: Z, U, V, T 200 hPa: Z§, U§, V§, T§, ψ, χ 250 hPa: Z, U, V, T 300 hPa: Z, U, V, T, D 400 hPa: Z, U, V, T, D 500 hPa: Z§, U§, V§, T§, D§, ζ 700 hPa: Z§, U§, V§, T§, D§, ω 850 hPa: Z§, U§, V§, T§, D§, ω, ψ, χ 925 hPa: Z, U, V, T, D, ω 1000 hPa: Z, U, V, T, D Surface: P¶, U¶, V¶, T¶, D¶, R¶ | 10 hPa: Z*, U*, V*, T* 20 hPa: Z*, U*, V*, T* 30 hPa: Z°, U°, V°, T° 50 hPa: Z°, U°, V°, T° 70 hPa: Z°, U°, V°, T° 100 hPa: Z°, U°, V°, T° 150 hPa: Z*, U*, V*, T* 200 hPa: Z, U, V, T 250 hPa: Z°, U°, V°, T° 300 hPa: Z, U, V, T, D*‡ 400 hPa: Z*, U*, V*, T*, D*‡ 500 hPa: Z, U, V, T, D*‡ 700 hPa: Z, U, V, T, D 850 hPa: Z, U, V, T, D 1000 hPa: Z, U*, V*, T*, D*‡ Surface: P, U, V, T, D*‡, R† |
| Forecast hours | 0 - 84 every 6 hours and 96 - 192 every 12 hours for 12UTC initial † Except analysis | 0 - 84 (every 6 hours) § 96 - 192 (every 24 hours) for 12UTC initial ¶ 90 - 192 (every 6 hours) for 12UTC initial | 0 - 72 every 24 hours and 96 - 192 every 24 hours for 12UTC ° 0 - 120 for 12UTC † Except analysis * Analysis only |
| Initial times | 00, 06, 12, 18UTC | 00, 06, 12, 18UTC | 00UTC and 12UTC ‡ 00UTC only |

| Model | GEPS | GEPS |
| --- | --- | --- |
| Area and resolution | Whole globe, 2.5°×2.5° | Whole globe, 1.25°×1.25 |
| Levels and elements | 250 hPa: µU, σU, µV, σV 500 hPa: µZ, σZ 850 hPa: µU, σU, µV, σV, µT, σT 1000 hPa: µZ, σZ Surface: µP, σP | 250 hPa: µU, σU, µV, σV, µW,σW 500 hPa: µZ, σZ 850 hPa: µU, σU, µV, σV, µT, σT, µW, σW ,Probability of temperature anomalies [±1, ±1.5, ±2σ] 1000 hPa: µZ, σZ Surface: µP, σP, Probability of 10 m sustained wind and gusts[10,15,25 m/s] †, Probability of precipitation [1,5,10,25,50,100 mm/24hour] † |
| Forecast hours | 0 - 192 every 12 hours | 0 - 264 every 12 hours † Except analysis |
| Initial times | 00, 12UTC | 00, 12 UTC |

| Model | GSM | GSM | GSM |
| --- | --- | --- | --- |
| Area and resolution | 5°S-90°N and 30°E-165°W, Whole globe 0.25° × 0.25° | 5°S-90°N and 30°E-165°W, Whole globe 0.5° × 0.5° | Whole globe, 1.25°×1.25° |
| Levels and elements | Surface: U, V, T, H, P, Ps, R†, Cla, Clh, Clm, Cll | 10 hPa: Z, U, V, T, H, ω 20 hPa: Z, U, V, T, H, ω 30 hPa: Z, U, V, T, H, ω 50 hPa: Z, U, V, T, H, ω 70 hPa: Z, U, V, T, H, ω 100 hPa: Z, U, V, T, H, ω 150 hPa: Z, U, V, T, H, ω 200 hPa: Z, U, V, T, H, ω, ψ, χ 250 hPa: Z, U, V, T, H, ω 300 hPa: Z, U, V, T, H, ω 400 hPa: Z, U, V, T, H, ω 500 hPa: Z, U, V, T, H, ω, ζ 600 hPa: Z, U, V, T, H, ω 700 hPa: Z, U, V, T, H, ω 800 hPa: Z, U, V, T, H, ω 850 hPa: Z, U, V, T, H, ω, ψ, χ 900 hPa: Z, U, V, T, H, ω 925 hPa: Z, U, V, T, H, ω 950 hPa: Z, U, V, T, H, ω 975 hPa: Z, U, V, T, H, ω 1000 hPa: Z, U, V, T, H, ω Surface: U, V, T, H, P, Ps, R†, Cla, Clh, Clm, Cll | 10 hPa: Z, U, V, T 20 hPa: Z, U, V, T 30 hPa: Z, U, V, T 50 hPa: Z, U, V, T 70 hPa: Z, U, V, T 100 hPa: Z, U, V, T 150 hPa: Z, U, V, T 200 hPa: Z, U, V, T, ψ, χ 250 hPa: Z, U, V, T, ζ, Div 300 hPa: Z, U, V, T, H, ω 400 hPa: Z, U, V, T, H, ω 500 hPa: Z, U, V, T, H, ω, ζ 600 hPa: Z, U, V, T, H, ω 700 hPa: Z, U, V, T, H, ω, ζ, Div 850 hPa: Z, U, V, T, H, ω, ψ, χ 925 hPa: Z, U, V, T, H, ω, ζ, Div 1000 hPa: Z, U, V, T, H, ω Surface: P, U, V, T, H, R† |
| Forecast hours | 0 - 132 (every 3 hours) 138 - 264 (every 6 hours) are available for 00* and 12 UTC Initial † Except analysis * From Feb 17 2021 | 0 - 132 (every 3 hours) 138 - 264 (every 6 hours) are available for 00* and 12 UTC Initial † Except analysis * From Feb. 17 2021 | 0 - 132 every 6 hours and 144 - 264 every 12 hours for 00* and 12UTC initial † Except analysis * From Feb. 17 2021 |
| Initial times | 00, 06, 12, 18 UTC | 00, 06, 12, 18 UTC | 00, 06, 12, 18 UTC |

Notes:	Z: geopotential height	U: eastward wind		V: northward wind

T: temperature	D: dewpoint depression	H: relative humidity

ω: vertical velocity	ζ: vorticity		ψ: stream function

χ: velocity potential	P: sea level pressure	Ps: pressure

R: rainfall	Cla: total cloudiness	Clh: cloudiness (upper layer)

Clm: cloudiness (middle layer)	Cll: cloudiness (lower layer)

Div: divergence	W:wind speed

The prefixes µ and σ represent the average and standard deviation of ensemble prediction results respectively.

The symbols °, *, ¶, §, ‡ and † indicate limitations on forecast hours or initial time as shown in the tables.

##### List of other products provided by RSMC Tokyo - Typhoon Center

(Available at the Global Information System Center Tokyo server:

https://www.wis-jma.go.jp/cms/)

| Data | Contents / frequency (initial time) |
| --- | --- |
| Satellite products | High density atmospheric motion vectors (BUFR) Himawari-8/9 (VIS, IR, WVx3: every hour), 60°S-60°N, 90°E-170°W Clear Sky Radiance (CSR) data (BUFR) Himawari-8/9 radiances and brightness temperatures averaged over cloud-free pixels: every hour |
| Tropical cyclone Information | Tropical cyclone related information (BUFR) • tropical cyclone analysis data (00, 06, 12 and 18 UTC) |
| Wave data | Global Wave Model (GRIB2) • significant wave height • peak wave period • wave direction Forecast hours: 0 – 84 every 6 hours (00, 06 and 18UTC) 0 – 84 every 6 hours and 96-192 every 12 hours (12 UTC) |
| Observational data | - Surface data (TAC/TDCF) SYNOP, SHIP, BUOY: Mostly 4 times a day (b) Upper-air data (TAC/TDCF) TEMP (parts A-D), PILOT (parts A-D): Mostly twice a day |
| SATAID service | - Satellite imagery (SATAID) Himawari-8/9 (b) Observation data (SATAID) SYNOP, SHIP, METAR, TEMP (A, B) and ASCAT sea surface wind (c) NWP products (SATAID) GSM (Available at https://www.wis-jma.go.jp/cms/sataid/) |

##### List of other products provided by RSMC Tokyo - Typhoon Center

(Available at the Numerical Typhoon Prediction Website:

https://tynwp-web.kishou.go.jp/)

| Products | Frequency | Details |
| --- | --- | --- |
| RSMC Advisories |  |  |
| RSMC TC Advisory | At least 8 times/day | - RSMC Tokyo - Typhoon Center’s TC analysis and forecasts up to 120-hours (linked to the JMA website at https://www.jma.go.jp/en/typh/) |
| Storm Wind Probability Map | 4 times/day | - Probabilistic forecast map for sustained wind equal to or above 50-kt for 1, 2, 3, 4 and 5 days ahead |
| Prognostic Reasoning | 4 times/day | - RSMC Tokyo Tropical Cyclone Prognostic Reasoning (WTPQ3X) |
| Graphical TC Advisory | 4 times/day | - Graphical TC Advisory including RSMC Tokyo - Typhoon Center’s TC analysis, track and intensity forecasts up to 24-hours and horizontal extents of cumulonimbus cloud and cloud top height associated with TCs potentially affecting aviation safety (linked to the Tropical Cyclone Advisory Center Tokyo Website at https://www.data.jma.go.jp/tca/data/index.html) |
| Remote Sensing |  |  |
| Satellite Analysis | At least 4 times/day | - Results and historical logs of RSMC Tokyo – Typhoon Center’s TC analysis conducted using satellite images (Conventional Dvorak analysis and Early-stage Dvorak analysis) |
| Satellite Imagery | Every 10 minutes | - Satellite imagery of Himawari-8/9 (linked to the JMA website at https://www.jma.go.jp/bosai/map.html#elem=ir&lang=en&contents=himawari |
| Satellite Microwave Products |  | - TC snapshot images - Warm-core-based TC intensity estimates - Weighted consensus TC intensity estimates made using Dvorak analysis and satellite microwave warm-core-based intensity estimates |
| Sea-surface AMV (ASwind) | Every 10 / 30 minutes | - AMV-based Sea-surface Wind in the vicinity of TC (linked to the Meteorological Satellite Center web site at https://www.data.jma.go.jp/mscweb/en/product/product_ASWind.html) |
| Radar Composite Imagery | Every hour | - Radar composite imagery of the Typhoon Committee Regional Radar Network |
| Atmospheric Circulation |  |  |
| Weather Charts | 4 times/day | - Weather maps for surface analysis, 24- and 48-hour forecasts (linked to the JMA website at https://www.jma.go.jp/bosai/weather_map/#lang=en) |
| NWP Multi Center Weather Charts | Twice/day | - Mean sea level pressure and 500 hPa Geopotential height (up to 168 hours) of deterministic NWP models from nine centers (BoM, CMA, CMC, DWD, ECMWF, KMA, NCEP, UKMO and JMA) |
| JMA GSM Analysis and Forecast | 4 times/day | - Upper-air analysis and forecast data based on JMA-GSM - Streamlines at 850, 500 and 200 hPa - Divergence at 200 hPa - Velocity potential at 200 hPa - Vertical Velocity in Pressure Coordinate at 500 hPa - Dew Point Depression at 600 hPa - Curvature Vorticity at 850 hPa - Vertical wind shear between 200 and 850 hPa - Sea Level Pressure - Genesis Potential Index |
| MJO phase diagram | Daily | - MJO phase and amplitude diagram and MJO Hovmöller diagram - (linked to the Tokyo Climate Center web site: - https://ds.data.jma.go.jp/tcc/tcc/products/clisys/mjo/monitor.html - https://ds.data.jma.go.jp/tcc/tcc/products/clisys/ASIA_TCC/mjo_cross.html) |
| Asian Monsoon Monitoring Indices | Daily, only during Apr. - Oct. | - Time series of vertical wind shear, OLR and other indices associated with SW Asian Monsoon (linked to the Tokyo Climate Center web site: https://ds.data.jma.go.jp/tcc/tcc/products/clisys/ASIA_TCC/monsoon_index.html) |
| Ocean Condition |  |  |
| SST | Once/day | - Sea surface temperature and related differences from 24 hours ago |
| TCHP | Once/day | - Tropical cyclone heat potential and related differences from 24 hours ago |
| Numerical TC Prediction |  |  |
| Track Bulletin | 4 times/day | - RSMC Tokyo Tropical Cyclone Track Forecast Bulletin - Track forecast by GSM (FXPQ2X) - Track forecast by GEPS (FXPQ3X) |
| TC Track Prediction | 4 times/day | - TC track prediction of deterministic NWP models from nine centers (BoM, CMA, CMC, DWD, ECMWF, KMA, NCEP, UKMO and JMA) and a related consensus - TC track prediction of EPS models from four centers (ECMWF, NCEP, UKMO and JMA) |
| TC Intensity | 4 times/day | - TC intensity forecast guidance based on the Statistical Hurricane Intensity Prediction Scheme (SHIPS) |
| TC Activity Prediction | Twice/day | - Two- and five-day TC activity prediction maps based on EPS models from four centers (ECMWF, UKMO, NCEP and JMA) and a related consensus |
| TC forecast validation | 4 times/day | - Real-time validation of TC track and intensity forecast of numerical forecast models and related products. |
| Marine Forecast |  |  |
| Storm Surge Forecasts | 4 times/day | - Distribution maps of deterministic storm surge forecast for RSMC Tokyo - Typhoon Center’s TC track forecast and probabilistic forecasts using GEPS ensemble prediction (up to 132 hours) - Time-series storm surge forecast charts for RSMC Tokyo - Typhoon Center’s TC track forecast and TC track forecasts from GEPS ensemble prediction (up to 132 hours) - Time-series representations of sea levels, related anomalies, and wind and sea level pressure based on official forecasts for stations of Typhoon Committee Members (up to 132 hours) - Time-series storm surge forecast charts for RSMC Tokyo - Typhoon Center’s ensemble TC track forecasts with box-and-whisker plots and probabilities of 1-, 2- and 3m-exceeding storm surges (up to 132 hours) |
| Ocean Wave Forecasts | Twice/day | - Distribution maps for ensemble mean, maximum, probability of exceeding various thresholds and ensemble spread of wave height and period based on the Wave Ensemble System (WENS) (up to 264 hours) - Time-series representations with box-and-whisker plots for wave height/period and probability of exceeding various wave height/period thresholds based on the WENS (up to 264 hours) |

##### Example of the products provided by RSMC Tokyo - Typhoon Center

(Left) Storm Wind Probability Map: 	Probabilistic forecast map for sustained wind equal to or above 50-kt with forecast time of 1, 2, 3, 4 and 5 days, to grasp the possible impact for areas of interest. (Right) Sea-surface AMV: AMV-based sea-surface wind in the vicinity of TCs estimated from Himawari-8/9 low-level AMVs. Data are available every 30 minutes for full-disk observation and every 10 minutes for Target Area observation, respectively.

(Left) Sea Surface Temperature analysed with observation data of satellites, buoys, ships. "SST > 26°C to a depth of 60 m" is one of the necessary conditions for TC development and genesis. (Right) Tropical cyclone heat potential: Total heat contents from sea surface down to the depth of the 26°C isotherm, operationally used as TC intensity guidance. Minimum threshold for rapid intensification of TC in the western North Pacific is around 40 kJ/cm2.

(Above) Deterministic Track Prediction of global NWP model of BoM, MSC, CMA, DWD, KMA, UKMO, NCEP, ECMWF and JMA. Track prediction of specific NWP models can be selected for display. (Below) Ensemble Track Prediction of NCEP, UKMO, ECMWF and JMA.

(Left) Storm surge forecast for RSMC Tokyo - Typhoon Center’s official track forecast. (Right) Ocean Wave Height produced by Wave Ensemble System of JMA.

##### Deterministic NWP models used in the Numerical Typhoon Prediction website

| System | Domain | Horizontal Resolution | Number of Vertical Levels | Forecast Range (Initial Time) | Specification of (Model/Data) |
| --- | --- | --- | --- | --- | --- |
| JMA deterministic Global model (GSM) | Global | TQ959 (~13 km) | 128 | 132 hours (06, 18 UTC) 264 hours (00, 12 UTC) | Model |
| BoM deterministic Global model (ACCESS-G) | Global | Lon: 0.18° Lat: 0.12° | - | 240 hours (00, 12UTC) | Data |
| CMA deterministic Global model (CMA _GFS) | Global | 0.25° | - | 120 hours (00, 06,12,18 UTC) | Data |
| CMC deterministic Global model (GDPS) | Global | 1.0° | - | 144 hours (00, 12UTC) | Data |
| DWD deterministic Global model (ICON) | Global | 0.25° | - | 174 hours (00, 12UTC) | Data |
| ECMWF deterministic Global model (IFS-HRES) | Global | 0.5° | - | 240 hours (00, 12 UTC) 90 hours (06, 18 UTC) | Data |
| KMA deterministic Global model (GDAPS) | Global | Lon: 0.125° Lat: 0.125° | - | 240 hours (00, 12UTC), | Data |
| NCEP deterministic Global model (GFS) | Global | 0.5° | - | 264 hours (00, 06, 12, 18 UTC) | Data |
| UKMO deterministic Global model | Global | Lon: 0.83° Lat: 0.56° | - | 120 hours (00, 12 UTC) | Data |

##### EPS used in the Numerical Typhoon Prediction website

| System | Domain | Horizontal Resolution | Number of Vertical Levels | Forecast Range (Initial Time) | Number of Ensemble Members | Specification of (Model/ Data) |
| --- | --- | --- | --- | --- | --- | --- |
| JMA Global EPS (GEPS) | Global | TQ479 (~27 km) | 128 | 132 hours (06, 18 UTC) 264 hours (00, 12 UTC) | 51 | Model |
| ECMWF Global EPS | Global | Only track data | - | 240 hours (00, 12 UTC) 144 hours (06, 18 UTC) | 51 | Data |
| NCEP Global EPS | Global | Only track data | - | 192 hours (00, 06, 12, 18 UTC) | 31 | Data |
| UKMO Global EPS | Global | Only track data | - | 192 hours (00, 06, 12, 18 UTC) | 36 | Data |

### APPENDIX 3-B

31

#### Analysis methods, forecasting methods and NWP for forecasting currently used by the NMSs of Typhoon Committee Members

##### Name of the Member: [China]

###### 1  Tropical Cyclone Analysis

| Parameter | Time | Methods | Other Sources |
| --- | --- | --- | --- |
| Dvorak Intensity (CI, T, DT, PT, MET number) | 00, 03, 06, 09, 12, 15, 18, 21 UTC | Dvorak EIR method (Dvorak, 1984) | Satellite observational data from FY-4 (AGRI images, GHI images, GIIRS sounding data and LMI lightning data)) and FY-3 (Atmospheric vertical temperature profile), other available satellite microwave and sounding data |
| Center Position, Accuracy of center position, Direction and speed of movement | 00, 03, 06, 09, 12, 15, 18, 21 UTC; 1hr interval within 24hr warning area | (1) Satellite images (2) Radar images (3) Other estimation methods which utilize surface observations |  |
| Minimum Sea Level Pressure (MSLP), Maximum Sustained Wind speed (MSW averaged in 2min), Radius of NE/SE/SW/SN at 30 kt radii (R30), 50 kt radii (R50) and 64kt radii(R64) | 00, 03, 06, 09, 12, 15, 18, 21 UTC | (1) Satellite imaage based on Dvorak technique (for MSLP, MSW ) (2) Weather map analysis with full utilization of all observational data available (SYNOP, SHIP, BUOY, ASCAT, AMV including sea surface wind estimated from low-level AMV) (3) Radii of 34-kt, 50-kt and 64-kt winds retrieved from satellite and Doppler winds, AWV observation, oil platform and buoys |  |

###### 2  Tropical Cyclone Forecasting

| Parameter | Issuance Time | Lead Time | Methods |
| --- | --- | --- | --- |
| Track for center position in Latitude and Longitude, Direction and speed of movement, Radius of probability circle | 00, 03, 06, 09, 12, 15, 18, 21 UTC | T + 12 h T + 24 h T + 36 h T + 48 h T + 60 h T + 72 h T + 96 h T + 120 h | (1) Simple consensus method using deterministic Global NWP models of and AI models (2) Global EPSs of CMA, ECMWF, NCEP and UKMO as reference (3) Deterministic regional NWP models of CMA-TYM, SHTM, CMA-trams as reference (4) OBEST method (a consensus method using EPSs of ECMWF, NCEP and UKMO (Dong and Zhang; 2016,Qi et al, 2014), STI Shanghai Selective Tropical Cyclone (STI-SSTC) , STI- western North Pacific tropical cyclone intensity prediction scheme (STI-WIPS) Radius of probability circle: Verification results of past TC track errors according to the ensemble spread of the Global EPSs of CMA, ECMWF, NCEP and UKMO (Chen et al, 2018). |
| Intensity for MSLP and ,MSW, | 00, 03, 06, 09, 12, 15, 18, 21 UTC | T + 12 h T + 24 h T + 36 h T + 48 h T + 60 h T + 72 h T + 96 h T + 120 h | (1) Statistical and dynamical guidance (Chen et al, 2018) (2) Deterministic Global NWP models of CMA and other major centers such as ECMWF, NCEP and UKMO as reference (3) Deterministic Regional NWP models of CMA-TYM, CMA-Trams as reference |

###### 3  NWP Systems in Operational Use

| System | Domain | Horizontal Resolution | Number of Vertical Levels | Forecast Range (Initial Time) | Number of Ensemble Members | Run by (own/other centers) |
| --- | --- | --- | --- | --- | --- | --- |
| CMA-GFS V4.2.3 | Global | 0.125° | 87 | 240h (00, 12UTC);120h (06,18UTC) | _ | Own |
| CMA-GEPS V2.0 | Global | 0.25° | 87 | 360h (00,12UTC) | 31 | Own |
| CMA-TYM V3.0 | 40-180.°E; 15°S-60.0°N | 0.09° | 68 | 120h (00,06,12,18UTC) | _ | Own |
| CMA-REPS V4.0 | 70-145°E;10-60°N | 0.03° | 50 | 72h (06,18UTC) | 15 | Own |
| CMA-MESO V6.0 | 70-145°E 10-60°N | 0.03° | 50 | 72h (00,12UTC) 36h (03,06,09,15,18,21UTC) | _ | Own |
| Shanghai Typhoon Model (SHTM also called SWARMS-TC) | West Pacific Ocean and South China Sea | 9 km | 56 | up to 120h, interval is 1h | _ | Own |
| Shanghai Weather And Risk Model System-Ensemble ( SWARMS-EN) | West Pacific Ocean and South China Sea | 9 km | 56 | up to 120h, interval is 1h | 21 | Own |
| CMA-TRAMS | 70°E-160°E ; 0.8N-54.8°N | 0.09° | 65 | 168hours (00,12 UTC) 72hours (06,18 UTC) | _ | Own |

###### Reference

Chen G. M., Lei X T, Zhang X P, 2018. Verification of tropical cyclone operational forecast in 2017[R]. Ha Noi : UNESCAP/WMO Typhoon Committee:19.

Dong L and Zhang F Q, 2016. OBEST: an observation based ensemble sub-setting technique for tropical cyclone track prediction [J]. Wea Forecasting, 31(1): 57-70.

Dvorak V. F., 1984: Tropical cyclone intensity analysis using satellite data, NOAA Technical Report, 11.

Qi L b, Yu H and Chen P Y 2014. Selective ensemble-mean technique for tropical cyclone track forecast by using ensemble prediction systems [J]. Q. J. R. Meteorol. Soc.140, 805-813.

##### Name of the Member: [Hong Kong, China]

###### 1  Tropical Cyclone Analysis

| Parameter | Time | Methods | Other Sources |
| --- | --- | --- | --- |
| - Position, direction and speed of movement - Intensity (maximum sustained 10-minute mean wind near TC centre) - Central pressure, - Wind radii (of strong, gale, storm and hurricane force winds) | Synoptic hour in general (also subject to observation reception time) | - Position, direction and speed of movement: - Satellite imagery - Radar imagery (reflectivity, zero-isodop on Doppler velocity) - Weather observation from synoptic stations, automatic weather stations, oil rigs and weather buoys - Intensity: - Dvorak analysis on satellite imagery - Radar imagery (Doppler wind) - Weather observation from synoptic stations, automatic weather stations, oil rigs and weather buoys - Dropsonde observations from reconnaissance flight - Central pressure: - Pressure observation from synoptic stations, automatic weather stations, oil rigs and weather buoys - Wind radii: - Weather observation from synoptic stations, automatic weather stations, oil rigs and weather buoys - Radar imagery (Doppler wind) - Dropsonde observations from reconnaissance flight | (a) Scatterometer observations, including Haiyang, ASCAT etc., for analysing position, intensity and wind radii. (b) Microwave images for analysing position and intensity. (c) NOAA Multiplatform Tropical Cyclone Surface Winds Analysis for analysing intensity and wind radii. (d) NOAA Synthetic Aperture Radar Wind Products for analysing intensity and wind radii. (e) AI position fix and D’vorak analysis. |

###### 2  Tropical Cyclone Forecasting

| Parameter | Issuance Time | Lead Time | Methods |
| --- | --- | --- | --- |
| Track | Around 1 to 2 hours from the synoptic hour (T) | Forecast positions for: T + 24 h T + 48 h T + 72 h T + 96 h T + 120 h | Weighted ensemble forecast track is generated from 5 NWP guidance including JMA, UKMO, NCEP, ECMWF and ECMWF EPS, and 5 AI models including AIFS, Fengwu, Fuxi, Graphcast and Pangu-Weather. The ensemble forecast track forms the basis for formulating the operational TC forecast track. The operational TC forecast track may be slightly adjusted considering other NWP guidance (e.g. CMA, other EPS products from CMC, KMA, JMA, NCEP and UKMO), other AI models, real-time observations and past performance of models. |
| Intensity (maximum sustained wind) | Around 1 to 2 hours from the synoptic hour (T) | Forecast intensity for: T + 24 h T + 48 h T + 72 h T + 96 h T + 120 h | The intensity forecast makes reference to the NWP intensity guidance products from ECMWF, JMA, NCEP, UKMO, CMA, NOAA HWRF, and AAMC-WRF of HKO and AI models. Factors such as rapid intensification chance deduced from statistical dynamical TC intensity forecast model, and environmental parameters such as sea surface temperature, wind shear, the ocean heat potential and land interactions are also considered in formulating the intensity forecast. |

###### 3  NWP Systems in Operational Use

| System | Domain | Horizontal Resolution | Number of Vertical Levels | Forecast Range (Initial time) | Number of Ensemble Members | Run by (own/other centers) |
| --- | --- | --- | --- | --- | --- | --- |
| ECMWF deterministic global model | Global | 0.1° | - | 240 hours (00, 12 UTC) 144 hours (06, 18 UTC) | N. A. | Other |
| ECMWF global EPS | Global | Only TC track and intensity data | - | 240 hours (00, 12 UTC) 144 hours (06, 18 UTC) | 51 | Other |
| ECMWF global EPS | Global | Ensemble member forecasts on surface and isobaric levels with horizontal resolution down to 0.1° covering selected domains | - | 360 hours (00, 12 UTC) 144 hours (06, 18 UTC) | 51 | Other |
| NCEP deterministic global model | Global | 0.25° | - | 384 hours (00, 06, 12, 18 UTC) | N.A. | Other |
| NCEP global EPS | Global | Only TC track and intensity data | - | 384 hours (00, 06, 12, 18 UTC) | 31 | Other |
| NCEP hurricane model (HAFS-A) | Domain based on the initial position of the TC | Only TC track and intensity data | - | Up to 126 hours (00, 06, 12, 18 UTC) | N.A. | Other |
| JMA deterministic global model (GSM) | Global | 0.25° | - | 264 hours (00, 12 UTC) 132 hours (06, 18 UTC) | N.A. | Other |
| JMA global EPS (GEPS) | Global | Only TC track and intensity data | - | 264 hours (00, 12 UTC) 132 hours (06, 18 UTC) | 51 | Other |
| UKMO deterministic global model | Global | Lon: 0.14° Lat: 0.09° | - | 168 hours (00, 12 UTC) 66 hours (06, 18 UTC) | N.A. | Other |
| UKMO global EPS | Global | Only TC track and intensity data | - | 192 hours (00, 06, 12, 18 UTC) | 36 | Other |
| CMA deterministic global model (GRAPES-GFS) | Global | 0.125° | - | 240 hours (00, 12 UTC) | N.A. | Other |
| CMA deterministic regional model (TRAMS) | 0.8°N-54.8°N 70°E-160°E | 0.09° | - | 168 hours (00, 12 UTC) 72 hours (06, 18 UTC) | N.A. | Other |
| CMC deterministic global model | Global | 0.15° | - | 240 hours (00, 12 UTC) | N.A. | Other |
| CMC global EPS | Global | Only TC track data | - | 240 hours (00, 12 UTC) | 21 | Other |
| DWD deterministic global model (ICON) | Global | 13 km | - | 180 hours (00, 12 UTC) | N.A. | Other |
| KMA deterministic global model (KIM) | Global | 0.35° | - | 288 hours (00, 12 UTC) | N.A. | Other |
| AAMC-WRF | NW: 60⁰N 45⁰E NE: 60⁰N 160⁰E SW: 20⁰S 45⁰E SE: 20⁰S 160⁰E | 10 km | 42 | 120 hours | N.A. | Own (Hon 2020) |
| ECMWF AIFS | Global | 0.25° | 13 | 360 hours | N.A. | Other |
| Fengwu | Global | 0.09° | 13 | 360 hours (00, 12 UTC) and 240 hours (06, 18 UTC) | N.A. | Own |
| Fuxi | Global | 0.25° | 13 | 360 hours (00, 12 UTC) and 240 hours (06, 18 UTC) | N.A. | Own |
| Graphcast | Global | 0.25° | 13 | 360 hours (00, 12 UTC) and 240 hours (06, 18 UTC) | N.A. | Own |
| Pangu-Weather | Global | 0.25° | 13 | 360 hours (00, 12 UTC) and 240 hours (06, 18 UTC) | N.A. | Own |

###### Reference

Chan P.W., Wu N.G., Zhang C.Z., Deng W.J., Hon K.K., 2018: The first complete dropsonde observation of a tropical cyclone over the South China Sea by the Hong Kong Observatory. Weather (2018), 10.1002/wea.3095

Dvorak V. F., 1984: Tropical cyclone intensity analysis using satellite data, NOAA Technical Report, 11.

ECMWF, 2021: IFS Documentation CY49R1, ECMWF. [Available at https://www.ecmwf.int/en/publications/ifs-documentation]

Hon, K.K., 2020 : Tropical Cyclone Track Prediction Using a Large-area WRF Model at the Hong Kong Observatory. Tropical Cyclone Research and Review, vol. 9, no. 1, 2020, pp. 67-74

Hong Kong Observatory, 2020 : WMO Technical Progress Report on the Global Data-processing and Forecasting System and Numerical Weather Prediction (NWP) Research. [Available at https://community.wmo.int/wmo-technical-progress-report-global-data-processing-and-forecasting-system-gdpfs-and-numerical-weather-prediction-nwp-research-2020].

Moldovan, G. et al., 2025: AIFS 1.1.0: An update to ECMWF's machine-learned weather forecast model AIFS, EGUsphere [preprint], https://doi.org/10.5194/egusphere-2025-4716.

Tam H.F., Choy C.W., Wong W.K., 2021: Development of objective forecast guidance on tropical cyclone rapid intensity change. Meteorological Applications, vol. 28, 2021, e1981

Zhang, C., Chan, P., Wu, N., Huang, Y. and Hon, K. (2020) The impact of dropsonde data on a numerical simulation of landfalling Typhoon Mangkhut. Meteorological Applications, 27(5), 1– 14.

Han, T., Gong, S., Ling, F., et al. FengWu-GHR: Learning the Kilometer-scale Medium-range Global Weather Forecasting. arXiv:2402.00059 (2024).

Chen, L., Zhong, X., Zhang, F. et al. FuXi: a cascade machine learning forecasting system for 15-day global weather forecast. npj Clim Atmos Sci 6, 190 (2023).

Remi Lam et al. Learning skillful medium-range global weather forecasting. Science 382,1416-1421(2023).

Bi, K., Xie, L., Zhang, H. et al. Accurate medium-range global weather forecasting with 3D neural networks. Nature 619, 533–538 (2023).

##### Name of the Member: [Japan]

###### 1 Tropical Cyclone Analysis

| Parameter | Time | Methods | Other Sources |
| --- | --- | --- | --- |
| Dvorak Intensity (CI, T, DT, PT, MET number) | 00, 06, 12, 18 UTC | Dvorak EIR method (Dvorak, 1984, Koba et al., 1991) and Early Dvorak Analysis (EDA: Tsuchiya et al., 2001 and Kishimoto, 2008) |  |
| Center Position, Accuracy of center position, Direction and speed of movement | 00, 03, 06, 09, 12, 15, 18, 21 UTC | Satellite images and other estimation methods which utilize surface observations |  |
| Central Pressure (CP), Maximum Sustained Wind speed (MSW), Maximum Gust Wind speed (MGW), 50 kt radii (R50), 30 kt radii (R30) | 00, 03, 06, 09, 12, 15, 18, 21 UTC | (1) Conversion from Dvorak method (Koba et al., 1991) (for CP, MSW and MGW) (2) Weather map analysis with full utilization of all observational data available (SYNOP, SHIP, BUOY, ASCAT, AMV including sea surface wind estimated from low-level AMV (Nonaka et al., 2019), etc.) (3) Statistical relationship between MSW and R50 selected by TC size | CPs estimated from TC warm core intensities observed by the Advanced Microwave Sounding Unit-A (AMSU-A) (Oyama, 2014) and MSW estimated from multi-channel microwave imager data (Sakuragi et al., 2014, Hoshino and Nakazawa, 2007) |

###### 2 Tropical Cyclone Forecasting

| Parameter | Issuance Time | Lead Time | Methods |
| --- | --- | --- | --- |
| Center position, Direction and speed of movement, Radius of probability circle (including for TDs expected to have TS intensity within 24 hours) | 00, 06, 12, 18 UTC | 24, 48, 72, 96, 120 hours | Center position, Direction and speed of movement: (1) Simple consensus method using deterministic Global NWP models of JMA and other major centers such as ECMWF, NCEP and UKMO (2) Global EPSs of JMA, ECMWF, NCEP and UKMO as reference (3) Deterministic regional NWP models of JMA and NCEP as reference Radius of probability circle: Verification results of past TC track errors according to the ensemble spread of the Global EPSs of JMA, ECMWF, NCEP and UKMO (Fukuda and Yamaguchi, 2019). |
| CP, MSW, MGW, R50 | 00, 06, 12, 18 UTC | 24, 48, 72, 96, 120 hours | CP, MSW and MGW: (1) Statistical and dynamical guidance (TIFS: Yamaguchi et al., 2018) (2) Deterministic Global NWP models of JMA and other major centers such as ECMWF, NCEP and UKMO as reference (3) Deterministic Regional NWP models of JMA and NCEP as reference R50: MSW-R50 development curve determined by TC size |

###### 3 NWP Systems in Operational Use

| System | Domain | Horizontal Resolution | Number of Vertical Levels | Forecast Range (Initial Time) | Number of Ensemble Members | Run by (own/other centers) |
| --- | --- | --- | --- | --- | --- | --- |
| JMA deterministic Global model (GSM) | Global | TQ959 (~13 km) | 128 | 132 hours (06, 18 UTC) 264 hours (00, 12 UTC) | - | Own |
| JMA Global EPS (GEPS) | Global | TQ479 (~27 km) | 128 | 132 hours (06, 18 UTC) 264 hours (00, 12 UTC) | 51 | Own |
| JMA deterministic regional model (Meso-scale Model: MSM) | Japan and its surrounding areas | 5 km (horizontal grids: 817 x 661) | 96 | 39 hours (03, 06, 09, 15, 18, 21 UTC) 78 hours (00, 12 UTC) | - | Own |
| JMA regional EPS | Japan and its surrounding areas | 5 km (horizontal grids: 817 x 661) | 96 | 39 hours (00, 06, 12, 18 UTC) | 21 | Own |
| JMA deterministic regional model (Local Forecast Model: LFM) | Japan and its surrounding areas | 2 km (horizontal grids: 1581 x 1301) | 76 | 18 hours (00, 03, 06, 09, 12, 15, 18, 21 UTC) 10 hours (hourly except for the above) | - | Own |
| ECMWF deterministic Global model | Global | 0.5° | - | 240 hours (00, 12 UTC) | - | Other |
| ECMWF Global EPS | Global | Only track data | - | 240 hours (00, 12 UTC) | 51 | Other |
| NCEP deterministic Global model | Global | 0.5° | - | 192 hours (00, 06, 12, 18 UTC) | - | Other |
| NCEP Global EPS | Global | 0.5° | - | 384 hours (00, 06, 12, 18 UTC) | 31 | Other |
| NCEP deterministic regional model (HWRF) | Domain based on the initial position of the TC | Only track and intensity data | - | Up to 126 hours (00, 06, 12, 18 UTC) | - | Other |
| UKMO deterministic Global model | Global | Lon: 0.83° Lat: 0.56° | - | 120 hours (00, 12 UTC) | - | Other |
| UKMO Global EPS | Global | Only track data | - | 168 hours (00, 06, 12, 18 UTC) | 36 | Other |

###### Reference

Dvorak V. F., 1984: Tropical cyclone intensity analysis using satellite data, NOAA Technical Report, 11.

Fukuda, J. and M. Yamaguchi, 2019: Determining Probability-Circle Radii of Tropical Cyclone Track Forecasts with Multiple Ensembles, RSMC Tokyo - Typhoon Center Technical Review, No. 21.

Hoshino, S. and T. Nakazawa, 2007: Estimation of Tropical Cyclone’s Intensity Using TRMM/TMI Brightness Temperature Data, Journal of the Meteorological Society of Japan, Vol. 85, No. 4, 437 - 454.

Japan Meteorological Agency: Outline of the Operational Numerical Weather Prediction at the Japan Meteorological Agency (http://www.jma.go.jp/jma/jma-eng/jma-center/nwp/outline-latest-nwp/index.htm).

Kishimoto, K., 2008: Revision of JMA’s Early Stage Dvorak Analysis and Its Use to Analyze Tropical Cyclones in the Early Developing Stage, RSMC Tokyo - Typhoon Center Technical Review, No. 10, 1 - 12.

Kishimoto, K., M. Sasaki, and M. Kunitsugu, 2013: Cloud Grid Information Objective Dvorak Analysis (CLOUD) at the RSMC Tokyo - Typhoon Center, RSMC Tokyo - Typhoon Center Technical Review, No. 15, 1 - 15.

Koba, H., T. Hagiwara, S. Osano and S. Akashi, 1991: Relationships between CI Number and Minimum Sea Level Pressure/Maximum Wind Speed of Tropical Cyclones, Geophysical Magazine, Vol. 44, No. 1, 15 - 25.

Nonaka, K., S. Nishimura and Y. Igarashi, 2019: Utilization of Estimated Sea Surface Wind Data Based on Himawari-8/9 Low-level AMVs for Tropical CycloneAnalysis, RSMC Tokyo - Typhoon Center Technical Review, No. 21.

Oyama, R., 2014: Algorithm and Validation of a Tropical Cyclone Central Pressure Estimation Method Based on Warm Core Intensity as Observed Using the Advanced Microwave Sounding Unit-A (AMSU-A), RSMC Tokyo - Typhoon Center Technical Review, No. 16.

Sakuragi, T., S. Hoshino, N. Kitabatake, 2014: Development and Verification of a Tropical Cyclone Intensity Estimation Method Reflecting the Variety of TRMM/TMI Brightness Temperature Distribution, RSMC Tokyo - Typhoon Center Technical Review, No. 16.

Tsuchiya, A., T. Mikawa and A. Kikuchi, 2001: Method of Distinguishing between Early Stage Cloud Systems that Develop into Tropical Storms and Ones that Do Not, Geophysical Magazine Series 2, Vol. 4, Nos. 1 - 4, 49 - 59.

Ujiie, M., M. Higuchi, T. Kadowaki, Y. Kuroki, K. Miyaoka, M. Oda, K. Ochi, R. Sekiguchi, H. Shimizu, S. Yokota, and H. Yonehara, 2021: Upgrade of JMA’s Operational Global NWP system. WGNE. Res. Activ. Earth Sys. Modell., 51. 6.09-6.10.

Yamaguchi, H., M.Ikegami, T.Iwahira, K.Ochi, R.Sekiguchi and T.Takakura, 2021: Upgrade of JMA's Global Ensemble Prediction System. WGNE. Res. Activ. Earth Sys. Modell., 51. 6.13-6.14.

Yamaguchi, M., H. Owada, U. Shimada, M. Sawada, T. Iriguchi, K. D. Musgrave and M. DeMaria, 2018: Tropical Cyclone Intensity Prediction in the Western North Pacific Basin Using SHIPS and JMA/GSM., Scientific Online Letters on the Atmosphere, Vol. 14, 138 - 143, doi:10.2151/sola.2018-024.

##### Name of the Member: [Macao, China]

###### 1 Tropical Cyclone Analysis

| Parameter | Time | Methods | Other Sources |
| --- | --- | --- | --- |
| Center Position, Direction and speed of movement | 00, 06, 12, 18 UTC (increase to an hourly basis when TC enters the 800 km alert zone of Macao) | - Satellite imagery - Radar observations - Surface observation from synoptic stations, automatic weather stations, oil drills over the South China Sea | (a) ASCAT observations (b) NRL Microwave images (c) NOAA Multiplatform Tropical Cyclone Surface Winds Analysis (d) NOAA Synthetic Aperture Radar Wind Products |
| Intensity |  | Dvorak EIR method (Dvorak, 1984) |  |
| Central Pressure (CP), Maximum Sustained Wind speed (MSW), Wind radii (strong wind and gale wind or above) |  | (1) Conversion from Dvorak method (Koba et al., 1991) (for CP and MSW ) (2) Weather map analysis with full utilization of all observational data available (SYNOP, BUOY, oil drills) |  |

###### 2 Tropical Cyclone Forecasting

| Parameter | Issuance Time | Lead Time | Methods |
| --- | --- | --- | --- |
| Center position, Track and Intensity | 00, 06, 12, 18 UTC (increase to an hourly basis when TC enters the 800 km alert zone of Macao) | 24, 48, 72, 96, 120 hours | (1) Simple consensus method using deterministic NWP models of CMA, ECMWF, JMA, NCEP and UKMO (2) Global EPSs of ECMWF, JMA and NCEP as reference (3) Consider all environmental factors such as wind shear, sea surface temperature and upper/lower level divergence/convergence, which are favourable for intensification of TCs. |

###### 3 NWP Systems in Operational Use

| System | Domain | Horizontal Resolution | Number of Vertical Levels | Forecast Range (Initial Time) | Number of Ensemble Members | Run by (own/other centers) |
| --- | --- | --- | --- | --- | --- | --- |
| JMA deterministic Global model (GSM) | Global | TL959 (~50 km) | 11 | 84 hours (00, 06, 18 UTC) 264 hours (12 UTC) | - | Other |
| ECMWF deterministic Global model | Global | 0.25° | - | 240 hours (00, 12 UTC) | - | Other |
| ECMWF Global EPS | Global | Only track data | - | 240 hours (00, 12 UTC) | 50 members 1 high resolution 1 control | Other |
| NCEP deterministic Global model | Global | 0.25° | - | 240 hours (00, 06, 12, 18 UTC) | - | Other |
| NCEP Global EPS | Global | Only track data | - | - | - | Other |
| UKMO deterministic Global model | Global | Lon: 0.23° Lat: 0.16° | - | 144 hours (00, 12 UTC) | - | Other |
| CMA deterministic regional model (TRAMS) | 0.8°N-54.8°N 70°E-160°E | 0.09° | - | 168 hours (00, 12 UTC) 72 hours (06, 18 UTC) | - | Other |

##### Name of the Member: [Malaysia]

###### 1 Tropical Cyclone Analysis

| Parameter | Time | Methods | Other Sources |
| --- | --- | --- | --- |
| Center Position, Accuracy of center position, Direction & speed of movement | 00, 03, 06, 09, 12, 15, 18, 21 UTC | Satellite images, NWP products and other estimation methods which utilize surface observations | TC information such as central pressure, maximum sustained wind speed, maximum gust wind speed, R50 (50 kt radii), R30 (30 kt radii) from RSMC Tokyo |

###### 2 Tropical Cyclone Forecasting

| Parameter | Issuance Time | Lead Time | Methods |
| --- | --- | --- | --- |
| Track (center position, distance from nearest town, direction and speed of movement) | 01, 04, 07, 10, 13, 16, 19, 22 UTC | 6, 12, 18, 24 hours | Track (center position, distance from nearest town, direction and speed of movement) refer to RSMC-Tokyo, Japan for South China Sea and Northwest Pacific region, and RSMC-New Delhi, India for Bay of Bengal region. |

###### 3 NWP Systems in Operational Use

| System | Domain | Horizontal Resolution | Number of Vertical Levels | Forecast Range (Initial Time) | Number of Ensemble Members | Run by (own/other centers) |
| --- | --- | --- | --- | --- | --- | --- |
| Mesoscale Deterministic Model: WRF driven by NCEP GFS model | [9 km]: Lon: 82.27 - 135.18°E Lat: 5.60°S - 31.34°N [3 km]: Lon: 97.15 - 121.03°E Lat: 3.45°S - 8.39°N [1 km]: Lon: 99.55 - 119.39 °E Lat: 0.53 – 7.44 °N | [9 km]: 655 x 479 [3 km]: 886 x 442 [1 km]: 2197 x 772 | 51 | 168 hours (00, 06, 12, 18UTC) | - | Own |
| Mesoscale Deterministic Model: WRF driven by UKMO model | [9 km]: Lon: 82.27 - 135.18°E Lat: 5.60°S - 31.34°N [3 km]: Lon: 97.15 - 121.03°E Lat: 3.45°S - 8.39°N [1 km]: Lon: 99.55 -119.39°E Lat: 0.53 -7.44 °N | [9 km]: 655 x 479 [3 km]: 886 x 442 [1 km]: 2197 x 772 | 51 | 60 (00, 06, 12, 18UTC) | - | Own |
| ECMWF deterministic Global model | Global | 0.5° | - | 240 hours (00, 12 UTC) | - | Other |
| NCEP deterministic Global model | Global | 0.5° | - | 192 hours (00, 06, 12, 18 UTC) | - | Other |
| JMA deterministic Global model (GSM) | Global | TL959 (~20 km) | 100 | 132 hours (00, 06, 18 UTC) 264 hours (12 UTC) | - | Other |

##### Name of the Member: [Philippines]

###### 1 Tropical Cyclone Analysis

| Parameter | Time | Methods | Other Sources |
| --- | --- | --- | --- |
| Dvorak Intensity (DT, PT, MET, FT and CI numbers) | 00, 06, 12, 18 UTC | - Dvorak EIR method (Dvorak 1984) - Early-stage Dvorak Analysis (Tsuchiya et al. 2001; Kishimoto 2008) | - Satellite fix bulletins from other NMSs via Numerical Typhoon Prediction website or GTS - CIMSS Advanced Dvorak Technique (ADT) (Olander and Velden 2007) |
| Center Position, Movement Speed and Direction | 00, 06, 12, 18 UTC | - Estimation of low-level circulation center using a combination of satellite images, weather radar scans, and surface observations (SYNOP, SHIP, BUOY). | - Satellite fix bulletins from other NMSs via Numerical Typhoon Prediction website or GTS - CIMSS Automated Rotational Center Hurricane Eye Retrieval (ARCHER) (Wimmers and Velden 2010) |
| Central Pressure (PRES), Maximum Sustained Winds (MXWD), Maximum Gust (GUST), | 00,06,12, 18 UTC | - Conversion of CI number to PRES and MXWD (Koba et al. 1991) - Conversion of MXWD to GUST (Harper et al. 2010) - Weather map analysis using all available observation data (SYNOP, SHIP, BUOY, ASCAT) | - PRES estimates from warm core intensity observations of Advanced Microwave Sounding Unit (AMSU) (Oyama 2014) via Numerical Typhoon Prediction website - Weighted average of PRES analyses from Dvorak, AMSU and ATMS (Oyama et al. 2016) via Numerical Typhoon Prediction website - CIMSS Satellite Consensus (SATCON) (Herndon and Velden 2018) |
| Radius of strong, gale-force, storm-force, and typhoon-force wind areas | 00, 06, 12, 18 UTC | - Weather map analysis using all available observation data (SYNOP, SHIP, BUOY, ASCAT) | - Sea surface winds estimated from low-level AMV (Nonaka et al. 2019) via Numerical Typhoon Prediction website - NOAA/NESDIS Multiplatform Tropical Cyclone Surface Winds Analysis (MTCSWA) (Knaff and DeMaria 2010) - CIMSS real-time wind radii estimates based on Knaff et al. (2016) |

###### 2 Tropical Cyclone Forecasting

| Parameter | Issuance Time | Lead Time | Methods |
| --- | --- | --- | --- |
| PSTN, MOVE | 00, 06, 12, 18 UTC | 12, 24, 36, 48, 60, 72, 96, 120 hours | - Simple and selective (subjective) consensus method using global deterministic and EPS models of major centers via Numerical Typhoon Prediction website - Global EPSs of NCEP, ECMWF, JMA and UKMO via Numerical Typhoon Prediction website as reference - Regional deterministic NWP models of PAGASA, NCEP, and HKO as reference - Analysis of environmental steering using actual 00 and 12 UTC upper-air charts (single layer approach) and CIMSS satellite AMV-derived deep-layer mean streamlines (Velden and Leslie 1991; Velden 1993) |
| Central Pressure (PRES), Maximum Sustained Winds (MXWD), Category (i.e., TD, TS, STS, TY, STY, LOW, XT) | 00, 06, 12, 18 UTC | 12, 24, 36, 48, 60, 72, 96, 120 hours | - Bias-corrected intensity prediction using weighted analog technique (Tsai and Elsberry 2014) - Logistic Growth Equation Model (LGEM; DeMaria 2009) - Typhoon Intensity Forecast Scheme based on SHIPS (TIFS; Yamaguchi et al. 2018; Ono et al. 2019) - Global deterministic models from major NWP centers via GTS as reference - Regional deterministic NWP models of PAGASA, NCEP, and HKO as reference |
| Radius of 70% Probability Circle | 00, 06, 12, 18 UTC | 24, 48, 72, 96, 120 hours | - Based on the direct positional error corresponding to cumulative ratio of 70% over the last 5 typhoon seasons. |

Note: Analyses and forecasts are made at 03, 09, 15, and 21 UTC for tropical cyclones that are landfalling or passing within 60 nmi of Philippine coastline. These additional analyses and forecasts commence within 24 hours of landfall or close approach and terminates once the cyclone leaves the 60-nmi coastal buffer.

###### 3 NWP Systems in Operational Use

| System | Domain | Horizontal Resolution | Number of Vertical Level | Forecast Range (Initial Time) | Number of Ensemble Members | Run by (own/other centers) |
| --- | --- | --- | --- | --- | --- | --- |
| PAGASA Regional Deterministic Model (WRF-ARW) | 3°N-25°N 115°E-135°E | 12 km (182 x 214 grids) | 42 | 144 hours (00, 03, 06, 09, 12, 15, 18, 21 UTC) | - | Own |
| PAGASA Regional Deterministic Model (WRF-ARW) | 5°N-21°N 116°E-127°E | 3 km (361 x 593 grids) | 42 | 48 hours (00, 03, 06, 09, 12, 15, 18, 21 UTC) | - | Own |
| JMA Global Deterministic Model (GSM) | Global | Track and intensity data only | - | 132 hours (00, 06, 18 UTC) 264 hours (12 UTC) | - | Other |
| NCEP Global Deterministic Model | Global | Track data only | - | 192 hours (00, 06, 12, 18 UTC) | - | Other |
| ECMWF Global Deterministic Model | Global | Track data only | - | 240 hours (00, 12 UTC) | - | Other |
| UKMO Global Deterministic Model | Global | Track and intensity data only | - | 120 hours (00, 12 UTC) | - | Other |
| KMA Global Deterministic Model | Global | Track and intensity data only | - | 168 hours (00, 12 hours) | - | Other |
| BoM Global Deterministic Model | Global | Track and intensity data only | - | 240 hours (00, 12 UTC) | - | Other |
| CMC Global Deterministic Model | Global | Track data only | - | 144 hours (00, 12 UTC) | - | Other |
| CMA Global Deterministic Model | Global | Track data only | - | 120 hours (00, 12 UTC) | - | Other |
| DWD Global Deterministic Model | Global | Track data only | - | 84 hours (00, 12 UTC) | - | Other |
| JMA Global EPS (GEPS) | Global | Track and intensity data only | - | 132 hours (06, 18 UTC) 264 hours (00, 12 UTC) | 51 | Other |
| NCEP Global EPS | Global | Track data only | - | 384 hours (00, 06, 12, 18 UTC) | 31 | Other |
| ECMWF Global EPS | Global | Track data only | - | 240 hours (00, 12 UTC) | 51 | Other |
| UKMO Global EPS | Global | Track data only | - | 168 hours (00, 06, 12, 18 UTC) | 36 | Other |
| NCEP Regional Deterministic Model (HWRF) | Based on the initial position of the TC | Track and intensity data only | - | 126 hours (00, 06, 12, 18 UTC) | - | Other |
| HKO Regional Deterministic Model (NHM) | 8°N-46.5°N 85°E-148°E | Track and intensity data only | - | 72 hours (00, 06, 12, 18 UTC) | - | Other |

Note: Apart from HWRF, deterministic and EPS model forecasts from other centers are made available via the JMA Numerical Typhoon Prediction website (https://tynwp-web.kishou.go.jp/) or the WMO Global Telecommunication System (GTS). HWRF forecasts are available via the HWRF Forecast Guidance website (https://www.emc.ncep.noaa.gov/gc_wmb/vxt/HWRF/index.php)

###### References

DeMaria, M., 2009: A Simplified Dynamical System for Tropical Cyclone Intensity Prediction. Mon. Wea. Rev., 137, 68–82, ttps://doi.org/10.1175/2008MWR2513.1

Dvorak, V., 1984: Tropical cyclone intensity analysis using satellite data. NOAA Tech. Rep. NESDIS, 11, 47 pp.

Harper, B.A., J.D. Kepert, and J.D. Ginger, 2010: Guidelines for Converting Between Various Wind Averaging Periods in Tropical Cyclone Conditions. WMO Tech. Doc. TD-No. 1555, 54 pp.

Herndon, D. and C.S. Velden, 2018: An Update on the SATellite CONsensus (SATCON) Algorithm for Estimating Tropical Cyclone Intensity. 33rd Conf. Hurr. Trop. Meteor. Ponte Vedra, FL, Amer. Meteor. Soc., 284, https://ams.confex.com/ams/33HURRICANE/webprogram/Paper340235.html.

Kishimoto, K., 2008: Revision of JMA’s Early Stage Dvorak Analysis and Its Use to Analyze Tropical Cyclones in the Early Developing Stage. RSMC Tokyo - Typhoon Center Technical Review No. 10, 12 pp.

Knaff, J.A. and M. DeMaria, 2010: NOAA/NESDIS Multiplatform Tropical Cyclone Surface Wind Analysis. User Manual, 25 pp.

Knaff, J.A., C.J. Slocum, K.D. Musgrave, C.R. Sampson, and B.R. Strahl, 2016: Using Routinely Available Information to Estimate Tropical Cyclone Wind Structure. Mon. Wea. Rev., 144, 1233–1247, https://doi.org/10.1175/MWR-D-15-0267.1

Koba, H., T. Hagiwara, S. Osano and S. Akashi, 1991: Relationships between CI Number and Minimum Sea Level Pressure/Maximum Wind Speed of Tropical Cyclones. Geophysical Magazine, 44, 15 - 25.

Olander, T.L. and C.S. Velden, 2007: The Advanced Dvorak Technique: Continued Development of an Objective Scheme to Estimate Tropical Cyclone Intensity Using Geostationary Infrared Satellite Imagery. Wea. Forecasting, 22, 287–298, https://doi.org/10.1175/WAF975.1

Ono, M., S. Notshuhara, J. Fukuda, Y. Igarashi, and K. Bessho, 2019: Operational Use of the Typhoon Intensity Forecasting Scheme Based on SHIPS (TIFS) and Commencement of Five-day Tropical Cyclone Intensity Forecasts. RSMC Tokyo - Typhoon Center Technical Review No. 21, 17 pp.

Tsai, H.C. and R.L. Elsberry, 2014: Applications of situation-dependent intensity and intensity spread predictions based on a weighted analog technique. Asia-Pacific J. Atmos. Sci., 50, 507–518, https://doi.org/10.1007/s13143-014-0040-7

Velden, C.S. and L.M. Leslie, 1991: The Basic Relationship between Tropical Cyclone Intensity and the Depth of the Environmental Steering Layer in the Australian Region. Wea. Forecasting, 6, 244–253, https://doi.org/10.1175/1520-0434(1991)006<0244:TBRBTC>2.0.CO;2

Velden, C.S., 1993: The relationship between tropical cyclone motion, intensity and the vertical extent of the environmental steering layer in the Atlantic basin. 20th Conf. Hurr. Trop. Meteor., San Antonio, TX., Amer. Meteor. Soc.

Tsuchiya, A., T. Mikawa and A. Kikuchi, 2001: Method of Distinguishing between Early Stage Cloud Systems that Develop into Tropical Storms and Ones that Do Not. Geophysical Magazine Series 2, 4, 49 - 59.

Wimmers, A.J. and C.S. Velden, 2010: Objectively Determining the Rotational Center of Tropical Cyclones in Passive Microwave Satellite Imagery. J. Appl. Meteor. Climatol., 49, 2013-2034, https://doi.org/10.1175/2010JAMC2490.1

Yamaguchi, M., H. Owada, U. Shimada, M. Sawada, T. Iriguchi, K.D. Musgrave, and M. DeMaria, 2018: Tropical Cyclone Intensity Prediction in the Western North Pacific Basin Using SHIPS and JMA/GSM. SOLA, 14, 138-143, https://doi.org/10.2151/sola.2018-024

##### Name of the Member: [Republic of Korea]

###### 1 Tropical Cyclone Analysis

| Parameter | Time | Methods | Other Sources |
| --- | --- | --- | --- |
| Center Position, Central pressure, Maximum sustained wind speed, Direction and speed of movement, 15 m/s radii, 25 m/s radii | 00, 06, 12, 18 UTC | (1) Satellite images (ASCAT, OSCAT etc) and other estimation methods which utilize surface observations (SYNOP, SHIP, BUOY, AWS, Radar) (2) Dvorak technique for center pressure and maximum sustained wind speed estimates (Dvorak, 1984, Koba et al., 1991) |  |

###### 2 Tropical Cyclone Forecasting

| Parameter | Issuance Time | Lead Time | Methods |
| --- | --- | --- | --- |
| Likelihood of development of organized convective cloud systems into TSs | 00, 06, 12, 18 UTC | 24, 48, 72, 96, 120 hours | - (1) EDA - (2) Synoptic analysis covering all observations - (3) Satellite imaginary - (4) Consensus method using deterministic Global NWP model (GDAPS, ECMWF, NECP, JMA etc.) - (5) 850 hPa and 200 hPa streamlines and steering flow, vertical wind shear of NWP models as reference |
| Center position, Direction and speed of movement, Radius of probability circle, Central pressure , Maximum sustained wind speed, 15 m/s radii, 25 m/s radii | 00, 06, 12, 18 UTC | 12, 24, 36, 48, 72, 96, 120 hours | Center position, direction and speed of movement : (1) Analysis of changes of circulation, organization of TC based on observations (SYNOP, SHIP, BUOY, AWS, Radar, Satellite) (2) Analysis of weather maps and comparison with NWP (3) Consensus method using KMA Global Data Assimilation and Prediction System (GDAPS) and other deterministic Global model (ECMWF, JMA, NCEP, etc.) (4) Global EPS (EPSG, ECMWF, JMA, NCEP, etc.) , AI based model(KMA: 9 types) and AIFS as reference Central pressure, maximum sustained wind speed: (1) Conversion with Dvorak technique and analysis of Satellite imaginary and observations(SYNOP, SHIP, BUOY, AWS) (2) KMA Global Data Assimilation and Prediction System (GDAPS) , other deterministic Global model (ECMWF, JMA, NCEP, etc.), Global EPS (EPSG, ECMWF, JMA, NCEP, etc.), NCEP deterministic regional model (HWRF), Statistical guidance(SPIKE) and AI based model (3) Analysis of Sea Surface Temperature and Ocean heat content (4) Wind shear of NWP model as reference Radius of probability circle: Expected TC locations with a probability of 70% at each lead time. Statistically measured by averaging forecast track errors for the latest 3 years |

###### 3 NWP Systems in Operational Use

| System | Domain | Horizontal Resolution | Number of Vertical Levels | Forecast Range (Initial Time) | Number of Ensemble Members | Run by (own/other centers) |
| --- | --- | --- | --- | --- | --- | --- |
| KMA Global Data Assimilation and Prediction System (GDAPS) | Global | 8 km | 91 | 372 hours (00,12 UTC) 87 hours (06, 18 UTC) | - | Own |
| KMA Global EPS (EPSG) | Global | 24 km | 91 | 372 hours (00,12 UTC) | 26 | Own |
| ECMWF deterministic Global model | Global | 0.1° | 137 | 360 hours (00, 12 UTC) | - | Other |
| ECMWF Global EPS | Global | 0.25° | 137 | 360 hours (00, 12 UTC) | 51 | Other |
| JMA deterministic Global model (GSM) | Global | Track and intensity only | - | 132 hours (00, 06, 18 UTC) 264 hours (12 UTC) | - | Other |
| JMA Global EPS (GEPS) | Global | Track and intensity only | - | 132 hours (06, 18 UTC) 264 hours (00, 12 UTC) | 51 | Other |
| NCEP deterministic Global model (GFS) | Global | Track and intensity only | - | 168 hours (00, 06, 12, 18 UTC) | - | Other |
| NCEP Global EPS | Global | Track and intensity only | - | 240 hours (00, 06, 12, 18 UTC) | 80 | Other |
| NCEP deterministic regional model (HWRF) | Regional | Track and intensity only | - | Up to 126 hours (00, 06, 12, 18 UTC) | - | Other |
| NCEP HAFS (Hurricane Analysis and Forecast System) | Regional | Track and intensity only | - | Up to 126 hours (00, 06, 12, 18 UTC) | - | Other |
| Navy Global Environmental Model (NavGEM) | Global | Track and intensity only |  | 144 hours (00,12 UTC) |  | Other |
| ECMWF Artificial Intelligence Forecasting System(AIFS) | Global | Track and intensity only |  | 360 hours (00, 06, 12, 18 UTC) |  | Other |

Name of the Member: [Thailand]

###### 1 Tropical Cyclone Analysis

| Parameter | Time | Methods | Other Sources |
| --- | --- | --- | --- |
| Dvorak Intensity (CI) | 00, 06, 12, 18 UTC | Devorak Technique (Dvorak, 1984) And SATIAD Program from JMA for analysis | Satellite observational data from Himawari-8/9 from Japan and FY4A from China |
| Center Position, Accuracy of center position, Direction and speed of movement | 00, 03, 06, 09, 12, 15, 18, 21 UTC and hourly from synoptic observation and AWS | Satellite images and Synoptic charts and other estimation methods which utilize surface observations | Observation and Weather Radar |

###### 2 Tropical Cyclone Forecasting

| Parameter | Issuance Time | Lead Time | Methods | Other Sources |
| --- | --- | --- | --- | --- |
| Center position, Direction and speed of movement, Radius of probability circle | 00, 06, 12, 18 UTC | 12, 24, 36, 48, 72, 96, 120 hours | Reference : RSMC TOKYO (Japan) in Pacific and South China sea region Reference : RSMC New Delhi (India) in Arabian and Andaman sea region - NWP products from TMD-HPC output | Center position, Direction and speed of movement: Simple consensus method using deterministic Global NWP models such as ECMWF, NCEP and JMA KMA and BOM (Australia) |

### APPENDIX 4-A

32

#### CLASSIFICATIONS OF TROPICAL CYCLONES IN THE WESTERN NORTH PACIFIC INTERNALLY USED BY MEMBERS

| Maximum sustained winds (knots) | ≤33 | 34 - 47 | 48 - 63 | ≥ 64 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Typhoon Committee (10 min) | Tropical Depression (TD) | Tropical Storm (TS) | Severe Tropical Storm (STS) | Typhoon (TY) |  |  |  |
| China (2 min) | TD | TS | STS | 64 - 80 TY | 81 - 99 Severe Typhoon (STY) | ≥ 100 Super Typhoon (Super TY) |  |
| Hong Kong, China (10 min) | TD* | TS | STS | 64 - 80 TY | 81 - 99 Severe Typhoon (ST) | ≥ 100 Super Typhoon (Super T) |  |
| Japan (10 min) | TD | TS | STS | 64 - 84 TY | 85 - 104 Very Strong TY | ≥ 105 Violent TY |  |
| Macao, China (10 min) | TD | TS | STS | 64 - 80 TY | 81 - 99 Severe Typhoon (ST) | ≥ 100 Super Typhoon (Super T) |  |
| Philippines (10 min) | TD | TS | STS | 64 - 99 TY |  | ≥ 100 Super TY |  |
| Republic of Korea (10min) | TD | TS | STS | 64~84 TY | 85~104 Very Strong TY | ≥ 105 Super TY |  |
| U.S.A. (1 min) | TD | TS |  | 64 - 129 TY |  |  | ≥ 130 Super TY |

*For Hong Kong, China, a Tropical Depression has maximum sustained winds of 22 – 33 knots.

### APPENDIX 4-B

35

#### EXAMPLES OF ADVISORIES ISSUED FROM RSMC TOKYO - TYPHOON CENTER

##### RSMC Guidance for Forecast by GSM

FXPQ20 RJTD 231200

RSMC GUIDANCE FOR FORECAST

NAME TY 1826 YUTU (1826)

PSTN 231200UTC 12.0N 149.6E

PRES  965HPA

MXWD  75KT

FORECAST BY GLOBAL MODEL

TIME     PSTN       PRES    MXWD

(CHANGE FROM T=0)

T=006 12.8N 149.0E -007HPA +007KT

T=012 13.5N 148.4E -012HPA +015KT

T=018 14.0N 147.5E -016HPA +011KT

T=024 14.5N 146.7E -018HPA +017KT

T=030 15.2N 145.8E -025HPA +023KT

T=036 15.7N 144.9E -025HPA +027KT

T=042 16.2N 144.0E -032HPA +028KT

T=048 16.3N 143.2E -032HPA +031KT

T=054 16.6N 142.4E -037HPA +035KT

T=060 16.7N 141.4E -035HPA +033KT

T=066 16.7N 140.3E -041HPA +033KT

T=072 16.8N 139.0E -039HPA +037KT

T=078 16.9N 137.7E -041HPA +035KT

T=084 16.9N 136.2E -040HPA +033KT

T=090 17.0N 135.0E -045HPA +036KT

T=096 17.0N 133.9E -043HPA +038KT

T=102 17.0N 132.8E -045HPA +038KT

T=108 16.8N 131.8E -047HPA +038KT

T=114 16.6N 130.9E -053HPA +041KT

T=120 16.5N 130.1E -054HPA +042KT

T=126 16.4N 129.2E -055HPA +042KT

T=132 16.4N 128.5E -051HPA +038KT=

##### RSMC Guidance for Forecast by GEPS

FXPQ30 RJTD 231200

RSMC GUIDANCE FOR FORECAST

NAME TY 1826 YUTU (1826)

PSTN 231200UTC 12.0N 149.6E

PRES  965HPA

MXWD  75KT

FORECAST BY GLOBAL ENSEMBLE PREDICTION SYSTEM

TIME     PSTN       PRES    MXWD

(CHANGE FROM T=0)

T=006 12.7N 149.1E -002HPA +001KT

T=012 13.2N 148.3E -001HPA +004KT

T=018 13.8N 147.6E -005HPA +004KT

T=024 14.3N 146.7E -005HPA +006KT

T=030 14.9N 145.9E -009HPA +009KT

T=036 15.4N 145.0E -009HPA +010KT

T=042 15.8N 144.2E -013HPA +010KT

T=048 16.1N 143.5E -012HPA +011KT

T=054 16.3N 142.7E -015HPA +012KT

T=060 16.5N 141.9E -014HPA +013KT

T=066 16.7N 141.0E -018HPA +017KT

T=072 16.9N 139.8E -017HPA +018KT

T=078 17.2N 138.6E -020HPA +018KT

T=084 17.4N 137.3E -020HPA +021KT

T=090 17.7N 136.0E -024HPA +021KT

T=096 17.8N 134.9E -023HPA +021KT

T=102 17.9N 133.9E -027HPA +023KT

T=108 17.9N 132.9E -026HPA +026KT

T=114 18.0N 132.1E -031HPA +028KT

T=120 17.9N 131.3E -031HPA +030KT

T=126 17.9N 130.6E -034HPA +030KT

T=132 18.0N 129.9E -033HPA +030KT=

##### RSMC Prognostic Reasoning

WTPQ30 RJTD 231200

RSMC TROPICAL CYCLONE PROGNOSTIC REASONING

REASONING NO.10 FOR TY 1826 YUTU (1826)

1.GENERAL COMMENTS

TY YUTU IS LOCATED AT 12.0N, 149.6E. INFORMATION ON THE CURRENT POSITION IS BASED ON ANIMATED MSI. POSITIONAL ACCURACY IS GOOD. THE SYSTEM IS IN A FAVORABLE ENVIRONMENT FOR DEVELOPMENT UNDER THE INFLUENCE OF HIGH SSTS, HIGH TCHP AND WEAK VWS. THIS HAS CAUSED THE SYSTEM TO DEVELOP OVER THE LAST SIX HOURS. HOWEVER, THE INFLUENCE OF DRY AIR IS UNFAVORABLE FOR SYSTEM DEVELOPMENT. INFORMATION ON CURRENT INTENSITY IS BASED ON DVORAK INTENSITY ANALYSES.

2.SYNOPTIC SITUATION

THE SYSTEM IS MOVING WESTWARD ALONG THE SOUTHERN PERIPHERY OF A MID-LEVEL SUB-TROPICAL HIGH. ANIMATED MSI SHOWS THE APPEARANCE OF AN EYE. WATER VAPOR IMAGERY SHOWS DRY AIR IN THE DIRECTION OF THE MOVEMENT. DMSP-F18/SSMIS 89 GHZ MICROWAVE IMAGERY SHOWS THE SYSTEM HAS A BAND WITH CURVATURE INDICATING THE CSC.

3.TRACK FORECAST

THE SYSTEM WILL MOVE NORTHWESTWARD ALONG THE PERIPHERY OF A MID-LEVEL SUB-TROPICAL HIGH UNTIL FT12. THE SYSTEM WILL THEN MOVE WEST-NORTHWESTWARD ALONG THE PERIPHERY OF A MID-LEVEL SUB-TROPICAL HIGH UNTIL FT120. THE JMA TRACK FORECAST IS BASED ON GSM PREDICTIONS, AND REFERENCE TO OTHER NWP MODELS. JMA TRACK FORECAST CONFIDENCE IS FAIR UNTIL FT48 BUT LOW THEREAFTER DUE TO SIGNIFICANT DIFFERENCES AMONG NUMERICAL MODEL OUTPUTS.

4.INTENSITY FORECAST

THE SYSTEM WILL DEVELOP UNTIL FT48 DUE TO THE INFLUENCE OF INTERACTION WITH HIGH SSTS, HIGH TCHP, WEAK VWS AND GOOD UPPER LEVEL OUTFLOW. THE SYSTEM WILL THEN MAINTAIN ITS INTENSITY UNTIL FT72 DUE TO THE INFLUENCE OF INTERACTION WITH HIGH SSTS, HIGH TCHP AND DRY AIR. THE JMA INTENSITY FORECAST IS BASED ON GUIDANCE DATA. =

##### RSMC Tropical Cyclone Advisory for Five-day Forecast

WTPQ50 RJTD 231200

RSMC TROPICAL CYCLONE ADVISORY

NAME  TY 1826 YUTU (1826)

ANALYSIS

PSTN  231200UTC 12.0N 149.6E GOOD

MOVE  W 11KT

PRES  965HPA

MXWD  075KT

GUST  105KT

50KT  60NM

30KT  270NM NORTHEAST 210NM SOUTHWEST

FORECAST

24HF  241200UTC 14.4N 146.2E 50NM 70%

MOVE  WNW 10KT

PRES  925HPA

MXWD  100KT

GUST  140KT

48HF  251200UTC 16.2N 143.2E 95NM 70%

MOVE  WNW 09KT

PRES  915HPA

MXWD  105KT

GUST  150KT

72HF  261200UTC 17.4N 139.8E 130NM 70%

MOVE  WNW 09KT

PRES  915HPA

MXWD  105KT

GUST  150KT

96HF  271200UTC 18.7N 135.6E 240NM 70%

MOVE  WNW 11KT

PRES  935HPA

MXWD  95KT

GUST  135KT

120HF 281200UTC 19.6N 132.6E 375NM 70%

MOVE  WNW 07KT

PRES  935HPA

MXWD  90KT

GUST  130KT =

### APPENDIX 4-C

36

#### STATIONS BROADCASTING CYCLONE WARNINGS FOR SHIPS ON THE HIGH SEAS

| Station |  | Call sign of coastal radio station | Area covered |
| --- | --- | --- | --- |
| Member | Station |  |  |
| China | Shanghai | XSG | Bohai Sea, Huanghai Sea, Donghai Sea, Shanghai Port, Taiwan Straits and sea around Taiwan province |
|  | Tianjin | XSZ | North and Central Huanghai Sea and Bohai Sea |
|  | Guangzhou | XSQ | Taiwan Straits, Bashi Channel, South China Sea and Beibu Wan Gulf |
| Hong Kong, China | Hong Kong | Broadcast via NAVTEX on 518 kHz | Waters inside the boundary line: 30°N 105°E to 30°N 125°E to 10°N 125°E, to 10°N 105°E, to 30°N 105°E |
| Japan | Hokkaido | JNL | Hokkaido area |
|  | Shiogama | JNN | Sendai area |
|  | Yokohama | JGC | Tokyo area |
|  | Nagoya | JNT | Nagoya area |
|  | Kobe | JGD | Kobe area |
|  | Hiroshima | JNE | Hiroshima area |
|  | Niigata | JNV | Niigata area |
|  | Maizuru | JNC | Maizuru area |
|  | Moji | JNR | Fukuoka area |
|  | Kagoshima | JNJ | Kagoshima area |
|  | Okinawa | JNB | Okinawa area |
| Malaysia | Klang | SSB 5 | Strait of Malacca |
|  | Labuan | SSB 16 | South China Sea |
|  | Kuching | SSB 5 | South China Sea |
| Philippines | Manila | DZR, DZG, DSP, DZD, DZF, DFH, DZO, DZN, DZS | Pacific waters inside the boundary line: 25°N 12°0E to 25°N 135°E, to 5°N 135°E, to 5°N 115°E, to 15°N 115°E, to 21°N 120°E, to 20°N 120°E |
|  | San Miguel | NPO | North Pacific waters east of 160°E; Philippine Sea, Japan Sea, Yellow Sea, East China Sea, South China Sea |
| Republic of Korea | Seoul | HLL | East Sea, Yellow Sea, Jeju, Chusan, Nagasaki, and Kagoshima areas Waters inside the boundary line: 43°N 120°E to 43°N 132°E to 27°N 132°E, to 27°N 120°E, to 43°N 120°E |
| Thailand | Bangkok | HSA | Gulf of Thailand, West coast of Southern Thailand, Strait of Malacca and South China Sea |
| U.S.A. | Honolulu, Hawaii | KMV-99 | Pacific Ocean |
| Viet Nam | Dannang | XVT 1-2 | Basco Gulf, Blendong Sea and Gulf of Thailand |
|  | Halphong | XVG 5, 9 | ditto |
|  | Ho Chi Minh Ville | XVS 1, 3, 8 | ditto |
|  | Nha Trang | XVN 1, 2 | ditto |

### APPENDIX 5-A

37

#### METEOROLOGICAL TELECOMMUNICATION NETWORK FOR THE TYPHOON COMMITTEE

Seoul

Beijing

Tokyo

Washington

Bangkok

Offenbach

Singapore

Pyongyang

Macao

Manila

Hong Kong

Hanoi

Phnom Penh

Vientiane

Kuala Lumpur

Circuits of Main Telecommunication Network

RTH			Main regional circuits

Regional circuits

NMC

Inter-regional circuits

### APPENDIX 5-B

39

#### PRESENT OPERATIONAL STATUS OF THE METEOROLOGICAL TELECOMMUNICATION NETWORK FOR THE TYPHOON COMMITTEE REGION

| 1. Main Telecommunication Network |  | Present Operational Status |
| --- | --- | --- |
|  | Beijing - Tokyo | Cable (MPLS), WMO FTP Beijing 30 Mbps/Tokyo 20 Mbps |
|  | Beijing - Offenbach | Cable (MPLS), TCP/IP Beijing 30 Mbps/Offenbach 50 Mbps |
|  | Washington - Tokyo | Internet, TCP/IP |
| 2. Main regional circuit |  |  |
|  | Tokyo - Bangkok | Cable (MPLS), TCP/IP Tokyo 6 Mbps/Bangkok 2 Mbps |
| 3. Regional circuits |  |  |
|  | Bangkok - Beijing | 2 Mbps leased line, FTP protocol CMACast (Satellite broadcast) |
|  | Bangkok - Hanoi | 2 Mbps leased line, FTP protocol and Internet, FTP protocol |
|  | Bangkok - Hong Kong | Internet, SFTP protocol |
|  | Bangkok - Phnom Penh | Internet (IPsec VPN), FTP protocol |
|  | Bangkok - Vientiane | Internet, FTP protocol |
|  | Beijing - Hanoi | 64 kbps leased line, CMACast (Satellite broadcast) |
|  | Beijing - Hong Kong | Cable (MSTP), 20 Mbps TCP/IP CMACast (Satellite broadcast) |
|  | Beijing - Macao | 20 Mbps leased line CMACast (Satellite broadcast) |
|  | Beijing - Pyongyang | 64 kbps leased line, CMACast (Satellite broadcast) |
|  | Beijing - Seoul | Cable (MPLS), TCP/IP Beijing 30 Mbps/Seoul 4 Mbps |
|  | Beijing - Vientiane | CMACast (Satellite broadcast) |
|  | Hong Kong - Macao | Internet (VPN) and Mobile leased line |
|  | Tokyo - Hong Kong | Cable (MPLS), WMO FTP Tokyo 6 Mbps/Hong Kong 1 Mbps |
|  | Tokyo - Seoul | Cable (MPLS), WMO FTP Tokyo 20 Mbps/Seoul 6 Mbps |
| 4. Inter-regional circuits |  |  |
|  | Bangkok - Kuala Lumpur | Cable (MPLS), TCP/IP 1 Mbps |
|  | Bangkok - Singapore | Cable (MPLS), TCP/IP 64 kbps |
|  | Tokyo - Manila | Cable (MPLS), TCP/IP Tokyo 6 Mbps/Manila 2 Mbps |
| 5. RTH radio broadcast |  |  |
|  | Bangkok | 1 FAX, 1 VOICE |
|  | Tokyo | 1 FAX |
| 6. Satellite broadcast |  |  |
|  | Operated by China: CMACast | Operational observations, warnings, NWP products, satellite image and fax distribution |
|  | Operated by Japan: HimawariCast (JCSAT-2B, 154°E) | Operational satellite image, NWP products, in-situ observation data and ASCAT ocean surface wind data distribution |
| 7. Internet Cloud Service |  |  |
|  | Operated by Japan: HimawariCloud | Operational satellite image in full resolution and all spectral bands |

### APPENDIX 5-C

42

#### LIST OF ADDRESSES, TELEX/CABLE AND TELEPHONE NUMBERS OF THE TROPICAL CYCLONE WARNING CENTERS IN THE REGION

| Centre | Mailing address | Telex/cable, Telephone, fax numbers |
| --- | --- | --- |
| Cambodia |  |  |
| Attn. Mr Ly Chana Deputy Director Department of Agricultural Hydraulics and Hydrometeorology | Norodom Boulevard | Tel.: (+855) 15 913081 Fax: (+855) 23 26345 |
| Attn. Mr Hun Kim Hak Chief of Cambodian National | Pochentong | Tel/Fax: (+855) 23 66193 66192 NMC 66191 Airport |
| China |  |  |
| National Meteorological Center China Meteorological Adm. (Director: Dr.Zhao Wei) | No. 46 Zhongguancun Nandajie, Beijing 100081 | Tel.: (+86) (10) 6840 9378 Fax: (+86) (10) 6217 5928 E-mail: wmc-bj@cma.gov.cn |
| Democratic People's Republic of Korea |  |  |
| Mr Ko Sang Bok Director Central Forecast Research Institute State Hydrometeorological Adm. | Oesong-dong Central District | Telex: 38022 TCT KP Tel.: (+850) (2) 321 4539 Fax: (+850) (2) 381 4410 |
| Hong Kong, China |  |  |
| Central Forecasting Office Hong Kong Observatory (Attn. Ms. S M. Tse) | 134A Nathan Road Tsim Sha Tsui Kowloon Hong Kong, China | Tel.: (+852) 2926 8371 (Office hours) (+852) 2368 1944 (24 hours) Fax: (+852) 2311 9448 (Office hours) E-mail: smtse@hko.gov.hk |
| Japan |  |  |
| Typhoon Center Weather Disaster Risk Reduction Division Atmosphere and Ocean Department Japan Meteorological Agency (Head: Dr. ISHIHARA Koji) | 3-6-9 Toranomon Minato-ku Tokyo 105-8431 | Tel.: (+81) (3) 6758 3900 ext.4231 Fax: (+81) (3) 3434 9047 (Office hours) |
| Lao People's Democratic Republic |  |  |
| Ministry of Agriculture and Forestry, Department of Meteorology and Hydrology, VIENTIANE | P.O. Box 811 Vientiane | Telex: 4306 ONU VTELS Cable: UNDEVPRO |
| Macao, China |  |  |
| Meteorological and Geophysical Bureau (Director: Leong Weng Kun) | Rampa do Observatório, Taipa Grande, Macau, China | Tel.: (+853) 88986173 Fax: (+853) 28850773 E-mail: meteo@smg.gov.mo |
| Malaysia |  |  |
| National Weather & Geophysics Operation Centre, Malaysian Meteorological Department, (Director: Mr. Maqrun Fadzli bin Mohd Fahmi) | Jalan Sultan 46667 Petaling Jaya Selangor Malaysia | Tel.: (+60) (3) 7967 8118 (+60) (3) 7967 8119 Fax: (+60) (3) 7955 0964 E-mail: maqrun@met.gov.my |

| Centre | Mailing address | Telex/cable, Telephone, fax numbers |
| --- | --- | --- |
| Philippines |  |  |
| Mr. Juanito S. Galang Chief, Marine Meteorological Services Section Weather Division, DOST-PAGASA | PAGASA Weather and Flood Forecasting Center, BIR Road, Pinyahan, Quezon City 1100 | Tel.: (+63) (2) 8284 0800 ext. 805 (24 hours) (+63) (2) 8284 0800 ext. 823 (Office hours) Fax.: (+63) (2) 892 (+63) (2) 892 Email: typhoon.ops@pagasa.dost.gov.ph junsgalang2313@gmail.com |
| Typhoon Committee Secretariat |  |  |
| Secretary: Dr. DUAN Yihong | Avenida de 5 de Outubro Coloane, Macau | Tel: (853) 8 8010531 Fax: (853) 8 8010530 E-mail: yujx@typhooncommittee.org |
| Republic of Korea |  |  |
| National Typhoon Center Korea Meteorological Administration (Deputy Director: Ms. Kyungho Lee) | 2 Seoseongro 810-gil, Namwon-eup, eogwipo, Jeju, 63614, Republic of Korea | Tel.: (+82) (70) 7850-6355 Fax: (+82) (64) 805-0368 E-mail: khlove1119@korea.kr |
| Thailand |  |  |
| Thai Meteorological Department (Director-General) | 4353 Sukhumvit Road, Bangna, Bangkok 10260 | Tel&FAX: (+66) (2) 398 9875 E-mail: tmd_inter@tmd.mail.go.th |
| Weather Forecast Division Thai Meteorological Department (Director: Mr. Surapong Sarapa) | 4353 Sukhumvit Road, Bangna, Bangkok 10260 | Tel&Fax: (+66) (2) 399 4001 E-mail: surapong_tmd@hotmail.com weatherman@metnet.tmd.go.th tmd_inter@tmd.mail.go.th |
| South East Asia Meteorological Telecommunication Center Thai Meteorological Department (Director: Ms. Kesaraporn Techapichetvanich) | 4353 Sukhumvit Road, Bangna, Bangkok 10260 | Tel.: (+66) (2) 399 4596 E-mail: gtsbkk@metnet.tmd.go.th kesaraporn.t@tmd.mail.go.th |
| USA |  |  |
| National Weather Service (Genevieve Miller, Meteorologist in charge) | 3232 Hueneme Road Barrigada Guam 96913 | Tel.: (+1-671) 472 0944 Fax: (+1-671) 472 7405 |
| RSMC Honolulu (Director: Mr. Christopher Brenchley) | 2525 Correa Road Suite 250 Honolulu, HI 96822 | Tel.: (+1-808) 973-5272 Fax: (+1-808) 973-5271 |
| Viet Nam |  |  |
| Forecast Division Forecast Department Hydro-Meteorological Service (Director: Nguyan Cong Thanh) | 4 Dan Thai Than Hanoi | Tel.: (+84) (4) 264020 Fax: (+84) (4) 254278 |

### APPENDIX 5-D

43

#### ABBREVIATED HEADINGS FOR THE TROPICAL CYCLONE WARNINGS

| Member | Abbreviated WMO Communication Headings |
| --- | --- |
| Cambodia |  |
| China | WTPQ20 BABJ |
| Democratic People's Republic of Korea |  |
| Hong Kong, China | WTPQ20 VHHH, WTSS20 VHHH |
| Japan | WTPQ50 - 55 RJTD |
| Lao People's Democratic Republic |  |
| Macao, China | For domestic dissemination only and WTMU40 VMMC |
| Malaysia | For domestic dissemination only |
| Philippines | WTPH20 - 22 RPMM |
| Republic of Korea | WTKO20 RKSL |
| Singapore | WTSR20 WSSS |
| Thailand | For domestic dissemination only |
| USA | WTPQ31 - 35 PGUM |
| Viet Nam | WTVS20 VNNN |

### APPENDIX 5-E

49

#### COLLECTION AND DISTRIBUTION OF INFORMATION RELATED TO TROPICAL CYCLONES

|  |  |  | Receiving station |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Data | Heading |  | TD | BJ | BB | HH | MM | SL | NN | KK | IV | PP | MC |
| Enhanced | SNCI30 | BABJ | BJ | O | BJ | BJ | TD | TD | BJ | BB | BB | BB |  |
| surface | SNHK20 | VHHH | HH | HH | BJ | O |  | TD | BB | BB | BB | BB | HH |
| observation | SNJP20 | RJTD | O | TD | TD | TD |  | TD | BB | BB | BB | BB |  |
|  | SNKO20 | RKSL | SL | TD | TD | TD |  | O | BB | BB | BB | BB |  |
|  | SNLA20 | VLIV | BB | BB | IV |  |  |  | BB | BB | O | BB |  |
|  | SNMS20 | WMKK | BB | BB | KK | BJ |  |  | BB | O | BB | BB |  |
|  | SNMU40 | VMMC |  | MC | BJ | BJ |  | TD | BB | BB | BB | BB | O |
|  | SNPH20 | RPMM | MM | TD | TD | TD | O | TD | BB | BB | BB | BB |  |
|  | SNTH20 | VTBB | BB | BB | O | BB |  | TD | BB | BB | BB | BB |  |
|  | SNVS20 | VNNN | BB |  | NN | BJ |  |  | O | BB | BB | BB |  |
| Enhanced | USCI01 | BABJ | BJ | O | BJ | BJ | TD | TD | BJ | BB | BB | BB |  |
| upper-air | USCI03 | BABJ | BJ | O | BJ | BJ | TD | TD | BJ | BB | BB | BB |  |
| observation | USCI05 | BABJ | BJ | O | BJ | BJ | TD | TD | BJ | BB | BB | BB |  |
|  | USCI07 | BABJ | BJ | O | BJ | BJ | TD | TD | BJ | BB | BB | BB |  |
|  | USCI09 | BABJ | BJ | O | BJ | BJ | TD | TD | BJ | BB | BB | BB |  |
|  | UKCI01 | BABJ | BJ | O | BJ | BJ |  | TD | BJ | BB | BB | BB |  |
|  | ULCI01 | BABJ | BJ | O | BJ | BJ |  | TD | BB | BB | BB | BB |  |
|  | ULCI03 | BABJ | BJ | O | BJ | BJ |  | TD | BB | BB | BB | BB |  |
|  | ULCI05 | BABJ | BJ | O | BJ | BJ |  | TD | BB | BB | BB | BB |  |
|  | ULCI07 | BABJ | BJ | O | BJ | BJ |  | TD | BB | BB | BB | BB |  |
|  | ULCI09 | BABJ | BJ | O | BJ | BJ |  | TD | BJ | BB | BB | BB |  |
|  | UECI01 | BABJ | BJ | O | BJ | BJ |  | TD | BB | BB | BB | BB |  |
|  | USHK01 | VHHH | HH | HH | BJ | O | TD | TD | BB | BB | BB | BB | HH |
|  | UKHK01 | VHHH | HH | HH | BJ | O |  | TD | BB | BB | BB | BB | HH |
|  | ULHK01 | VHHH | HH | HH | BJ | O |  | TD | BB | BB | BB | BB | HH |
|  | UEHK01 | VHHH | HH | HH | BJ | O |  | TD | BB | BB | BB | BB | HH |
|  | USJP01 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | UKJP01 | RJTD | O | TD | TD | TD |  | TD | BB | BB | BB | BB |  |
|  | ULJP01 | RJTD | O | TD | TD | TD |  | TD | BB | BB | BB | BB |  |
|  | UEJP01 | RJTD | O | TD | TD | TD |  | TD | BB | BB | BB | BB |  |
|  | USKO01 | RKSL | SL | TD | TD | TD | TD | O | BB | BB | BB | BB |  |
|  | UKKO01 | RKSL | SL | TD | TD | TD |  | O | BB | BB | BB | BB |  |
|  | ULKO01 | RKSL | SL | TD | TD | TD |  | O | BB | BB | BB | BB |  |
|  | UEKO01 | RKSL | SL | TD | TD | TD |  | O | BB | BB | BB | BB |  |
|  | USMS01 | WMKK | BB | TD | KK | TD | TD | TD | BB | O | BB | BB |  |
|  | UKMS01 | WMKK | BB | TD | KK | TD | TD | TD | BB | O | BB | BB |  |
|  | ULMS01 | WMKK | BB | TD | KK | TD | TD | TD | BB | O | BB | BB |  |
|  | UEMS01 | WMKK | BB | TD | KK | TD | TD | TD | BB | O | BB | BB |  |
|  | USPH01 | RPMM | MM | TD | TD | TD | O | TD | BB |  | BB | BB |  |
|  | UKPH01 | RPMM | MM | TD | TD | TD | O | TD | BB |  | BB | BB |  |
|  | ULPH01 | RPMM | MM | TD | TD | TD | O | TD | BB |  | BB | BB |  |
| Continued to | UEPH01 | RPMM | MM | TD | TD | TD | O | TD | BB |  | BB | BB |  |
| the next page | USTH01 | VTBB | BB | BB | O | BB | TD | TD | BB | BB | BB | BB |  |
|  |  |  | Receiving station |  |  |  |  |  |  |  |  |  |  |
| Type of Data | Heading |  | TD | BJ | BB | HH | MM | SL | NN | KK | IV | PP | MC |
| Enhanced | UKTH01 | VTBB | BB | BB | O | BB |  | TD | BB | BB | BB | BB |  |
| Upper-air | ULTH01 | VTBB | BB | BB | O | BB |  | TD | BB | BB | BB | BB |  |
| observation | UETH01 | VTBB | BB | BB | O | BB |  | TD | BB | BB | BB | BB |  |
|  | USVS01 | VNNN | BB | TD | NN | TD | TD | TD | O | BB | BB | BB |  |
|  | UKVS01 | VNNN | BB | TD | NN | TD |  | TD | O | BB | BB | BB |  |
|  | ULVS01 | VNNN | BB | TD | NN | TD | TD | TD | O | BB | BB | BB |  |
|  | UEVS01 | VNNN | BB | TD | NN | TD | TD | TD | O | BB | BB | BB |  |
|  | URPA10 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | URPA11 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | URPA12 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | URPA14 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | URPN10 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | UZPA13 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | UZPN13 | KNHC | * |  | TD | TD |  | TD | BB | BB | BB | BB |  |
|  | UZPN13 | KWBC | * | TD | TD | TD |  | TD | BB | BB | BB | BB |  |
|  | UZPN13 | PGTW | * | TD | TD | TD |  | TD | BB | BB | BB | BB |  |
|  | IUDC01 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC02 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC03 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC04 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC05 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC06 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC07 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC08 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC09 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUDC10 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
| Enhanced | SNVB20 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
| ship | SNVD20 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
| observation | SNVE20 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | SNVX20 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | SNVB21 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | SNVD21 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | SNVE21 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | SNVX21 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | SNVX20 | RPMM | MM | TD | TD | TD | O | TD | BB |  | BB | BB |  |
|  | SNVX20 | VHHH | HH | HH | BJ | O | TD | TD | BB | BB | BB | BB | HH |
|  | SNVX20 SMVX01 SIVX01 SNVX20 | VNNN VTBB VTBB VTBB | BB BB BB BB | TD BB BB BB | NN O O O | TD BB BB BB |  | TD TD TD TD | O BB BB BB | BB BB BB BB | BB BB BB BB | BB BB BB BB |  |
| Enhanced | SBCI30 | BABJ | BJ | O | BJ | TD | TD | TD | BJ | BB | BB | BB |  |
| radar | SCCI30 | BABJ |  | O | BJ | BJ |  |  | BB | BB | BB | BB |  |
| observation | SBCI60 | BCGZ |  | O | BJ |  |  |  | BJ | BB | BB | BB |  |
| Continued to | SCCI60 | BCGZ | HH | O | BJ |  |  |  | BB | BB | BB | BB |  |
| the next page | SBHK20 | VHHH | HH | HH | BJ | O | TD |  | BB | BB | BB | BB | HH |

|  |  |  | Receiving station |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Data | Heading |  | TD | BJ | BB | HH | MM | SL | NN | KK | IV | PP | MC |
| Enhanced | ISBC01 | VHHH | HH | HH | HH | O | TD | TD |  | BB | BB | BB |  |
| radar | ISBC01 | RJTD | O | TD | TD | TD | TD | TD |  | BB | BB | BB |  |
| observation | SDKO20 | RKSL |  |  |  |  |  | O |  |  |  |  |  |
|  | SDMS20 | WMKK | BB | TD | KK | TD |  |  | BB | O | BB | BB |  |
|  | SDPH20 | RPMM | MM | TD | TD |  |  | TD | BB |  | BB | BB |  |
|  | SDVS20 | VNNN | BB | TD | NN | TD | TD |  | O | BB | BB | BB |  |
| Satellite | TPPN10 | PGTW |  |  | TD | TD |  |  | BB | BB | BB | BB |  |
| guidance | TPPN10 | PGUA | * |  | TD | TD |  |  | BB | BB | BB | BB |  |
|  | TPPA1 | RJTY | * | TD | TD | TD | TD |  | BB | BB | BB | BB |  |
|  | TPPA1 | RODN | * | TD | TD | TD | TD |  | BB | BB | BB | BB |  |
|  | IUCC10 | RJTD | O | TD | TD | TD | TD | TD |  | BB | BB | BB |  |
|  | IUCC01 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUCC02 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUCC03 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
|  | IUCC04 | VHHH | HH | HH | HH | O |  |  |  |  |  |  |  |
| Tropical | FXPQ01 | VHHH | HH | HH | BJ | O |  |  | BB | BB | BB | BB | HH |
| Cyclone | FXPQ02 | VHHH | HH | HH | BJ | O |  |  | BB | BB | BB | BB | HH |
| Forecast | FXPQ03 | VHHH | HH | HH | BJ | O |  |  | BB | BB | BB | BB | HH |
|  | FXPQ21 | VHHH | HH | HH |  | O |  |  |  |  |  |  |  |
|  | FXPQ20 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ21 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ22 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ23 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ24 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ25 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ29 | VTBB |  |  | O |  |  |  |  |  |  |  |  |
|  | FXPQ30 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ31 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ32 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ33 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ34 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPQ35 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | FXPH20 | RPMM | MM | TD | TD | TD | O | TD | BB | BB | BB | BB |  |
|  | FXSS01 | VHHH | HH | HH | BJ | O |  |  | BB | BB | BB | BB | HH |
|  | FXSS02 | VHHH | HH | HH | BJ | O |  |  | BB | BB | BB | BB | HH |
|  | FXSS03 | VHHH | HH | HH | BJ | O |  |  | BB | BB | BB | BB | HH |
|  | FXSS21 | VHHH | HH | HH |  | O |  |  |  |  |  |  |  |
|  | FXPN03 | RKSL |  |  |  | TD |  | O |  |  |  |  |  |

|  |  |  | Receiving station |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Data | Heading |  | TD | BJ | BB | HH | MM | SL | NN | KK | IV | PP | MC |
| Warning | WDPN31 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WDPN32 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WHCI28 | BCGZ |  |  | BJ | BJ |  |  | BJ | BB | BB | BB |  |
|  | WHCI40 | BABJ | BJ | O | BJ | BJ |  |  | BJ | BB | BB | BB |  |
|  | WSPH | RPMM | * | TD | TD | TD | O | TD | BB | BB | BB | BB |  |
|  | WTMU40 | VMMC | BJ | MC | BJ | BJ |  |  | BB | BB | BB | BB | O |
|  | WTPN21 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPN31 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPN32 | PGTW | * | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPH20 | RPMM | MM | TD | TD | TD | O |  | BB |  | BB | BB |  |
|  | WTPH21 | RPMM | MM | TD | TD | TD | O |  | BB |  | BB | BB |  |
|  | WTPH22 | RPMM | MM | TD | TD | TD | O |  | BB |  | BB | BB |  |
|  | WTPQ20 | VHHH | HH | HH | BJ | O |  | TD | BB | BB | BB | BB | HH |
|  | WTSS20 | VHHH | HH | HH | BJ | O |  |  | BB | BB | BB | BB | HH |
|  | WTVS20 | VNNN |  |  | NN | BJ |  |  | O | BB | BB | BB |  |
|  | WTKO20 | RKSL | SL | TD | TD | TD |  | O | BB | BB | BB | BB |  |
| Prognostic | WTPQ30 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
| Reasoning | WTPQ31 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPQ32 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPQ33 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPQ34 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPQ35 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
| Five-day | WTPQ50 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
| forecast | WTPQ51 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPQ52 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPQ53 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPQ54 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |
|  | WTPQ55 | RJTD | O | TD | TD | TD | TD | TD | BB | BB | BB | BB |  |

Note: Meaning of abbreviation

O	:	Data originating centre

TD	:	Data transmitting centre	- Tokyo

BJ	:			- Beijing

BB	:			- Bangkok

HH	:			- Hong Kong

MM	:			- Manila

SL	:			- Seoul

NN	:			- Hanoi

KK	:			- Kuala Lumpur

IV	:			- Vientiane

PP	:			- Phnom Penh

MC	:			- Macao

*	:	Places other than described above

### APPENDIX 5-F

#### TABLE of Abbreviated headings (TTAAii CCCC)

50

| TT | Data designator |
| --- | --- |
| FK FX | Tropical cyclone advisories Miscellaneous forecasts |
| SB | Radar reports PART A |
| SC | Radar reports PART B |
| SD | Radar reports |
|  | (PART A and PART B) |
| SN | Synoptic reports |
|  | (non-standard hours) |
| TP | Satellite guidance |
| UA | Aircraft reports (AIREP) |
| UE | Upper-level observation, PART D |
| UK | Upper-level observation, PART B |
| UL | Upper-level observation, PART C |
| US | Upper-level observation, PART A |
| WD | Prognostic reasoning for typhoon |
| WH | Marine/Coastal flood warnings |
| WO | Other warnings |
| WC | Tropical cyclone (SIGMET) |
| WT | Tropical cyclone warnings |
| WW | Warning and weather summary |

| TABLE of Abbreviated Headings |  |
| --- | --- |
| (TTAAii CCCC) for BUFR |  |
| TTAAii CCCC | Data type |
| ISBC01 RJTD | Radar reports |
| ISBC01 VHHH | Radar reports |
| IUCC01-04 VHHH | SAREP reports |
| IUCC10 RJTD | SAREP reports |
| IUDC01-10 VHHH | Dropsonde reports |

| AA | Geographic designator |
| --- | --- |
| CI | China |
| HK | Hong Kong, China |
| JP | Japan |
| KO | Republic of Korea |
| KP | Cambodia |
| LA | Lao People’s Democratic Republic |
| MS | Malaysia |
| MU | Macao, China |
| PA | Pacific area |
| PH | Philippines |
| PN | North Pacific area |
| PQ | Western North Pacific |
| PW | Western Pacific area |
| SS | South China Sea area |
| TH | Thailand |
| VS | Viet Nam |

| CCCC | Location indicator |
| --- | --- |
| BABJ | Beijing |
| BCGZ | Guangzhou |
| KWBC | Washington |
| PGFW | San Diego (Fleet Weather Central) |
| PGTW | Honolulu (JTWC) |
| PGUM | Guam (Agana) |
| RJTD | Tokyo |
| RJTY | Yokota |
| RKSL | Seoul |
| RKSO | Osan |
| RODN | Okinawa / Kadena AB |
| RPMK | Clark AB |
| RPMM | Manila / Intl. |
| VDPP | Phnom Penh |
| VHHH | Hong Kong |
| VLIV | Vientiane |
| VMMC | Macao |
| VNNN | Hanoi |
| VTBB | Bangkok |
| WMKK | Kuala Lumpur |

### APPENDIX 6-A

51

#### EXAMPLE OF THE MESSAGE FORMAT FOR INQUIRY ON DOUBTFUL AND GARBLED REPORTS

##### Example 1. Inquiry on a doubtful report

BMBB01 VTBB 220245

RJTD

PLEASE CHECK THE FOLLOWING REPORT

BULLETIN 	SNTH20 VTBB

DATE AND TIME	210200

LOCATION 	48300

CONTENT	SECTION 1, 2ND GROUP: 80540

REGARDS

RSMC TOKYO =

##### Example 2. Inquiry on a garbled report

BMRR01 RPMM 210425

RJTD

AHD SNPH20 RPMM 210400 =

### APPENDIX 6-B

1

#### STANDARD PROCEDURES FOR THE VERIFICATION OF TROPICAL CYCLONE AND FORECAST AT NATIONAL METEOROLOGICAL CENTRES

1. 	General

Each Member will verify each tropical cyclone which affects it and summarize the verification made in a year

2. 	Basis for verification

The best initial tropical cyclone position, central pressure and maximum sustained wind as determined from a post-analysis conducted by the RSMC.

3. 	Points for verification

(1) Error statistics in each method (bias and standard deviation) by using common work sheets as shown in APPENDIX 6-C.  Statistical computations involve positioning of the centre, prediction of movement, and analysis and forecast of intensity of a tropical cyclone.

(2) Discussion of following points;

(i)	relative merits of each technique,

(ii)	effects of inaccuracies on the forecast,

(iii)	effects of meagreness of available relevant real-time observations,

(iv)	variation from one geographical area to another,

(v)	climatological factors in climatological and/or statistical method,

(vi)	large-scale circulation pattern for giving rise to extremely poor prediction performance.

### APPENDIX 6-C

4

#### Verification sheet for positioning of the centre, prediction of movement, and analysis and forecast of intensity of tropical cyclones

Tropical Cyclone   	(           )

Method

| Date | Analysed position |  | Revised position |  | Error |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | λA | R | λR | Δ | Δλ | ΔR |

Verification sheet for positioning of the centre, prediction of movement, and analysis and forecast of intensity of tropical cyclones

Tropical Cyclone   	(           )

Method			Forecast period  	24-hour   (check one)

48-hour

| Initial Date | Initial position |  | Forecast position |  | Revised position |  | Error |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | λI | F | λF | R | λR | Δ | Δλ | ΔR | Δα | ΔSP |

#### Verification sheet for positioning of the centre, prediction of movement, and analysis and forecast of intensity of tropical cyclones

Tropical Cyclone	(           )

Analysis	24-hour forecast	48-hour forecast

Method

| Date |  | Pa | Pr | ΔPa |  | Pf | Pr | ΔPf |  | Pf | Pr | ΔPf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note :

Pr :	Revised central pressure

Pa :	Analysed central pressure, 	ΔPa = Pa - Pr

Pf :	Predicted central pressure,	ΔPf = Pf - Pr

### APPENDIX 7-A

6

#### LIST OF DATA ARCHIVED BY RSMC TOKYO - TYPHOON CENTER

##### (a) Observation data (except for Himawari imagery data)

Kinds of data:	SYNOP, METAR, SHIP, BUOY, TEMP, PILOT, Aircraft,

Wind Profiler, AMV, Scatterometer, MW Sounder, MW Imager,

CSR, Hyperspectral IR Sounder, GNSS-RO, Ground-based

GNSS

##### (b) Himawari imagery data

###### Himawari Standard Data (HSD):

Kind of data:	Himawari full-spec imagery data

Data format: 	Himawari Standard Format (https://www.data.jma.go.jp/mscweb/en/himawari89/space_segment/hsd_sample/HS_D_users_guide_en_v13.pdf)

###### Meteorological Satellite Center Monthly Report (DVD):

Kinds of data:	Himawari images in SATAID and PNG formats.

(https://www.data.jma.go.jp/mscweb/en/product/library_report.html)

Area coverage:

SATAID: 	115°E ~ 150°E and 15°N ~ 50°N

PNG:	Full earth disk as seen from 140°E

##### (c) Objective Analysis data

###### Global Surface/Atmospheric Analysis data

Kinds of data:	Grid point data of the objective surface/atmospheric analysis

Area coverage:	Global area covered by 1.25 X 1.25 latitude-longitude grid system.

Time of analysis:	00, 06, 12 and 18 UTC

Element and layer:

Surface:	Sea surface pressure (Ps), temperature (Ts),

Dew point depression 				(Ts - Tds), wind (Us, Vs);

Specific pressure levels (1000 - 0.4 hPa):

Geopotential height (Z), temperature (T), wind (U, V),

Dew point depression (T-Td)

##### Western North Pacific Sea Surface Temperature Analysis data

Kinds of data:	Grid point data of the objective sea surface temperature analysis

Area coverage:	Western North Pacific area (100°E ~ 180°E and 0° ~ 60°N			)

covered by 0.1 X 0.1 latitude-			longitude grid system.

Time of analysis:	18 UTC

Element: SST, SST anomalies from the JMA climatology

(d) Others

Typhoon bogus

### APPENDIX 7-B

8

#### GLOBAL TROPICAL CYCLONE TRACK AND INTENSITY DATA SET - REPORT FORMAT

| Position | Content |
| --- | --- |
| 1-9 | Cyclone Identification code composed by 2 digit numbers in order within the cyclone season, area code and year code. 01SWI2000 shows the 1st system observer in South-West Indian Ocean basin during the 2000/2001 season. Area codes are as follows: ARB = Arabian Sea ATL = Atlantic Ocean AUB = Australian Region (Brisbane) AUD = Australian Region (Darwin) AUP = Australian Region (Perth) BOB = Bay of Bengal CNP = Central North Pacific Ocean ENP = Eastern North Pacific Ocean ZEA = New Zealand Region SWI = South-West Indian Ocean SWP = South-West Pacific Ocean WNP = Western North Pacific Ocean and South China Sea |
| 10-19 | Storm Name |
| 20-23 | Year |
| 24-25 | Month (01-12) |
| 26-27 | Day (01-31) |
| 28-29 | Hour-universal time (at least every 6 hourly position -00Z, 06Z, 12Z and 18Z) Latitude indicator: 1 = North latitude; 2 = South latitude |
| 31-33 | Latitude (degrees and tenths) |
| 34-35 | Check sum (sum of all digits in the latitude) |
| 36 | Longitude indicator: 1 = West longitude; 2 = East longitude |
| 37-40 | Longitude (degrees and tenths) |
| 41-42 | Check sum (sum of all digits in the longitude) |
| 43 | Position confidence 1 = good (< 30 nm; < 55 km) 2 = fair (30 - 60 nm; 55 - 110 km) 3 = poor (> 60 nm; > 110 km) 9 = unknown |
| 44-45 | Dvorak T-number (99 for no report) |
| 46-47 | Dvorak CI-number (99 for no report) |
| 48-50 | Maximum average wind speed (whole values) (999 for no report) |
| 51 | Units 1 = kt, 2 = m/s, 3 = km/h. |
| 52-53 | Time interval for averaging wind speed (minutes for measured or derived wind speed, 99 if unknown or estimated). |
| 54-56 | Maximum Wind Gust (999 for no report) |
| 57 | Gust Period (seconds, 9 for unknown) |
| 58 | Quality code for wind reports: 1 = Aircraft or Dropsonde observation 2 = Over water observation (e.g. buoy) 3 = Over land observation 4 = Dvorak estimate 5 = Other |
| 59-62 | Central pressure (nearest hectopascal) (9999 if unknown or unavailable) |
| 63 | Quality code for pressure report (same code as for winds) |
| 64 | Units of length: 1 = nm, 2 = km |
| 65-67 | Radius of maximum winds (999 for no report) |
| 68 | Quality code for RMW: 1 = Aircraft observation 2 = Radar with well-defined eye 3 = Satellite with well-defined eye 4 = Radar or satellite, poorly-defined eye 5 = Other estimate |
| 69-71 | Threshold value for wind speed (gale force preferred, 999 for no report) |
| 72-75 | Radius in Sector 1: 315° - 45° |
| 75-79 | Radius in Sector 2: 45° - 135° |
| 80-83 | Radius in Sector 3: 135° - 225° |
| 84-87 | Radius in Sector 4: 225° - 315° |
| 88 | Quality code for wind threshold 1 = Aircraft observations 2 = Surface observations 3 = Estimate from outer closed isobar 4 = Other estimate |
| 89-91 | Second threshold value for wind speed (999 for no report) |
| 92-95 | Radius in Sector 1: 315° - 45° |
| 95-99 | Radius in Sector 2: 45° - 135° |
| 100-103 | Radius in Sector 3: 135° - 225° |
| 104-107 | Radius in Sector 4: 225° - 315° |
| 108 | Quality code for wind threshold (code as for row 88) |
| 109-110 | Cyclone type: 01 = tropics; disturbance (no closed isobars) 02 = < 34 knot winds, < 17 m/s winds and at least one closed isobar 03 = 34 - 63 knots, 17-32 m/s 04 = > 63 knots, > 32 m/s 05 = extratropical 06 = dissipating 07 = subtropical cyclone (nonfrontal, low pressure system that comprises initially baroclinic circulation developing over subtropical water) 08 = overland 09 = unknown |
| 111-112 | Source code (2 - digit code to represent the country or organization that provided the data to NCDC USA. WMO Secretariat is authorized to assign number to additional participating centers, organizations) 01 = RSMC Miami - Hurricane Center 02 = RSMC Tokyo - Typhoon Center 03 = RSMC-tropical cyclones New Delhi 04 = RSMC La Reunion - Tropical Cyclone Centre 05 = Australian Bureau of Meteorology 06 = Meteorological Service of New Zealand Ltd. 07 = RSMC Nadi - Tropical Cyclone Centre 08 = Joint Typhoon Warning Center, Honolulu 0912= Madagascar Meteorological Service 1012 = Mauritius Meteorological Service 1112 = Meteorological Service, New Caledonia 12 = Central Pacific Hurricane Center, Honolulu |
| Headings | 1-19 Cyclone identification code and name; 20-29 Date time group; 30-43 Best track positions; 44-110 Intensity, Size and Type; 111-112 Source code. |

### APPENDIX 8-A

14

#### TROPICAL CYCLONE FORECAST COMPETENCY IN THE TYPHOON COMMITTEE REGION

##### Category 1

This competency unit is relevant to dedicated or specialized TC forecasters working in a TC office at an unsupervised level. It includes:

• analyzing broad-scale environment and determine TC position, intensity and structure;

• forecasting TC track, intensity and structure;

• determining potential TC-related hazards;

• formulating and issuing TC-related warning products;

• communicating relevant TC information to internal and external stakeholders.

##### Category 2

This competency unit is relevant to general forecasters who provide a range of TC forecast services based on information from the ‘parent’ RSMC or other agencies, and/or available data. It includes:

• accessing, interpreting, and adapting TC analysis and forecast;

• determining potential TC-related hazards;

• formulating and issuing TC-related warning products;

• communicating relevant TC information to internal and external stakeholders.

| Analyze broad-scale environment and determine TC position, intensity and structure (for Category 1) |  |  |
| --- | --- | --- |
| Description A range of observational information is analysed to interpret the synoptic scale environment, the position, intensity and structure of the tropical circulation |  |  |
| Performance criteria |  | analyzes the synoptic scale environment to assess the likely influence on the disturbance in a range of situations |
|  |  | determines TC centre location and current movement in accordance with standard operating procedures in a range of situations |
|  |  | determines TC intensity in accordance with standard operating procedures in a range of situations |
|  |  | determines TC structure in accordance with standard operating procedures in a range of situations |
| Back- ground | Know-ledge | standard operating procedures for TC analysis |
|  |  | basic TC climatology and general impacts of ENSO on TC behaviors |
|  |  | capabilities and limitations of different observational data types |
|  |  | TC structure dynamics and conceptual models |
|  |  | synoptic scale factors that affect the tropical cyclone intensity including shear, ocean temperatures, upper-level flow, stability, landfall, vorticity and low to mid-level moisture |
|  |  | strengths and limitations of intensity analysis methods including Dvorak technique and other ones, such as ADT, CLOUD, AMSU intensity estimation and SATCON. |
|  | Skills | uses data viewing software and other applications in the forecast process |
|  |  | interprets observations, weather radar and satellite derived information such as scatterometry and cloud drift winds |
|  |  | interprets satellite imagery including water vapor, visible, infra-red, and microwave for TC analysis |
|  |  | uses Dvorak technique for TC centre location and intensity estimation. |
|  |  | estimates the intensity from a number of inputs |
|  |  | interprets wind shear from shear analyses and prognoses |
|  |  | assesses the environment for motion and intensity changes |
|  |  | interprets NWP guidance material |

| Forecast TC track, intensity and structure (for Category 1) |  |  |
| --- | --- | --- |
| Description A range of information including numerical weather prediction (NWP) and objective aids in addition to an understanding of conceptual synoptic forecast approaches are used to forecast the track, intensity and structure in warning products that are issued in accordance with documented procedures. |  |  |
| Performance criteria |  | interprets NWP-predicted synoptic scale environment to assess the likely influence on the disturbance in a range of situations |
|  |  | determines TC forecast track in accordance with standard operating procedures in a range of situations |
|  |  | determines TC forecast intensity in accordance with standard operating procedures in a range of situations |
|  |  | determines TC forecast structure in accordance with standard operating procedures and timelines in a range of situations |
| Back-ground | Know-ledge | standard operating procedures for TC forecasts |
|  |  | relative strengths and limitations of NWP in predicting cyclone movement, structure and intensity |
|  |  | basic concept of rapid intensification/weakening, landfall process, and extra tropical transition |
|  |  | verification results of official TC forecasts and NWP guidance |
|  |  | basic theory of TC ensemble forecasts |
|  |  | synoptic factors that affect TC genesis, motion, intensity, and structure |
|  |  | track forecasting techniques including consensus and ensemble forecasts |
|  |  | intensity forecasting methods |
|  | Skills | evaluates model predictions against observed conditions to assess the most likely forecast environment for motion and intensity changes |
|  |  | evaluates TC genesis potential using observations and NWP guidance including ensembles |
|  |  | interprets NWP guidance material including ensemble output to determine forecast uncertainty |
|  |  | uses software systems to determine forecast parameters |

| Access, interpret, and adapt TC analysis and forecast (for Category 2) |  |  |
| --- | --- | --- |
| Description Guidance products from RSMC and other agencies are appropriately interpreted and assessed. Technical information including satellite and other observational information are interpreted taking into consideration the guidance products |  |  |
| Performance criteria |  | evaluates and adapt TC analysis and forecast based on information from RSMCs or other TC forecast agencies, and/or available data |
|  |  | interprets technical forecast guidance in order to assess impact potential upon forecast region of responsibility |
|  |  | interprets observational and satellite information appropriately |
| Back-ground | Know-ledge | standard operating procedures for TC analysis and forecasts |
|  |  | capabilities and limitations of different observational data types |
|  |  | TC structure dynamics and conceptual models |
|  |  | synoptic scale factors that affect the tropical cyclone intensity including shear, ocean temperatures, upper-level flow, stability, landfall, vorticity and low to mid-level moisture |
|  |  | relative strengths and limitations of NWP in predicting cyclone movement, structure and intensity |
|  |  | synoptic factors that affect TC genesis, motion, intensity, and structure |
|  |  | track forecasting techniques including consensus and ensemble forecasts |
|  |  | intensity forecasting methods |
|  |  | strengths and limitations of Dvorak technique, and other intensity analysis guidance, such as ADT, CLOUD, AMSU intensity estimation, and SATCON |
|  | Skills | uses data viewing software and other applications in the forecast process |
|  |  | interprets observations, weather radar, satellite and satellite derived information at a general level |
|  |  | assesses the environment for impact on the TC at a general level |
|  |  | interprets NWP guidance material |
|  |  | interprets official TC forecast products from official agencies |

| Determine potential TC-related hazards (for Category 1 &2 ) |  |  |
| --- | --- | --- |
| Description Potential TC-related hazards such as high winds, rainfall, waves and storm surge are determined, taking also into consideration mesoscale weather phenomena, for key locations according to appropriate thresholds and including estimates of uncertainty. |  |  |
| Performance criteria |  | forecasts extent of cyclonic winds (e.g. gales, storm force) and onset times for key locations using available guidance in a range of situations. |
|  |  | forecasts rainfall using available guidance in a range of situations and liaise with relevant organizations to determine potential flooding and landslide. |
|  |  | forecasts waves in accordance with standard operating procedures. |
|  |  | forecasts storm tide potential considering various TC forecast scenarios and confidence levels (worst case, most likely, alternate TC forecast scenario ). |
| Back-ground | Know-ledge | standard operating procedures for TC-related hazards including wave and storm surge associated with tropical cyclones. |
|  |  | potential TC-related hazards in a range of synoptic and mesoscale situations |
|  |  | basic theory of wave and storm surge |
|  | Skills | interprets guidance material of NWP and/or other Centres such as RSMCs. |
|  |  | assesses rainfall potential using probabilistic rainfall guidance, such as eTRaP and consensus model guidance (OCF, PME). |
|  |  | determines onset, duration, coverage and associated uncertainties of weather phenomena |
|  |  | interprets TC storm surge forecast guidance |

| Formulate and issue TC-related warning products (for Category 1 &2 ) |  |  |
| --- | --- | --- |
| Description Forecast production systems are used to produce and disseminate a range of TC-related waning products according to operating procedures. |  |  |
| Performance criteria |  | liaises effectively with internal staff in the development of TC forecast scenarios and impact on other services. |
|  |  | formulates TC-related warning products, in consideration of potential impacts, in accordance with standard operating procedures in a range of situations. |
|  |  | determines the appropriate key messages for general and technical audiences in a range of situations. |
|  |  | issues the range of TC-related warning products in accordance with standard operating procedures and timelines in a range of situations. |
| Back-ground | Know-ledge | standard operating procedures for warning issuance and contingency plans of relevant DRR authorities such as local governments. |
|  |  | local characteristics of potential impacts of tropical cyclones |
|  |  | level of threat posed by storm tide |
|  |  | user needs and significant impact thresholds |
|  |  | product styles and standards |
|  | Skills | uses appropriate software to determine range of potential impacts and produce warning products |
|  |  | communicates with colleagues to formulate warning products |
|  |  | compiles products and key messages for different audiences |
|  |  | converts technical concepts into concise and easy to understand language |

| Communicate relevant TC information to internal and external stakeholders (for Category 1 &2 ) |  |  |
| --- | --- | --- |
| Description Forecasters are required to communicate information to internal and external users appropriate to their needs. |  |  |
| Performance criteria |  | logically structures briefings and presentations to contain relevant, timely, and understandable information |
|  |  | delivers briefings, presentations and interviews to suit the intended audience explaining technical information in concise, clear and easy to understand language |
|  |  | communicate with related internal and external parties, such as DRR emergency managers, RSMCs, other TC forecast centres and weather services in neighboring areas |
|  |  | responds to requests for information appropriately |
| Back-ground | Know-ledge | principles of effective communication, including presentation and interviews |
|  |  | presentation and meeting formats and requirements |
|  |  | legislation, regulations, policies, procedures and guidelines relating to workplace communication in the public sector such as privacy, confidentiality, freedom of information |
|  | Skills | compiles products and key messages for different audiences |
|  |  | converts technical concepts into concise and easy to understand language |
|  |  | facilitates and engages in communication exchanges |
|  |  | uses equipment for presentations |

Details are shown in 4.2.

For converting the wind speeds of different averaging periods such as 1-min, 2-min, 3-min and 10-min, Tropical Cyclone Programme of WMO recommends to follow the guidelines as shown in the Appendix 1-A.

The ICAO Asia Pacific Region ROBEX Handbook describes the ROBEX scheme, which consists of a number of Regional OPMET Centres (ROCs), Regional OPMET Data Banks (RODBs) and Inter-regional OPMET Gateways (IROGs) to deliver to the aviation users the required OPMET information in the form of predefined bulletins.

"Tropical cyclone" is a generic term that includes tropical depression, tropical storm, severe tropical storm and typhoon.

Classifications internally used by Members are shown in Appendix 4-A.

The AFS is comprised of a number of systems and applications that are used for ground-ground (i.e. point-to-point and/or point-to-multipoint) communications in the international aeronautical telecommunication service. In accordance with the ROBEX scheme, the (AFS) systems used to disseminate SIGMET/tropical cyclone advisory information and air-reports include the aeronautical fixed telecommunications network (AFTN) and the air traffic services message handling system (AMHS).

As detailed in Harper et al. (2010), this traditional assumption is without a firm basis.

More information available on

https://nsmc.org.cn/nsmc/en/home/index.html

More information available on https://www.jma.go.jp/jma/jma-eng/satellite/HimawariRequest.html

Coast station VRX closed on 1 October 2006.

Confidence in the center position: Degree of confidence in the center position of a tropical cyclone expressed as the radius of the smallest circle within which the center may be located by the analysis. “position good” implies a radius of less than 30 nm, 55 km; “position fair”, a radius of 30 to 60 nm, 55 to 110 km; and “position poor”, radius of greater than 60 nm, 110 km.

no longer used